3GPP TSG-WG SA2 Meeting #140E e-meeting 	S2-2005662

Elbonia, August 19 – September 1, 2020	(revision of S2-200xxxx)

Source:	Huawei, HiSilicon

Title:	KI #6: evaluation and conclusion

Document for:	Approval

Agenda Item:	8.7

Work Item / Release:	FS_ID_UAS-SA2 / Rel-17

Abstract: This contribution provides evaluations and conclusions for KI#6 UAV Controller and UAV association.

1. Introduction/Discussion

The solutions of KI#6: UAV Controller and UAV association include solution #2, #5, #20 and #21. The following table provides an analysis and comparison, focusing on aspects of KI#6.

| Solution | Evaluation | Impacts |
| --- | --- | --- |
| Sol#2:3GPP reference architecture for UAV remote identification | Architecture solution, no specific details on how to address KI#6 | none |
| Sol#5:UAV authentication and authorization by USS/UTM based on NAS supplementary and secondary authentication and authorization procedures | UAV/UAVC pairing authentication and authorization is done by USS/UTM during NSSAA-like procedure or PDU session secondary authentication and authorization procedure, the result is notified to SMF or AMF or UFES | - PDU session secondary authentication and authorization procedure enhancements - New secondary authentication and authorization like NSSAA - UAV/UAVC pairing authentication and authorization result is notified to SMF or AMF or UFES |
| Sol#20:UAV and UAVC Association and Connectivity Control Using Secondary Authorization | UAV/UAVC pairing authentication and authorization is done by USS/UTM during PDU session secondary authentication and authorization procedure, the result is notified to SMF | - PDU session secondary authentication and authorization procedure enhancements - UAV/UAVC pairing authentication and authorization result is notified to SMF |
| Sol#21:Connectivity setup for C2 communication and association between UAV and UAV-C | Before PDU session setup, UAV/UAVC pairing authentication and authorization is done by USS/UTM and the result (including a UTID) is notified to the PLMN/UAV/UAVC. During PDU session setup, the PLMN re-authenticates the UAV/UAVC based on the result and the UTID. | - UAV/UAVC pairing authentication and authorization result and UTID is notified to SMF - PDU session establishment procedure enhancement - UTID |

Proposal 1: UAV/UAVC pairing authentication and authorization is done by USS/UTM during PDU session secondary authentication and authorization procedure, the result is notified to SMF.

Proposal 2: UAV/UAVC pairing authentication and authorization is done by USS/UTM during New secondary authentication and authorization procedure like NSSAA, the result is notified to UFES.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.754 v0.2.0.

# * * * * First change (all new) * * * *

7	Evaluation

7.6	Key Issue 6: UAV Controller and UAV association

Sol#2: this solution has no specific details on how to address this key issue.

Sol#5: this solution proposes that UAV/UAVC pairing authentication and authorization is done by USS/UTM during NSSAA-like procedure or PDU session secondary authentication and authorization procedure, the result is notified to SMF or AMF or UFES.

Sol#20: this solution proposes that UAV/UAVC pairing authentication and authorization is done by USS/UTM during PDU session secondary authentication and authorization procedure, the result is notified to SMF.

Sol#21: this solution proposed that UAV/UAVC pairing authentication and authorization is done by USS/UTM and the result (including a UTID) is notified to the PLMN/UAV/UAVC before PDU session setup, hence, the SMF can re-authenticate/authorizes the UAV/UAVC based on the result and the UTID during PDU session setup.

# * * * * Second change (all new) * * * *

8	Conclusions

8.6	Key Issue 6: UAV Controller and UAV association

When separate PDU Sessions for C2 communication, or when there is a static association between UAV and UAVC then UAV/UAVC pairing authentication and authorization is done by USS/UTM during PDU session secondary authentication and authorization procedure, the result is notified to SMF.

In addition, UAV/UAVC pairing authentication and authorization can also be done by USS/UTM during new secondary authentication and authorization procedure like NSSAA if available, the result is notified to UFES.

# * * * * End of changes * * * *