3GPP TSG-WG SA2 Meeting #140E e-meeting 	S2-2005415

Elbonia, August 19 – September 2, 2020	(revision of S2-200xxxx)

Title:	[DRAFT] LS on RAN impact of FS_5MBS Study

Release:	Release 17

Work Item:	FS_5MBS, NR_MBS-Core

Source:	[Huawei to be] SA2

To:	RAN2, RAN3

Cc:

Contact Person:

Name:	Meng Li

Tel. Number:

E-mail Address:	Raymond DOT limeng AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	TR 23.757 v0.4.0 (and available shortly after SA2#140E TR 23.757 v0.5.0)

1. Overall Description:

SA2 has been discussing on Rel-17 FS_5MBS solutions since SA2 #135 meeting in TR 23.757.

SA2 would like to kindly inform RAN2 and RAN3 the following interim agreements in SA2:

-	SA2 will provide means to provide QoS requirements for an MBS Session to RAN nodes.

-	[Depends on the outcome of SA2#140E] SA2 agrees the Architecture X in section A.X is used as the baseline architecture.

-	[Depends on the outcome of SA2#140E] SA2 agrees that for each MBS session an own shared N3 tunnel will be used for DL MBS data delivery from MB-UPF to NG-RAN nodes. This tunnel can use either IP multicast transport (NG-RAN sends IGMP/MLD Join/Leave to a multicast router) or point-to-point unidirectional N3 tunnels from MB-UPF to NG-RAN nodes.

SA2 would like to kindly ask RAN2 and RAN3 the following questions:

- SA2 sees the benefit that a UE can transit into CM-IDLE/RRC-INACTIVE state after having the transmission resource configuration for a MBS session to save power, 	SA2 would appreciate RAN2 and RAN3 feedback on this .

- SA2 assumes 5G CN can trigger a notification to CM-IDLE/RRC-INACTIVE UEs before PTM/PTP delivery can commence at start of a MBS Session. SA2 has two options for the notification: a) group paging, and b) legacy session start notification. Which option do RAN2 and RAN3 intend to support for this purpose?

- Some proposed Xn/N2 handover solutions in the SA2 study propose MBS data forwarding from S-RAN to the T-RAN to address the potential data loss in case of UE moving to a T-RAN supporting 5MBS that may or may not have the on-going MBS session. SA2 would appreciate RAN2 and RAN3 feedback on the forwarding consideration at Xn/N2 handover.

- [Depends on the outcome of SA2#140E] The SA2 study focuses on multicast transmission, but also includes some solution for broadcast transmission. Do RAN2 and RAN3 intend to also support broadcast transmission (i.e. transmission of data without knowledge about the UEs receiving the broadcasted data) in Rel-17?

2. Actions:

To RAN2 and RAN3 group.

ACTION:

SA2 respectfully requests RAN2 and RAN3 to take the above information into account and to answer the questions raised by SA2.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #141E		October 12 – 23, 2020			Elbonia

TSG-SA2 Meeting #142E		November 16 – 20, 2020			Elbonia