SA WG2 Meeting #140E (e-meeting)	S2-2004995

Aug 19 – Sep 01, 2020, Elbonia	(was S2-20XXXXX)

Source:	vivo

Title:	KI#1: Interim conclusions

Document for:	Approval

Agenda Item:	8.2

Work Item / Release:	FS_eNPN /Rel-17

Abstract of the contribution: it propose the interim conclusion for KI#1.

- Text proposal

It is proposed to capture the following into TR 23.070-07.

8	Conclusions

Editor's note:	This clause will capture conclusions from the study.

8.X	Key Issue #1: Enhancements to Support SNPN along with credentials owned by an entity separate from the SNPN

Editor's note:	These are interim conclusion for Key issue #1.

A UE is able to access a SNPN with the credentials owned by a SP separate from the SNPN and the SP credential is possible to be 3GPP credential. The SP separate from the SNPN can be 1) a PLMN; 2) another SNPN, etc.

Editor’s note: The possibility of the credential owned by the SP as non 3GPP credential and whether the SP can be a 3rd party other than PLMN or SNPN is pending to the input from SA3 and need further study.

It is recommended to apply a unified architecture for accessing to a SNPN with SP credential: A roaming similar architecture. But roaming is still not supported between SNPN and PLMN or another SNPN.

- The SP at least provides the UE authentication service (as described in AUSF service in 23.502) to the NF in the SNPN in order to authenticate the UE.

A SP controlled network priority list is configured and updated by the SP to the UE. The UE use it to select a suitable network, including SNPN.

It is left to the UE implementation to choose one credential to access to a SNPN if the UE has multiple credentials allowed for the SNPN.

Note: KI#1 and KI#2 has overlapped requirement for mobility and service continuity. In order to have an overall evaluation, the mobility, service continuity handling detail and whether home routed like accessing to SP via SNPN will be concluded in KI#2 and it is not captured here.