SA WG2 Meeting #140E	S2-2005659

Electronic meeting, Aug 19 – Sep 01, 2020	(Revision of S2-200nnnn)

Source:	Nokia, Nokia Shanghai Bell

Title:	KI#2 Update of Solution #1

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	FS_5MBS / Rel-17

Abstract of the contribution:

This paper updates solution #1 to resolve the open editor´s notes.

# 1	Discussion

For both the basic service level and the enhanced service level editor´s notes remain that it is FFS whether the list is complete.

Those editor´s notes could not be removed at SA2#139e because the significance of Switching between 5GC Shared MBS traffic delivery method and 5GC Individual MBS traffic delivery method was controversial.

The main motivation for such a switching is to provide MBS data to UEs that move out of a network area where MBS is supported. It depends on the network deployment if such network areas exist in a PLMN. The importance of this feature is thus strongly dependent on the deployment. This matches the definition of the enhanced service level:

The requirements for the Basic service level also apply for the Enhanced Service Level. Additional requirements for the Enhanced Service Level are listed below. Different requirements out of this set may be necessary to address specific use cases.

Proposal The Switching between 5GC Shared MBS traffic delivery method and 5GC Individual MBS traffic delivery method is added to the enhanced service level and editor´s notes about the completeness of the lists are removed

# 2	Conclusions

Proposal: Agree the proposed changes to Solution #1.

Proposed changes to TR 23.757

## 6.1	Solution #1: Multicast service levels

### 6.1.1	Functional description

The following service levels for the multicast communication service are defined:

NOTE 1:	Transport Only mode and Full-Service mode of operation as defined in TS 23.246 [4] differ from the service levels defined here.

Basic Service Level

Requirements for the basic service level are mandatory to be supported. The following requirements are defined:

-	Media are transported transparently through the 5GS;

-	Request to receive the multicast service;

-	Efficient packet distribution from the 5GS ingress to (R)AN node(s).

-	Efficient data delivery from (R)AN node(s) to the UE.

Enhanced Service Level

The requirements for the Basic service level also apply for the Enhanced Service Level. Additional requirements for the Enhanced Service Level are listed below. Different requirements out of this set may be necessary to address specific use cases.

-	Local MBS service;

-	User authentication and authorization;

NOTE 2:	User authentication and authorization can be done by 5GS or AF or both, or even not be needed for a multicast communication service.

-	Explicit configuration of multicast session by network function external to the 5GS including:

-	Group member management;

-	Enhanced QoS support;

-	Content security protection.

-	Switching between 5GC Shared MBS traffic delivery method and 5GC Individual MBS traffic delivery method.

### 6.1.2	Procedures

It is expected that procedures to address the requirements for the service levels defined in this solution proposal are proposed as part of other solution proposals.

### 6.1.3	Impacts on services, entities and interfaces

It is expected that an impact analysis to address the requirements for the service levels defined in this solution proposal are proposed as part of other solution proposals.