SA WG2 Meeting #141E (e-meeting)	S2-2006835

Oct 12 – Oct 23, 2020, Elbonia

Source:	CATT

Title:	KI #7 evaluation

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	FS_5MBS / Rel-17

Abstract of the contribution: This contribution proposes the evaluation for Key Issue #7.

- Discussion

For Key Issue #7 "Reliable delivery method switching between unicast and multicast", there are the following solutions in the TR:

-	Solution #11, #12, #18, #22~#31, #39 and #40.

These solutions can be categorized as follows:

- Switching between 5GC Shared and Individual MBS traffic delivery methods: Solution #23, #24, #25, #28, #30, #31 and #40 (single RAN case).

- Switching between PTP and PTM delivery methods: Solution #18, #22 and #25.

- Inter-RAN handover / UE mobility related delivery method switching: Solution #11, #12, #26, #27, #29, #39 and #40 (dual RANs case).

Evaluations are made on the solutions for each category.

# 2. Proposal

It is proposed to include the following changes in TR 23.757.

***** Start of 1st Change (all new text) *****

7.X	Key Issue #7: Reliable delivery method switching between unicast and multicast

Solution #11, #12, #18, #22~#31, #39 and #40 are proposed to address Key Issue #7 in the present document. These solutions can be categorized as follows:

- Switching between 5GC Shared and Individual MBS traffic delivery methods: Solution #23, #24, #25, #28, #30, #31 and #40 (single RAN case).

- Switching between PTP and PTM delivery methods: Solution #18, #22 and #25.

- Inter-RAN handover / UE mobility related delivery method switching: Solution #11, #12, #26, #27, #29, #39 and #40 (dual RANs case).

Following are the evaluations on the solutions for each category:

1) Switching between 5GC Shared and Individual MBS traffic delivery methods

A comparison between solutions for switching between 5GC Shared and Individual MBS traffic delivery methods is shown in Table 7.X-1.

Table 7.x-1: Comparison of solutions for switching between 5GC Shared and Individual MBS traffic delivery methods

|  | Sol#23 | Sol#24 | Sol#25 | Sol#28 | Sol#30 | Sol#31 | Sol#40 (single RAN case) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Delivery method switching triggers/ criteria | - Application level trigger. - QoS requirements. - UE MBS subscription changes. - Handover or reselection to target RAN supporting or not supporting MBS. - Predefined threshold (i.e. UE number) | - Content/ Application Server decisions. - Consumption reports sent by the UE. - Handover. | - Number of the UEs receiving the multicast service in a certain multicast group | - Number of devices receiving the specific content. - Number of devices in a specific location area related to the specific content. -Measurement of multicast signalling strength | Whether there're multiple PDU Sessions corresponding to the same TMGI according to the multicast session information in all UE AS Context | - MBS service related network policy, (e.g. predefined traffic threshold, network status /performance changes); - UE subscription for MBS service changes. - UE moving in or out of MBS Service Area; - Change of user preferences | - UE MBS subscription changes. - Number of UEs requesting the same content in a specific area |
| NFs triggering delivery method switching | UE, SMF, MSF, RAN | RAN, MBSF, Content/Application Server | (MB-)SMF | UE, (MB-)SMF | RAN | UE, RAN, AMF, SMF, UDM, PCF, AF | UE, SMF |
| Procedures for delivery method switching | UE or network initiated PDU Session Establishment or PDU Session Modification procedure | MBSF initiated session update procedure | SMF/UPF initiated new procedure | UE initiated User Multicast Session Join / User Multicast Context Update procedure | RAN triggered delivery method switching from individual to shared | PDU Session Modification procedure | - UE triggered PDU Session Modification procedure. - SMF triggered new procedure |
| Baseline architecture used | 1 | 2 (NOTE 2) | 2 (NOTE 2) | 1 | 1 & 2 | 1 | 1 |
| NOTE 1: Though different architectures are used, architecture 1 and 2 share common functionalities, e.g. in AF, RAN, etc. |  |  |  |  |  |  |  |

2) Switching between PTP and PTM delivery methods

All the three solutions, i.e. Sol #18, #22 and #25 propose that the NG-RAN decides on using PTP or PTM delivery method over radio interface for MBS data, and may dynamically switch between these two delivery methods.

In addition, Sol#18 proposes that the CN provides MBS assistance information to the NG-RAN to assist in the selection of PTP or PTM delivery method. This is pending for the confirmation of RAN WG.

3) Inter-RAN handover / UE mobility related delivery method switching

A comparison between solutions for inter-RAN handover / UE mobility related delivery method switching is shown in Table 7.X-2.

However, which solution(s) is/are chosen still depends on the feedback from RAN WGs and the conclusion of relevant solutions to other KIs (esp. KI#1).

Table 7.X-2: Comparison of solutions for Inter-RAN handover / UE mobility related delivery method switching

|  | Sol#11/#12 | Sol#26 | Sol#27 | Sol#29(/#15) | Sol#39 | Sol#40 (dual RANs case) |
| --- | --- | --- | --- | --- | --- | --- |
| Procedures for delivery method switching | Enhanced Xn HO: MB Session resource setup during Xn Handover preparation or execution phase Enhanced N2 HO: MB Session resource setup during N2 Handover preparation phase | Enhanced Xn/N2 HO: Forwarding multicast data via mapped QoS flow over PDU session from source to target RAN | Enhanced Xn/N2 HO: - Multicast or unicast transport established in target RAN, depending on whether target RAN supports MBS. - Forwarding MBS data to Target RAN supporting MBS. | Enhanced Xn/N2 HO with UE initiated NAS multicast session join. | Multicast->Uni-cast:UE initiated PDU Session establishment or activation; Unicast->Multi-cast: UE initiated PDU session release or deactivation. | Enhanced Xn/N2 HO: - Multicast or unicast transport established in target RAN, based on UE request of unicast or multicast mode in application layer. - Forwarding multicast data over PDU session |
| Baseline architecture used | 2 | 2 | 1 | 1 |  | 1 |
| Based on which solution | 2 | - | 3 | 4 |  |  |

***** End of Changes *****