3GPP TSG-WG SA2 Meeting #141E e-meeting 	S2-2006845

Elbonia, October 12 – 23, 2020	(revision of S2-200xxxx)

Source:	Huawei, HiSilicon

Title:	KI#3: Conclusion for the solutions

Document for:	Approval

Agenda Item:	8.4

Work Item / Release:	FS_eNS_ph2/ Rel-17

Abstract: This contribution introduces the conclusion of all the solutions of KI#3.

1. Introduction/Discussion

According to the evaluation on solutions of KI#3 as described in clause 7.3 of TR 23.700-40 and S2-20xxxxx, Solution #13 could resolve KI#3 with the least impact. It is recommended to use Solution #33 to resolve KI#3. The EN in evaluation part is move to conclusion phase.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-40.

# * * * * First change * * * *

7.3	Evaluation on solutions of KI#3

Editor's note:	This clause will provide some interim evaluation based on solutions #13, #20, #21, #22 that will need further updates to address e.g. roaming aspects.

High level aspects of the solutions:

-	Solution 22 has RAN impact. It lets RAN to enforce the SMBR (Slice Maximum Bitrate).. Currently, RAN is able to be aware of the S-NSSAI of the PDU Session. And RAN is able to be enforce the UE AMBR per UE and GFBR/MFBR per QoS Flow.

-	Solution 13 uses UPF to enforce the DL slice level bitrate. This solution will require to select the same SMF/PCF and UPF for all the PDU Sessions within the slice. It is not necessary to introduce such limitation.

-	Distribution based solutions, i.e. Solution 20&21, let a centralized NF distribute the SMBR into pieces (i.e. Session AMBR and/or MFBR). They have no RAN impact. However, solutions do not explain how to resolve the fact that since the SMBR is distributed into Session-AMBRs, the aggregated SMBR enforced may be smaller than the SMBR, as such the SLA would not be fulfilled, as the UE will be throttled while SMBR is not fully consumed. The situation could be worse when a large amount of PDU Sessions exist as the SMBR is distributed over more Session AMBR.

# * * * * Second change * * * *

8	Conclusions

Editor's note:	This clause will capture conclusions from the study.

8.x	Conclusion on solutions of KI#3

Solution #22 is recommended as the baseline to resolve KI#3.

Editor´s note:  Solution#22 needs to be further checked by RAN WG2 and RAN WG3, due to RAN impacts.

# * * * * End of changes * * * *