SA WG2 Meeting #140-e 	S2-2006851

October 12 – October 23, 2020, Online

Source:	China Telecom

Title:		KI#12_Solution Evaluation and Interim Conclusion for NWDAF-assisted RFSP policy

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_Ph2 / Rel-17

Abstract of the contribution: Interim conclusion for 23.700-91-050 of eNA Key Issue #12

1 Proposal

The aim of key issue 12 is to address the use case #1 NWDAF-assisted RAT/frequency selection. It is to study:

-	The kind(s) of information is made available to the NWDAF for the data analytics, e.g. the information collected for assisting RFSP index) and from which provided NFs.

-	The kind(s) of information should the NWDAF output to the NWDAF consumer as the data analytics.

-	The method(s) for NWDAF consumer of analytics, such as PCF, uses the NWDAF analytics to enhance the RFSP index value assigned to a UE.

It proposes to document the following texts into TR 23.700-91.

* * * * Start of Changes * * * *

8.  Conclusions

8.X 	Conclusion on solutions for Key Issue #12: NWDAF-assisted RFSP policy

Interim conclusion:

It is proposed to take the merged solution, Candidate Solution #4, as the baseline of normative work for KI#12, as it is more reasonable to take user experience into account when using NWDAF to generate analytic for RAT/frequency selection.

And Solution #40 and Solution #41 can be complementary solutions.

* * * * End of Changes * * * *