3GPP TSG-WG SA2 Meeting #141E e-meeting 	S2-2006987

Elbonia, October 12 – 23, 2020	(revision of S2-200xxxx)

Source:	Huawei, HiSilicon

Title:	KI #1: Conclusion

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	FS_5MBS / Rel-17

Abstract: This contribution contains the conclusion of KI#1.

1. Introduction/Discussion

During the Conference call of 5MBS, to conclude Key Issue #1, it was suggested to start from the conclusion proposal from 103 (KI #1 Conclusion, Qualcomm). This document is trying to cooperate such proposal and suggestion some additional concepts/details/characteristics, and is also based on the evaluation of S2-2006986.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.757.

# * * * * First change (All Text New)* * * *

8.1.X	Conclusions

Editor's Note: This conclusion may not be complete, additional concept/details/characteristics could be added.

Based on the evaluation, the following principles are considered as the baseline for normative work:

-	It shall be possible for the application to be unaware of 5GS specific or internal information, e.g., TMGI.

-	UE shall support multicast session join/leave operation via CP (NAS signalling for SM procedure), and UP (IGMP, MLD) Join/Leave. When the join/leave is via UP, the IP multicast address is used as the identification of the multicast session.

-	It shall be possible to associate the unicast PDU Session and multicast session, and provide such association to RAN. The unicast PDU Session is used for 5GC individual MBS traffic delivery when needed.

-	When creating the association, it shall be possible to establish the context for unicast QoS flow for 5GC individual MBS traffic delivery and provide the context to the RAN.

-	MB-UPF acts as the MBS session anchor when 5GC shared MBS traffic delivery method is used, and PSA acts as the unicast session anchor when 5GC individual MBS traffic delivery method is used and PSA receives the MBS data from MB-UPF.

-	It shall be possible to enable AF provide its preference and associating information of the delivery method to the 5GC, for 5GC determining the delivery method.

-	MBS capability of the RAN shall be considered when determining the delivery method.

Editor's Note: Alignment based on progress/decision in RAN WGs will be performed as part of the normative phase.

# * * * * End of changes * * * *