SA WG2 Meeting #141E e-meeting	S2-2007032

Elbonia, October 12 – 23, 2020	(revision of S2-200xxxx)

Source:	China Mobile

Title:	KI#2: Evaluation and Interim conclusion on exposing UE mobility analytics for multiple NWDAFs case

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_Ph2/ Rel-17

Abstract of the contribution: This contribution proposes an interim evaluation and Interim conclusion for KI#2 regarding exposing UE mobility analytics.

1. Discussion

Sol#25 in the TR 23.700-91 address the exposing UE mobility analytics aspect of KI #2 in the multiple NWDAF instances deployment scenario.

-Solution #25: Exposing UE mobility analytics for multiple NWDAFs case

This contribution paper proposes to evaluate and conclude the solutions for the exposing UE mobility analytics aspect of KI #2.

2. Text Proposal

It is proposed the following changes to TR 23.700-91 [2].

# * * * * First change * * * *

7	Overall Evaluation

Editor's note:	This clause will provide evaluation of different solutions.

7.x  Key Issue #2

7.x.x	Exposing UE mobility analytics for multiple NWDAFs case

Solutions #25 proposes a solution of exposing UE mobility analytics for multiple NWDAFs to enhance the existing mechanism of data collection and analytics generation. To obtain UE mobility analytics in Target Area of Interest, the NWDAF could trigger procedure to the target NWDAF serving Target Area of Interest. The target NWDAF could provide data analytics to the NWDAF including SUPI list and their related mobility analytics, who once stayed in Target Area of Interest in target period.

# * * * * Second change * * * *

8	Conclusions

Editor's note:	This clause is intended to list conclusions that have been agreed during the course of the study item activities. This should also capture the guiding principles and documentation approach for creating CRs to normative specifications within the responsibility of SA WG2.

8.x	Key Issue #2: Exposing UE mobility analytics for multiple NWDAFs case

Regarding Exposing UE mobility analytics aspect, it is concluded that the Solution#25 is selected as basis for further normative work.

# * * * * End of changes * * * *