3GPP TSG-SA WG2 Meeting #141E 	S2-2007075

October 12 – 23, Electronic, Elbonia	 (revision of S2-xxxxxxx)

Source:	CATT

Title:	KI#1-Update to solution 5

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_ph2 / Rel-17

Abstract of the contribution: This contribution proposes to update solution#6.

Discussion

Following solutions are touching support of hierarchical architecture among multiple NWDAFs:

Proposal

It is proposed to update the TR 23.700-91.

FIRST CHANGE

## 6.5	Solution #5: AI Model sharing architecture

### 6.5.1	Description

This solution is proposed for Key Issue #1: Logical decomposition of NWDAF and possible interactions between logical functions and Key Issue #19: Trained data model sharing between multiple NWDAF instances.

AI Function (AIF) trains the AI Model(s) and provides AI Model(s) to 5G NFs (e.g. NWDAF). The following figure 6.5.1-1 shows the network data analytics architecture supporting AIF.

Figure 6.5.1-1: Network data analytics architecture supporting AIF

The AIF is decomposed from a NWDAF.

NOTE:	The AIF within 3rd party AF is not considered in this release.

The AIF supporting the following functionalities:

-	AI Model Training;

-	AI Model Repository;

-	AI Model Exposure.

In order to support AIF consumer to discover an AIF instance that is able to provide some specific type of AI Model, each AIF instance should provide the list of AI Model ID(s) that it support when registering to the NRF, in addition to other NRF registration elements of the NF profile. Other NFs requiring the discovery of an AIF instance that provides support for some specific type of AI Models may query the NRF and include the AI Model ID(s) that identifies the desired type of AI Model. The AI Model ID identifies an AI Model and correlates with Analytics ID defined in TS 23.288 [5].

In order to support a specified AI Model training, the AIF may need to collect data from data source (e.g. 3GPP NF(s), data centre).

The AIF service consumer provides the following input parameters listed below.

-	A list of AI Model ID(s): identifies requested AI Model(s);

-	Filter information: indicates the conditions to be fulfilled (e.g. accuracy, performance).

The AIF provides to the consumer the output information listed below.

-	A list of AI Model ID(s);

-	AI Model.

### 6.5.2	Procedures

#### 6.5.2.1	AI Model exposure procedure

Figure 6.5.2.1-1: procedure of AI Model exposure

1.	AIF service consumer (e.g. AF, 5GC NF) requests the AIF for an AI Model indicated by AI Model ID(s). The AIF may need to train the AI Model to meet the requirement. The parameters that can be provided by the AIF service consumer are listed in clause 6.5.1.

2.	When a request for an AI Model received, the AIF either responds with the requested AI model if it has, or responds with time when the AI model will be available if AI model is not ready.

3.	If the AIF needs to train the designated AI model, it collects data from data source (e.g. AF, 5GC NFs) for AI Model training.

4.	The AIF performs AI model training.

5.	The AIF service consumer re-sends the request according to the time indicated by AIF in step 2.

6.	The AIF provides a response including the requested AI model to the consumer.

#### 6.5.2.2	NWDAF analytics service supporting AIF

Figure 6.5.2.2-1: NWDAF analytics service supporting AIF

1.	Consumer NF sends an Analytics request/subscribe including Analytics ID to NWDAF.

2.	In order to provide the requested analytics, the NWDAF may need to determine which AI model is used for the requested analytics. If the NWDAF do not have a proper AI Model for the request analytics, the NWDAF may discover from the NRF the AI Model ID and AIF instance address with requested Analytics ID and send a request to the AIF for the AI Model indicated by the AI Model ID.

3.	The NWDAF may need to collect data from 3GPP NFs for the requested analytics.

4.	The NWDAF derives requested analytics,.

5.	If the analytics result does not meet the requirement (e.g. accuracy), the NWDAF may need to request the AIF to train the AI Model indicated by AI Model ID. The NWDAF may provide the collected data related to the analytics to help the AIF to adjust the AI model.

6.-The NWDAF derives new requested analytics using the updated AI model provided by the AIF.

7.	The NWDAF responses to the consumer with the analytics result.

### 6.5.3	Impacts on services, entities and interfaces

A new network function AIF is introduced. The AIF provides AI Model training and AI Model exposure service.

No impact on current services, entities and interfaces, except:

-	NWDAF requests AI Model from AIF;

-	NF profile in NRF includes list of AI Model ID(s).

End of CHANGE