3GPP TSG-SA WG2 Meeting #141E	S2-2007088

12 - 23 October, 2020, Electronic

Source:	China Mobile，

Title:	KI #3A, Sol #23: Updates to remove ENs.

Document for:	Approval

Agenda Item:	8.5

Work Item / Release:	FS_IIoT/ Rel-17

Abstract: This paper updates solution #23 and proposes to remove the editor’s note from the solution #23.

1. Introduction

The EN is removed. The reason is that the N6 delay measurement is for specific slice and DNN, when UE using the S-NSSAI and DNN for PDU session establishment, the N6 delay is allowed and specific UPF can be selected. The application information e.g. target server IP address, is provided by AF through NEF and the corresponding packet filter rule or PFD will be configured in UPF through N4 rule. The UE gets the target application server IP address through application layer.

2. Text Proposal

It is proposed to capture the following changes in TR 23.700-20.

# * * * * First change * * * *

## 6.23	Solution #23: Transmission Delay Measurement on N6

### 6.23.1	Description

This solution is for key issue#3A,which addresses exposure of deterministic QoS aspects related to:

b)	Ability for AF to indicate periodicity, burst size, burst arrival time (as defined in Rel-16 for TSC Assistance information) and Survival Time, optionally burst spread (variation of burst arrival time for DL traffic resulting from jitter on N6, if applicable) along with Timing Domain (reference for these parameters) associated with these parameters to the NEF.

In Rel-16 standarization, the UL/DL transmission delay in 5G system can be measured based on QoS monitoring mechanism. The main idea is SMF activates the end to end UL/DL packet delay measurement between UE and PSA UPF through encapsulating time stamp in GTP-U header.

For unsynchronized scenario, the RTT/2 can be obtained baesd on sending QoS Monitoring Packet, which is specifically used for UL/DL packet delay measurement. But for N6 case, it is not suitable for asking application server to create specific packet for delay measurement. And for this IIoT scenario , it is supposed that the equipments in N6 support deterministic transmission, therefore the time synchronization is required.

This solution proposes a mechanism to measure the delay on N6 with the pre-condition that the application server deployed in N6 supports for time synchronization.

In this solution, the first step is to realize the time synchronization between PSA UPF and the application server before or after the PDU session establishment. Other solutions for key issue 3B can be considered to realize this.

During the PDU session establishment, the SMF sends N4 rule to PSA UPF for N6 delay measurement.The rule may include traffic direction, IP 5 tuples, the frequency of N6 delay measurement and observation period. For the UL data, the N4 rule also includes construction of extension header for carrying N6 delay measurement indication. For the DL data, the N4 rule also includes extracting the timestamp from extension header and removing the extra header.

Then PSA UPF can insert the delay measurement indication in the UL data packet when the UE sending the packet. If the packet supports IPv6 routing, the delay measurement indication is carried on IPv6 extension header. While if the packet supports IPv4, the delay measurement indication can be carried on e.g. In-band Network Telemetry header which is referred to https://p4.org/assets/INT-current-spec.pdf. The PSA UPF is responsible for constructing IPv6 extension header or INT header. There is a pre-agreement between 5GS and application server that what kind of packet header should be selected.

The UE may use specific S-NSSAI/DNN for the PDU session establishment and the N4 rule related with N6 delay measurement will be sent by SMF during the PDU session establishment procedure and specific UPF which can enable the N6 delay measurement may be selected by SMF. AF may provide application server IP address or PFD through NEF for packet filtring rule generation in SMF/UPF. Therefore when UE sends specific paket which intents for the target application server in the PDU session, the N6 delay measurement will be triggered by UPF, and the UE gets the target application server IP address through application layer mechanism.

When the application server in N6 accepts the UL packet and identifies the delay measurement indication, the application server should record this packet's IP 5 tuples. When the application server sends the DL data which matches the recording IP 5 tuples, the application server should insert the sending timestamp in the corresponding IPv6 extension header or INT header. PSA UPF extracts the sending timestamp and calculates the N6 transmission delay.

Therefore 5GS can calculate the accurate packet burst arrivel time of RAN side by adding burst sending time provided by AF (already known by network in current specifications), N6 transmission delay and CN PDB.

### 6.23.2	Procedures

The N6 transmission delay measurement procedure is described in Figure 6.23.2-1.

# Figure 6.23.2-1 Procedure for N6 transmission delay measurement

1.	PSA UPF and application server supports IEEE 802.1AS, and can be synchronized before or after the PDU session establishment.

2.	UE initiates PDU Session Establishment Request, SMF sends N4 rule to UPF through N4 session establishment request or N4 session modification request. The N4 rule may include traffic direction, IP 5 tuples, the frequency of N6 delay measurement and observation period. For the UL data, the N4 rule also includes construction of IPv6 extension header or INT header for carrying N6 delay measurement indication. For the DL data, the N4 rule also includes extracting the timestamp in IPv6 extension header or INT header.

3.	UE sends a UL data packet.

NOTE:	The UL data packet contains normal application data that the application anyway sends; i.e. the UE does not create any special data packets.

4.	UPF detects that the destination address of the UL data packet is the application server on the N6 port, UPF constructs the extention header and inserts the N6 transmission delay measurement indication in the specified IPv6 extension header or INT header, and trigger the N6 transmission delay measurement.

5.	The UL data packet is sent from PSA UPF to the application server.

6.	When the server receives the UL data packet, it recognizes the N6 delay measurement indication in the extension header and decides to start the N6 delay measurement for the service flow. When the server sends DL data packets, it will add a timestamp representing the sending time in the corresponding place in the extension header.

7.	When PSA UPF receives the DL data packets, it extracts the timestamp from the extension header and calculates the unidirectional downlink N6 delay according to the packet receiving time and the timestamp. PSA UPF removes the extention header.

8.	The DL data packet is sent from PSA UPF to UE.

9.	UPF reports the calculated N6 transmission delay to SMF through N4 Association Update Procedure.

10.	SMF calclulates the Burst Arrival Time for RAN based on AF providing burst sending time, N6 transmission delay and CN-PDB.Then SMF sends the updated Burst Arrival Time to RAN.

### 6.23.3	Impacts on Existing Nodes and Functionality

Editor's note:	This clause captures impacts on existing 3GPP nodes and functional elements.

SMFs:

-	Configure the UPF for N6 delay, and send the corresponding N4 rules to UPF.

-	Calculate the precise Burst Arrival time with considering N6 delay.

# UPF:

-	Support the synchronization with application server.

-	Based on the N4 rules, the UPF can detect the corresponding data packets and construct the IPv6 extension header or INT header for UL packet. And the N6 delay indication can be carried on the extension header.

-	Extract the timestamp in DL packet and remove IPv6 extension header or INT header

-	Calculate the N6 transmission delay periodically and report to SMF.

# * * * * second of changes * * * *