3GPP TSG-SA WG2 Meeting #141E 	S2-2007184

12 – 23 October, 2020, Elbonia	 (revision of S2-200xxxx)

Source:	CATT

Title:	KI #4: Evaluation and conclusion

Document for:	Approval

Agenda Item:	8.4

Work Item / Release:	FS_eNS_ph2 / Rel-17

Abstract of the contribution: this paper proposes to evaluate the solutions for KI#4, summarize the common part of the solutions and make conclusion based on the common part.

Discussion

This pCR is proposed to evaluate the solutions for KI#4, summarize the common part of the solutions and make conclusion based on the common part.

Based on the Table 6.0-1 in TR 23.700-40, the solutions proposed to solve KI#4 are shown in Table 1.

Table 1: Solutions proposed to solve KI#4

| Solution#'s | Solution Titles | Key Issue#'s |
| --- | --- | --- |
| 8 | AMF and O&M based solution | 1, 2 & 4 |
| 9 | Monitoring multiple quotas of number of UEs/PDU Sessions per S-NSSAI at NWDAF | 1, 2 & 4 |
| 18 | Proactive Slice Quota Management in AMF | 1, 2, 4, 5 |
| 23 | Network slice quota event notification | 4 |
| 33 | Event notification of Slice SLA attributes | 4 |
| 34 | AF interaction for event notification | 4 |
| 43 | UE Slice Maximum Bit Rate related event notification | 4 |

The analyses of the solutions in Table 1 are shown in Table 2.

Table 2: Analyses of Solutions proposed to solve KI#4

| Solution#'s | Triggers for Event Report | Even consumer | Event provider | How to subscribe the event |
| --- | --- | --- | --- | --- |
| 8 | Max number or threshold reached (or went below) | O&M | AMF which is responsible for UE number and PDU Session number counting. | In scope of SA5 |
| 9 | UE quota is reached | NWDAF | AMF which is responsible for UE number counting | Via Nnwdaf interface |
| 18 | Quota information is overflown or periodical timer expires or notification event is met | AF | SQM which manages the slice quota within the PLMN | Via NEF |
| 23 | Threshold for quota notification reached or periodic reporting. The event related to the following slice attributes: max number UE, max number of PDU Session, max UL or DL data rate per slice per UE. | AF | NSQ-NF which resides with network entity that is responsible and/or holds the up-to-data network slice quota information. | Via NEF |
| 33 | Threshold for quota notification reached or periodic reporting event. The event related to the following slice attributes: Maximum number of UEs per S-NSSAI, maximum number of PDU sessions per S-NSSAI, maximum UL/DL data rate per S-NSSAI | AF, OAM, 5GC NF | Primary PCF which is responsible to maintain the global quota status of slice attributes. | Via NEF |
| 34 | Event notification on slice related quotas, e.g. a specified threshold is reached | AF | Slice PCF which supports network slice related quota management | Via NEF |
| 43 | UE Slice Maximum Bit Rate reached | AF | NG-RAN which is responsible for UE slice bit rate control | Via NEF |

Based on the analyses in Table 2, there are five solutions support that AF subscribes slice attributes related event notification via NEF. Three solutions support that NF (e.g. NWDAF, OAM) subscribes slice attributes related event notification. All solutions support the NF which is responsible for quota control acts as event provider. The difference is that the NF acting as event provider is different. The event report triggers include the maximum number or threshold is reached and periodical timer expires. The event report is related to the slice attributes of maximum number of UEs, maximum number of PDU Sessions, maximum UL/D L data rate per S-NSSAI and UE Slice Maximum Bit Rate.

Based on the analyses above, it is proposed to make the following conclusion for KI#4:

-	AF and NF (e.g. 5GC NF, OAF) can subscribe the event report related to network slice attributes. The AF subscribes the event report via NEF.

-	The event report is generated by the NF which is responsible for network slice attribute control/counting. Which NF controls or counts the attributes depends on the conclusion of KI#1, 2, 3 and 5.

-	The network slice attributes include maximum number of UEs, maximum number of PDU Sessions, maximum UL/DL data rate per S-NSSAI and UE Slice Maximum Bit Rate. The event report includes the maximum number or threshold of a network slice attribute is reached or the periodical timer expires.

Proposal

This pCR is proposed to evaluate the solutions for KI#4, summarize the common part of the solutions and make conclusion based on the common part.

First change

7.x	Evaluation on solutions of KI#4

The solutions of KI#4 include solutions #8, 9, 18, 23, 33, 34 and 43. The analyses of the solutions are shown in Table 7.x-1.

Table 7.x-1: Analyses of Solutions of KI#4

| Solution#'s | Triggers for Event Report | Even consumer | Event provider | How to subscribe the event |
| --- | --- | --- | --- | --- |
| 8 | Max number or threshold of UE number or PDU Session number reached (or went below) | O&M | AMF which is responsible for UE number and PDU Session number counting. | In scope of SA5 |
| 9 | UE quota is reached | NWDAF | AMF which is responsible for UE number counting | Via Nnwdaf interface |
| 18 | Quota information is overflown or periodical timer expires or notification event is met | AF | SQM which manages the slice quota within the PLMN | Via NEF |
| 23 | Threshold for quota notification reached or periodic reporting. The quota relates to the following slice attributes: max number UE, max number of PDU Session, max UL or DL data rate per slice per UE. | AF | NSQ-NF which resides with network entity that is responsible and/or holds the up-to-data network slice quota information. | Via NEF |
| 33 | Threshold for quota notification reached or periodic reporting event. The quota relates to the following slice attributes: Maximum number of UEs per S-NSSAI, maximum number of PDU sessions per S-NSSAI, maximum UL/DL data rate per S-NSSAI | AF, OAM, 5GC NF | Primary PCF which is responsible to maintain the global quota status of slice attributes. | Via NEF |
| 34 | Event notification on slice related quotas, e.g. a specified threshold is reached | AF | Slice PCF which supports network slice related quota management | Via NEF |
| 43 | UE Slice Maximum Bit Rate reached | AF | NG-RAN which is responsible for UE slice bit rate control | Via NEF |

Based on the analyses in Table 2, there are five solutions support that AF subscribes slice attributes related event notification via NEF, three solutions support NF (e.g. NWDAF, OAM) subscribes slice attributes related event notification. All solutions support the NF which is responsible for quota control acts as event provider. The difference is that the NF acting as event provider is different. The event report triggers includes the maximum number or the threshold of quota of network slice attributes is reached or periodical timer expires. The slice attributes includes maximum number of UEs, maximum number of PDU Sessions, maximum UL/D L data rate per S-NSSAI and UE Slice Maximum Bit Rate.

Second change

8	Conclusions

8.x	Conclusion of KI#4

Conclusions of KI#4 include:

-	AF and NF (e.g. 5GC NF, OAM) can subscribe the event report related to network slice attributes. The AF subscribes the event report via NEF.

-	The event report is generated by the NF which is responsible for network slice attribute control/counting.

NOTE:	Which NF(s) controls or counts the attributes depends on the conclusion of KI#1, 2, 3 and 5.

-	The event report includes the maximum number or threshold of the quota of network slice attributes is reached or the periodical timer expires. The network slice attributes include maximum number of UEs, maximum number of PDU Sessions, maximum UL/DL data rate per S-NSSAI and UE Slice Maximum Bit Rate.

End of changes