3GPP TSG-SA2 Meeting #141e			S2-2007313

E-meeting, 12 – 23 October 2020

Title:	[DRAFT]  Reply LS on Clarification of CAG only UE accessing EPS network

Response to:	S2-2006801 / C1-205222

Release:	Release 16

Work Item:	Vertical_LAN

Source:	SA2

To:	CT1

Cc:	RAN2

Contact person:

#### Name:	Peter Hedman

E-mail Address:	Peter (dot) hedman (at) ericsson (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	S2-2007314

1. Overall Description:

SA2 thanks CT1 for the LS on Clarification of CAG only UE accessing EPS network.

CT1 asked "whether a UE configured to access 5GS only via CAG Cell(s) can select an E-UTRAN cell and register to EPS"

SA2 answer:

The CAG-only indication is only applicable for 5GS and does not impact whether a UE can or cannot register to EPS. Whether the UE can select EPS is controlled by “Core Network type restriction” i.e. a PLMN operator restricting the access to 5GC sets it to “5GC only” and the UE gets rejected when trying to access EPC.

SA2 approved the attached CR clarifying the above.

2. Actions:

To CT1:

ACTION: 	SA2 asks CT1 to take the above information into account.

3. Date of Next SA2 Meetings:

3GPP SA2#142e	16th – 20th November 2020	e-meeting