3GPP TSG-WG SA2 Meeting #141E e-meeting 	S2-2007342

Elbonia, October 12 – 23, 2020	(revision of S2-200xxxx)

Source:	Qualcomm Incorporated

Title:	Sol#9 Update: clarifications of solution 9.

Document for:	Approval

Agenda Item:	8.7

Work Item / Release:	FS_ID_UAS / Rel-17

Abstract: This contribution proposes some clarifications for Solution #9.

# 1. Discussion

The P-CR introduces simplifications and clarifications to solution #9, considering also that some part of the solution has been merged into others.

The LS from ACJA received by 3GPP indicates the following:

UAV identification at USS level

ASTM and the Federal Aviation Administration (FAA) Notice of Proposed Rule Making (NPRM) have defined three types of UAV identities to be used for UAV identification in the framework of Remote Identification. In addition to Serial Number and UUII contained in the ASTM standard, a dynamic Session ID as defined in the FAA NPRM is also considered.

It has been brought to GUTMA’s attention that 3GPP has been discussing solutions in which the Session ID may be dynamically allocated by the USS to the UAV operator (and thus their UAV) in collaboration with the SDSP (the 3GPP Mobile Network Operator or MNO). This offers interesting opportunities for greater collaboration and data sharing between the USS and SDSP MNO, both in support of the defined intent of the Remote ID and the wider location, tracking, and data services to the UAV operator. This can be e.g. used in order to enable routing of queries by a ground entity (e.g. police car) to the SDSP based on the Session ID received in the Broadcast Remote Identification. As an example, this would enable the querying entity to retrieve the UAV location and additional information not broadcasted (e.g. the full UAV identity), and for the SDSP MNO to route the query to the USS in order to retrieve further UAV-related information. The format of such Session ID would of course be defined by the USS community and not the MNO specifically, to ensure consistency for an USS using multiple MNOs. It should be noted that with this mechanism the Session ID is tied to the MNO for the lifetime of the Session ID.

GUTMA sees the potential with exploring such an optional approach for allocation and expanded use of the Session ID.

Solution #9 addresses exactly this scenario, and clarifications are added to address it.

# 2. Proposal

It is proposed to adopt the following changes in TR 23.754.

* * * * First Change * * * *

## 6.9	Solution #9: UAV Identities Allocation and Usage for Remote Identification

### 6.9.1	Introduction

The solution defines a set of mechanisms to enable a UAV to obtain an identity to be used for Remote Identification transparently to the 3GPP system or with the support of the 3GPP system. Such identity is used for identification of the UAV towards the UTM and the TPAEs.

The solution assumes that the CAA-Level UAV ID is defined outside 3GPP, however proposes functional assumptions for such identity, and describes its usage in the 3GPP system.

Though the details of the CAA-Level UAV ID format used to identify a UAV with the USS/UTM, and used for Remote Identification, are defined outside of 3GPP, the use of such identity for enabling Remote Identification via the 3GPP system are highly important to the system design for 3GPP. Specifically, how such identity is used to enable a TPAE to query about UAV information using UAV2 needs to be defined with respect to the 3GPP system, and a solution identified as to which entity resolves the CAA-Level UAV ID to a network function that can reply to such queries.The solution proposes multiple options for the CAA-Level UAV ID assignment and resolutions.

The solution also addresses network publishing aspects for Remote Identification.

### 6.9.2	Functional Description

The following clauses describe the building blocks of the solution.

#### 6.9.2.1 Adoption of a UAV Flight Enablement Subsystem (UFES)

In this solution, the UAV Flight Enablement Subsystem defined in solution 5 in clause 6.5.2.1is a 3GPP subsystem that interfaces the 3GPP System with the USS and support that USS in performing UAV Remote Identification and tracking. In addition, the UFES terminates UAV2 towards a TPAE, either directly or via NEF/SCEF interfacing.

NOTE:	In some 3GPP network deployments, the UFES may actually act as the USS, for 3GPP operators that choose to provide USS services for UAVs. In such cases, UFES and USS are a single functional entity in the 3GPP MNO network.

In addition to the functionality described in clause 6.5.2.1, the UFES can perform the following functions:

-	provides remote identification and tracking information to TPAEs/USS/UTM that may be monitoring a UAV, upon request from the TPAEs/USS/UTM. Per Civil Aviation Authority regulation (e.g. FAA), The remote identification and tracking information can include identification information of the UAV, UAV controller, UAV pilot/operator, UAV pre-configured or historic flight path, UAV and its UAV controller location and operation time information, UAV operation status ( such as battery level if in auto-pilot mode), so on

-	support the USS in the 3GPP-assisted CAA-Level UAV ID assignment described in clause 6.9.2.3.

#### 6.9.2.3 Functional Assumptions about the CAA-Level UAV ID for Remote Identification

From a functional point of view, though the actual format of the CAA-Level UAV ID is defined outside 3GPP and is not decided by 3GPP, it is assumed that the CAA-Level UAV ID used for Remote Identification contains at least the following information:

-	an identity unique to the UAV, which may preferably h temporary validity: this identifies uniquely the UAV with the entity that allocates the CAA-level UAV ID. The identity formats identified in the conclusions in clause 8 are supported: Serial Number Identification, a CAA-Issued Registration Identifier (aka Session ID), and USS/UTM-Issued UUID.

NOTE 1:	The details of such identity are defined outside 3GPP. Whether a single identity is used, or a set of identities through which the UAV cycles periodically (similarly to the V2X design) is FFS.

NOTE 2:	Whether privacy or confidentiality requirements will apply to the unique UAV temporary identity depends on regulations in various regions. Solutions may be identified by SA3 or externally to 3GPP, as in the case of V2X.

-	CAA-level UAV ID Routing Information, used by an entity attempting to retrieve the UAV data (e.g. the TPAE) to identify and address the appropriate entity where to send the query. In solutions for UAV authentication and authorization (e.g. solution 5 in clause 6.5), this is also used by functions in the 3GPP system to identify the appropriate USS that serves the UAV. The exact format of the CAA-Level UAV ID Routing Information is defined outside 3GPP, but the use is based on the USS Discovery description in clause 6.5.2.3.

#### 6.9.2.4	CAA-Level UAV ID assignment

Two potential types of CAA-level UAV ID assignment can be considered:

1.	USS-assigned CAA-Level UAV ID: the ID is assigned completely at USS level, and uses one of the formats defined in 6.9.2.4.2.

2.	3GPP-assisted CAA-Level UAV ID assignment: in this scenario

-	the USS e.g. delegates to the UFES the role of "resolver" of the CAA-Level UAV ID, and return to an entity (e.g. the TPAE) querying information about the UAV based on the CAA-Level UAV ID the UAV data that the UFES retrieves from the USS.

-	The CAA-Level UAV ID is still assigned by the USS, but with the support of functionality of the 3GPP system. The USS decides the specific identity, and  the UFES may provide the "CAA-level UAV ID Routing Information" to enable a resolver of the CAA-level UAV ID to resolve to the UFES, if such function is delegated by the USS to the UFES.

-	In case of 3GPP-assisted CAA-Level UAV ID assignment, the UFES interacts with the USS to allocates the UAV identities to be used for Remote Identification (i.e. the CAA-Level UAV ID).

It has to be noted that a UAV is already aware of the different types of identity that may be assigned by a USS, thus this solution does not require further awareness of the UAV regarding which entity assigns the CAA-Level UAV ID.

NOTE:	It is assumed that the UAV is not aware of which assignment mechanisms is used for the CAA-Level UAV ID.

##### 6.9.2.4.1	3GPP Assisted CAA-Level UAV ID Assignment

The solution for the specific scenarios of 3GPP Assisted CAA-Level UAV ID Assignment considers the allocation to the UAV of a CAA-Level UAV ID by the USS in collaboration with the UFES, for the use by the UAV for UAV authentication and authorization with the 3GPP system and the USS, and for the use for Remote Identification.

The CAA-Level UAV ID allows a TPAE to identify and address the network function from which the UAV registration information with the civil aviation authority (e.g. FAA) can be retrieved.

NOTE:	It is assumed that the TPAE has authorization and security agreements in place to be able to perform such retrieval. This is out of scope of 3GPP.

In particular, the CAA-Level UAV ID is used by the TPAE to identify a network function that can "resolve" the CAA-Level UAV ID to the actual UAV information, e.g. UAV serial number, pilot information, UAS operator, etc. Such ID resolver receives the query from the TPAE, containing information including the CAA-Level UAV ID, and identifies to which UAV the query relates.

When the UAV registers with the USS before registering to a 3GPP system, the UAV operator provides information about the serving PLMN to the USS. In order to allocated a CAA-Level UAV ID, the USS interacts with the 3GPP network if 3GPP Assisted CAA-Level UAV ID Assignment is desired, using a NEF/SCEF service. The 3GPP network selects a UFES to respond to the USS, and the UFES provides to the USS the CAA-Level Routing Information to enable an entity attempting to resolve the identity received in a Broadcast Remote ID message to address the UFES.

When the UAV registers with the 3GPP network and is authenticated/authorized, or when PDN connectivity/PDU session is established for USS and/or C2 connectivity, the serving UFES provides the same information to the USS, and the USS would use it if a new CAA-Level UAV ID is allocated.

In this option, the USS acts as the CAA-Level UAV ID resolver, but obtains the routing information part of the CAA-Level UAV ID from the UFES, thus queries from the TPAE are directed to the UFES.

#### 6.9.2.5	Resolution of UAV Identity for Broadcast Remote Identification

##### 6.9.2.5.1	USS-assigned CAA-Level UAV ID

If the CAA-Level UAV ID is assigned by an USS according to mechanisms and formats defined in other standards, e.g. ASTM, then the CAA-Level UAV ID resolution must be based on mechanism and network infrastructure solutions defined outside of 3GPP.

If the USS assigns the CAA-Level UAD ID implementing mechanisms that can be based on ProSe discovery concepts, then the identity resolution takes place as in the case of 3GPP Assigned CAA-Level UAV ID or 3GPP Assisted CAA-Level UAV ID Assignment.

##### 6.9.2.5.2	3GPP Assisted CAA-Level UAV ID Assignment

When the UAV performs a registration with the USS with the UFES involvement:

1.	The UE obtains the CAA-Level UAV ID from the USS, and uses the CAA-Level UAV ID for Remote Identification messages

2.	Monitoring entities (e.g. TPAE) use the CAA-Level UAV ID to identify the "resolver" for the identity. This, depending on the format of the CAA-Level UAV ID, may happen via DNS or other mechanism. Once the TPAE identifies the network function to contact, it sends a query containing the CAA-Level UAV ID to obtain the UAV information.

NOTE:	Monitoring entities are entities, as defined e.g. in [2], entities that monitor the Broadcast Remote Identification messages from UAVs, and do not required to be assigned to a UAV or associated to a UAV.

#### 6.9.2.6	Applicability for Network publishing Remote Identification and tracking

When 3GPP system is used to provide the network publish function for UAV remote Identification as defined by the CAA, UFES can be used to coordinate with UAV and the other 3GPP functions to provide UAV remote Identification and tracking information to TPAE or USS/UTM.

In case of in case of 3GPP Assisted CAA-Level UAV ID Assignment, it is assumed that the mapping between USS assigned CAA-level UAV ID and the associated 3GPP UAV ID is known and stored by the UFES after UAV being authorized by the USS/UTM, via mechanisms defined in other solutions. If UFES receives a remote identification and tracking query with the USS-assigned CAA-Level UAV ID, the UFES uses the mapped 3GPP UAV ID to coordinate with different 3GPP functions to collect the UAV remote identification and tracking information. In addition, the UFES can retrieve aviation-level information (e.g. pilot information, USS operator, etc.) from the USS to provide it to the querying party (e.g. TPAE).

There are two ways for UFES to collect UAV's remote identification and tracking information:

1.	Direct communication between UAV and UFES, such as using two-way query/response communications between UFES and UAV to allow UAV to provide the UAV remote Identification and tracking information, as well as the information for networked UAV controller; or allowing UAV and network UAV controller periodically send their remote identification information to the UFES. CAA-Level UAV ID is used for the communication between UAV and UFES.

NOTE:	The communication between UAV and UFES can be either Control-plane based using NAS message with an aviation container which contains the UAV tracking information and can be transparent to 3GPP system; or user-plane based which UAV/network UAV controller communicate with UFES using internet-based application level protocol via the data plane established for UAV.

2.	UFES uses existing 3GPP services, such as network exposure function or LCS, to track the UAV and provide the information to TPAE or UTM per request. UFES may use the mapped 3GPP UAV ID for the interaction with different 3GPP functions.

### 6.9.3	Procedures

#### 6.9.3.1	Use of CAA-level UAV ID for Broadcast Remote Identification

This clause provides an example of how the CAA-Level UAV ID is used for Broadcast Remote Identification.

Figure 6.9.3.1-1: Use of CAA-Level UAV ID For Broadcast Remote Identification.

1.	The UAV generates the following for Remote Identification broadcasting:

-	UAV Identification: this is the CAA-Level UAV ID which is expected to be protected against spoofing (i.e. to avoid impersonation) and for confidentiality (i.e. protected between the UAV and the UFES/USS), and verifiable by the TPAE by querying the UFES/USS to retrieve the UAV information (e.g. UAV Hardware ID, UAV Pilot, UAV Operator, etc.)

-	UAV Flight Information: this corresponds to dynamic flight information (e.g. height, direction, speed, etc.), protected for spoofing and verifiable by the received party (e.g. a TPAE or another UAV).

Editor's note:	How the information in the RID broadcasting is protected is FFS and shall be defined by SA3 or outside 3GPP.

2-3. Upon receiving the UAV Broadcast Remote Identification information, a receiving entity (e.g. UAV) verifies the validity of the Flight Information, and may use such information for e.g. collision avoidance.

4-5. Upon receiving the UAV broadcast Remote Identification information from an UAV, the TPAE identifies the relevant UFES/USS using the the CAA-Level UAV ID as described in clause 6.9.2.5 and based on the CAA-Level UAV ID functional assumptions.

6.	The TPAE discovers the UFES/USS address (e.g. via DNS if the UFES/USS Routing Information is an FQDN), and requests information verification from the UFES/USS by providing the information received in the Broadcast Remote Identification information. In case of USS assigned CAA-Level UAV ID, the query is routed directly to the USS. In case of 3GPP-assisted CAA-Level UAV ID assignment, the query is routed to the UFES which in turn interacts with the USS, as needed, and retrieves the UAV information (UAV Hardware ID, UAV pilot, UAV operator, etc.) and returns it to the TPAE.

### 6.9.4	Impacts on services, entities and interfaces

Editor's note:	This clause lists impacts to services, entities and interfaces.

* * * * End of Change * * * *