3GPP TSG-SA/WG2 Meeting #141E	S2-2007372

# Elbonia, Oct. 2020, Electronic meeting

Source:	ZTE

Title:	KI#7, Sol#44 update

Document for:	Approval

Agenda Item:	8.4

Work Item / Release:	FS_eNS_Ph2 / Rel-17

Abstract of the contribution: This contribution propose to update solution #44 to add another option for key issue#7

2	Proposal

It is proposed to agree the following changes in TR 23.700-40.

************* Start Changes *************

6.44	Solution #44: RAN controlled steering of the UE to a network slice in a different frequency band

6.44.1	Introduction

This solution addresses the below requirements from Key Issue #7: Support of 5GC assisted cell selection to access network slice.

-	How does 5GS steer UEs to a 5G-AN (e.g. a specific frequency band) that can support the network slices that the UE can use.

-	What information does 5GS need to take a decision to steer UE to a proper 5G-AN.

6.44.2	High Level Description

The solution is based on the following high level assumptions and principles:

-	The UE is allocated Allowed NSSAI which are supported by all cells in the Registration area.

-	The NG-RAN is configured by O&M with RRM policies related to an S-NSSAI.

6.44.3	Procedures

Figure 6.44.3-1 - Steering the UE to a network slice in different frequency band

1)	The UE is in idle mode, registered via RAN-1 for S-NSSAI-1, which preferably operates only in frequency band 1 (FB-1) and S-NSSAI-2, which preferably operates in frequency band 2 (FB-2).

2)	An application in the UE needs to establish service on S-NSSAI-2. S-NSSAI-2 is defined in the network to prefer to use FB-2.

3)	The UE establishes RRC connection with RAN-1.

4)	The UE triggers PDU Session Establishment Request on S-NSSAI-2 via RAN-1, and PDU Session Establishment continues towards SMF as per current specifications.

5)	SMF accepts the PDU session.

6)	AMF forwards SM message from SMF to NG-RAN

The NG-RAN takes the decision whether to perform option A or B or C.

Option A (e.g. used if the S-NSSAI-2 is preferred to be served by current FB-2 and the FB-1 can accept the PDU session):

7a)	PDU Session establishment procedure on S-NSSAI-2 via RAN-1 as per current specifications.

8a)	RAN-1 triggers inter-frequency cell change to RAN-2 which is preferred for S-NSSAI-2,

Option B (e.g. used if UE CP should not be moved):

7b)	NG-RAN adopts the AS and UP configurations to serve the new UP for S-NSSAI-2 e.g. DC/CA

8b)	PDU Session establishment procedure on S-NSSAI-2 via RAN-1 as per current specifications.

Option C (e.g. used if the S-NSSAI-2 is preferred to be served by current FB-2 and the FB-1 cannot accept the PDU session):

7c)	the RAN-1 rejects the N2 request and triggers the intra AMF and inter-frequency cell change to RAN-2 which is preferred for S-NSSAI-2.

8c) after the handover the AMF resends the N2 request towards RAN-2 as per current specifications

9)	After PDU session on S-NSSAI-2 is deactivated or released, the NG-RAN steers the UE accordingly to the policies for the Allowed NSSAI/ RFSP.

6.44.4	Impacts on services, entities and interfaces

No impacts

*************** End Changes ***************