SA WG2 Meeting #141E	S2-2007480

12 October - 23 October 2020, Elbonia

Source:	Nokia, Nokia Shanghai Bell, NTT DOCOMO

Title:	KI#2, Evaluation: Observation on solutions addressing NWDAF interactions in hierarchical NWDAF architecture

Document for:	Discussion/Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_Ph2 / Rel-17

Abstract of the contribution: Proposes several observations derived from the categorization of the solutions marked as related to Key Issue #2.

# 1		Discussion

It is proposed to include the following evaluation on KI#2 into TR 23.700-91.

# 2	Proposal

It is proposed to update TR 23.700-91 as follows:

*** Change 1 (all new text) ***

*** (to be added right after Table 7.2-b Solutions focusing on Analytics / data aggregation in hierarchical NWDAF architecture) ***

From table 7.2-x and table 7.2-b, the following observations are made:

Observation O1: Most solutions apply a hierarchical NWDAF architecture with at least 2 layers. Solutions #14 (option A) and #53 also describe an option allowing for a flat NWDAF architecture, whereby benefits of such flat NWDAF architecture are not described.

Observation O2:  Most solutions support a dynamic NWDAF architecture. Solutions #16, #17, #18 support a static NWDAF hierarchy with a dynamic NWDAF to hierarchy mapping. Solutions #12, #14, #23, #53 explicitly mention about the possibility to do some pre-configuration of the NWDAF instances, e.g. based on operator's policy to serve a specific set of entities, while this is assumed to also be possible for other solutions.

Observation O3: Solutions #10, #11, #12, #14, #17, #19, #23, #24, #53, #60 describe collection and aggregation of Analytics Output from other NWDAFs. Solutions #10 and #60 in addition describe the exchange of Aggregation metadata to support the aggregation of Analytics Output obtained from multiple distributed / lower layer NWDAFs.

Observation O5: All solutions, except one option in solution #11, use existing NRF mechanisms to discover other NWDAFs.

Observation O6: Solutions that are proposing to extend the NWDAF capabilities (e.g. analytics aggregation) extend the NWDAF NF Profile in NRF to include the new NWDAF capabilities.

Observation O7: Solution #16, describes a use case where the NWDAF service consumer contacts a lower layer NWDAF, while the procedure in solution #18, shows the case where the NWDAF service consumer contacts the top layer NWDAF. Solution #60 mentions that NF service consumer can be configured to give preference to a distributed NWDAF over a Central NWDAF, in case multiple NWDAFs are returned by NRF.

Observation O8: Solution #17 mentions that "NRF will return the best matching NWDAF in the hierarchy that can support the requestes Analytics". This implies a change to the existing NRF behaviour to determine "the best matching NWDAF" and to know and understand the NWDAF hierarchy.

*** End Change ***