SA WG2 Meeting #S2-141E	S2-2007497

12 October - 23 October 2020, Electronic, Elbonia	(revision of S2-200xxxx)

Source:	ZTE

Title:	KI#1, sol#17, update the description and remove the Note

Document for:	Approval

Agenda Item:	8.5

Work Item / Release:	FS_IIoT/ Rel-17

Abstract of the contribution: This paper update the solution description to correct the editorial error and remove the Note that is not fully correct.

1. Discussion

This paper corrects some editorial error and adds IEEE 1588.

There is a Note in the solution 17 for the optimization:

NOTE:	The exchange of Announce messages inside 5GS can be minimized by DS-TT's or NW-TT's sending Announce messages from a DS-TT in SlavePort state to the NW-TT inside 5GS only when there are changes comparing with the previous messages. The changes may include Announce message receipt timeout events. In this case, NW-TT and DS-TT with port(s) with MasterPort role(s) can generate Announce messages based on previous the Announcement message information.

It means:

- if there is no change, the NW-TT/DS-TT port does not send the received Announce message to NW-TT BMCA function

But for the NW-TT BMCA function, it cannot distinguish the case:

- DS-TT/NW-TT port receives the Announce message which is not changed comparing with previous messages.

- DS-TT/NW-TT port does not receive the Announce message;

- When the Announce message receipt timeout events, NW-TT and DS-TT with port(s) with MasterPort role(s) can generate Announce messages based on previous the Announcement message.

But this optimization does not work, when the NW-TT BMCA function determines the state of port is changed from MasterPort to PassivePort.

So there is technical issue on the optimization, it proposes removing the Note.

2. Proposal

It proposes to agree the following change to the TR.

*****************************************START of CHANGE **********************************************

6.17	Solution #17: U-plane BMCA solution for the key issue#1

6.17.1	Introduction

The solution is proposed to solve Key Issue #1: the UL Time Synchronization. Specially, this solution resolve the BMAC part of UL Time Synchronization. It can work with existing solution#1.

In the IEEE 802.1AS [6] or IEEE 1588-2008 [13], before the time synchronization is performed, the time-synchronization spanning tree shall be established first for particular TSN time domain. The spanning tree is to 1) avoid the message loop for time synchronization and 2) determine the port state of the (g)PTP instance.

There are two method to establish the time-synchronization spanning tree:

-	BMCA (best master clock algorithm).

-	The port states and grandmaster are configured.

To support the BMCA in the 5GS logical TSN Bridge, a centralized handling is required.

6.17.2	Functional Description

In the figure 6.17.2-1, it introduces a BMCA function in the NW-TT. All the BMCA related process is handled by BMCA function

Figure 6.17.2-1: 5GS TNS bridge support BMCA

When the DS-TT port or NW-TT port receives the Announce message, it forward the message to BMCA function in U-plane.

The BMCA function run the BMCA algorithm according to 802.1AS [6] or IEEE 1588-2008 [13]. The BMCA function:

-	determines the GrandMaster clock for a clock domain;

-	determines and stores the port state of DS-TT port and NW-TT port of the 5GS TSN bridge;

-	update the Announce message and send it out via the port.

6.17.3	Procedures

The procedure for BMCA is described in Figure 6.17.3-1. It assume there are 3 GM clock candidates. The clock-1 resides in the network connect to the DS-TT1/UE1, the clock-2 resides in the network connect to the DS-TT2/UE2, and the clock-3 resides in the network connect to the NW-TT.

Figure 6.17.3.-1: Procedure for BMCA via 5G System

The NW-TT port is Port-N.

1.	The DS-TT1/UE1 establish the PDU Session for TSC which is specified in the TS 23.502 [3]. The UPF/NW-TT allocates the Port-1 for the DS-TT1/UE1.

2.	The DS-TT2/UE2 establish the PDU Session for TSC which is specified in the TS 23.502 [3]. The UPF/NW-TT allocates the Port-2 for the DS-TT2/UE2.

3.	The (g)PTP Announce message from clock-3 arrives the Port-N (i.e. NW-TT).

4.	The BMCA function in the UPF/NW-TT determine the clock 3 as GM clock for the time domain. The BMCA function locally assigns the state of port which receives the Announce message from clock 3 as SlavePort (i.e. Port-N), and Port-1 and Port-2 as MasterPort.

5.	The BMCA function generates and forwards the Announce message(s) from GM clock (i.e. clock-3) to MasterPort(s) based on the updated PortStates and AnnouceSendTimer of port.

6.	The (g)PTP Announce message from clock-2 arrives the Port-2 (i.e. DS-TT2/UE2). The DS-TT2/UE2 forward the message to UPF/NW-TT.

7.	The BMCA function in the UPF/NW-TT determine the clock 2 as GM clock for the time domain. The BMCA function locally assigns the state of port which receives the Announce message from clock 2 as SlavePort (i.e. Port-2), and Port-1 and Port-N as MasterPort.

8.	The BMCA function generates and forwards the Announce message(s) from GM clock (i.e. clock-2) to MasterPort(s) based on the updated PortStatesand and AnnouceSendTimer of port.

9.	The (g)PTP Announce message from clock-1 arrives the Port-1 (i.e. DS-TT1/UE1).The DS-TT1/UE1 forward the message to UPF/NW-TT.

10.	The BMCA function in the UPF/NW-TT determine the GM clock and Port state are not changed. The BMCA function generates and forwards the Announce message(s) from GM clock (i.e. clock-2) to MasterPort(s) based on the existing PortStates and AnnouceSendTimer of port.

6.17.4	Impacts on services, entities and interfaces

NW-TT/UPF:

-	There is BMCA function inside the NW-TT/UPF.

-	The BMCA function run the BMCA algorithm according to 802.1AS [6] or IEEE 1588-2008 [13]. It:

-	determines the GrandMaster clock for a clock domain;

-	determines and stores the port state of DS-TT port and NW-TT port of the 5GS TSN bridge;

-	updates and sends Announce message to port with MasterPort role.

*******************************************End of CHANGE **********************************************