SA WG2 Meeting #S2-141E	S2-2007499

12 October - 23 October 2020, Electronic, Elbonia	(revision of S2-200xxxx)

Source:	ZTE

Title:	KI#2, sol#12 update to remove the ENs

Document for:	Agreement

Agenda Item:	8.5

Work Item / Release:	FS_IIoT/ Rel-17

Abstract of the contribution: This paper update the solution#12 to remove the ENs.

1. Discussion

In the last meeting, the revision to remove the ENs are not accepted because the comments said, there is following constriction in 23.501 clause 5.8.2.13.3.2.

The mechanism is supported in the following conditions:

-	When N19 is used, there is a full mesh of N19 tunnels between UPFs serving the 5G VN group;

-	There is no support of forwarding packets with destination MAC address not known by SMF/UPF (i.e. no support for new UE MAC addresses from the UE during the PDU Session lifetime)

-	There is no support for forwarding a broadcast/multicast packet with source address not known to SMF/UPF.

-	Each UPF supports one N6 interface instance towards the data network, or only supports N19-based forwarding without N6;

-	Multicast group formation of selected members of a 5G VN is not described in this release of the specification.

But this restriction is only applied for the VN group with PDR/FAR mechanism, is not for general packet handling.

2. Proposal

*****************************************START of CHANGE **********************************************

6.12	Solution #12: The bridge U-Plane model for UE-UE communication

6.12.1	Introduction

This solution is to Key Issue 2: UE-UE TSC communication. Specially, this solution proposes a unified u-plane forwarding architecture. This solution does not touch the port pair determination, 5GS bridge delay of the port pair calculation and the configuration of deterministic QoS for the UE-UE TSC. It can work with existing solution for Key issue#2, e.g. solution 2, 3 and 4.

6.12.2	Functional Description

In the figure 6.12.2-1, it introduces a FP (forwarding process) in the NW-TT. All the downlink and uplink traffic are sent to the FP first, and then the FP decides where the packet is forwarded to.

Figure 6.12.2-1: The forwarding process architecture

With this solution, the static filter entry is only sent to NW-TT in the BMIC (Bridge management information container).

6.12.3	Procedures

There are two phase for the u-plane handling.

Phase-1: receiving phase:

Figure 6.12.3-1: receiving phase

As shown in the figure 6.12.3-1, when the downlink or uplink data is received, it is forwarded to Forwarding process in the NW-TT.

-	When the UPF receives the uplink traffic from PDU session for TSC, it forward to the FP/NW-TT.

-	When the NW-TT receives the downlink packet from N6/DN, it forward to FP.

Phase-2: forwarding phase:

Figure 6.12.3-2: Forwarding phase

As shown in the figure 6.12.3-2, the FP decides how to forward the packet according to port map in the forward rule (i.e. static filter entry).

-	If the packet matches the port of NW-TT, the FP forwards the packet to the Port of NW-TT;

-	Otherwise, the FP forwards the packet to the UPF. The UPF uses the N4 rules, e.g., PDR/FAR/QER/URR to handle the packet to the related PDU session.

-	When receives the UL packets with unknown destinations address, the packet is sent to all NW-TT ports.

Editor's note:	It is FFS how packet replication is achieved (i.e., for broadcast, multicast or unknown unicast frames).

NOTE:	static filter entry is used by NW-TT to select the egress NW-TT port for packet delivery.

6.12.4	Impacts on services, entities and interfaces

NW-TT: support the Forwarding process.

*******************************************End of CHANGE **********************************************