3GPP TSG-WG SA2 Meeting #141E	S2-2007553

Elbonia, October 12 – 23, 2020	(Revision of S2-200xxxx)

Source:	NTT DOCOMO, Lenovo, Motorola Mobility

Title:	Key Issue #1 solution evaluation

Document for:	Approval

Agenda Item:	8.4

Work Item / Release:	FS_eNS_ph2 / Rel-17

Abstract of the contribution: This contribution evaluates solutions addressing the KI#1.

# Discussion

The following set of evaluation criteria are used to evaluate the proposed solutions for KI#1 to support network slice related quota on the maximum number of UEs:

1)	Comply to the working assumptions and requirements as described in clause 4 of TR23.700-40.

2)	Address all the issues identified in the KI#1 as described in clause 5.1.

3)	Impact to the UE

4)	Impact to the CN, the serving PLMN, the home PLMN, e.g. AMF, NSSF, UDM, CHF, PCF in term of signalling interfaces, control flows and internal logic etc. including changes to the semantics of existing services/parameters.

5)	Impact to (R)AN

6) 	Impact to O&M

Alternative#1: (Preferable)

From all 10 solutions proposed for KI#1 (Solution #1-#4, #8, #9, #15, #18, #19, #38), some may have a complete solution and some not. But overall, we can summarize that there are three main functionalities for supporting quota management on the maximum number of UEs as described below.

- NW Slice quota information storage function: This function is responsible for storing a NW Slice quota information, which consists one of the following information.

- An indication whether a S-NSSAI is subject to the network slice quota checking

- The maximum number of UEs for the S-NSSAI (either the global quota of the overall maximum number of UEs in a S-NSSAI or the local quota that is a subset of global quota)

- NW Slice quota monitoring function: This function is responsible for counting and collecting the number of UEs that have been registered for a S-NSSAI that is subject to the network slice quota checking. It may also have a functionality to send an indication whether the network slice quota has been reached.

- NW Slice quota enforcement function: This function is responsible for enforcing a network slice SLA, i.e., to accept or reject the Requested S-NSSAI by taking the network slice quota into account. In case of rejection, the function may provide a rejection cause and a back off time.

Table 1 below shows an overview of key impacts of all solutions and in particular where the above functions are placed in the 5G system.

Table 1: Key impacts of the solutions

|  | UE Impact | RAN Impact | 5GC CN Impact |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Existing NF |  |  | New NF |  |  |  |
|  |  |  | NW Slice Quota Storage | NW Slice Quota Monitor | NW Slice Quota Enforce | NW Slice Quota Storage | NW Slice Quota Monitor | NW Slice Quota Enforce |  |
| Sol#1 | Yes | No | UDR, PCF | PCF | PCF AMF | - | - | - | (Note 1) (Note 5) |
| Sol#2 | Yes | No | - | - | AMF | NSQ |  | - | (Note 1) (Note 5) |
| Sol#3 | Yes | No | NSSF | NSSF | NSSF, AMF | - | - | - | (Note 1) (Note 5) |
| Sol#4 | Yes | No | NWDAF |  | AMF, PCF (roaming case) | - | - | - | (Note 1) (Note 5) |
| Sol#8 | Yes | No | O&M, AMF | O&M, AMF | AMF | - | - | - | (Note 1) (Note 2) (Note 5) |
| Sol#9 | Yes | No | UDM, NWDAF | NWDAF, CHF | AMF | - | - | - | (Note 1) (Note 5) |
| Sol#15 | Yes | No | - | - | AMF | - |  | - | (Note 3) |
| Sol#18 | No | No | - | - | AMF | SQM |  | - | (Note 4) (Note 5) (Note 7) |
| Sol#19 | No | No | NWDAF |  | - | QCF |  | QEF | (Note 4) (Note 6) (Note 7) |
| Sol#38 | No | No | CHF | CHF | AMF, CHF | - | - | - | (Note 1) (Note 2) (Note 5) |
| NOTE 1: Solution requires a UE to support a (new/existing) rejection cause and a (new/existing) back-off timer due to the network slice quota has been reached. NOTE 2: Solution has an impact on O&M to support either a network slice quota monitoring and/or a network slice quota distribution (if any) NOTE 3: Solution only addresses the aspect of back-off timer to be sent to the UE for network slice quota enforcement. NOTE 4: Solution does not describe whether the UE should be aware of a rejection cause due to a network slice quota has been reached. NOTE 5: Solution requires a change in both the H-PLMN and the V-PLMN to support a network slice quota monitoring and a network slice quota enforcement. NOTE 6: No descriptions of roaming aspect NOTE 7: Although the solution proposes a new NF, it also mentions that this new NF could be deployed together with existing NF. In such case, no new NF is needed. |  |  |  |  |  |  |  |  |  |

Alternative#2:

Alternative to the Table 1 shown above, we may simplify it by only capturing the key impacts of each solution without mentioning about the three functionalities related to the network slice quota management. This simplified comparison is shown in Table 2 below.

Table 2: Key impacts of the solutions

|  | UE Impact | RAN Impact | 5GC CN Impact |  | Notes |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Existing 5GC CN NF | New 5GC CN NF |  |
| Sol#1 | Yes | No | UDR, PCF, AMF | - | (Note 1) (Note 5) |
| Sol#2 | Yes | No | AMF | NSQ | (Note 1) (Note 5) |
| Sol#3 | Yes | No | AMF, NSSF | - | (Note 1) (Note 5) |
| Sol#4 | Yes | No | AMF, NWDAF, PCF (only in roaming case) | - | (Note 1) (Note 5) |
| Sol#8 | Yes | No | AMF | - | (Note 1) (Note 2) (Note 5) |
| Sol#9 | Yes | No | UDM, AMF, NWDAF, CHF | - | (Note 1) (Note 5) |
| Sol#15 | Yes | No | AMF | - | (Note 3) |
| Sol#18 | No | No | AMF | SQM | (Note 4) (Note 5) (Note 7) |
| Sol#19 | No | No | - | QCF, QEF | (Note 4) (Note 6) (Note 7) |
| Sol#38 | No | No | AMF, CHF | - | (Note 1) (Note 2) (Note 5) |
| NOTE 1: Solution requires a UE to support a (new/existing) rejection cause and a (new/existing) back-off timer due to the network slice quota has been reached. NOTE 2: Solution has an impact on O&M to support either a network slice quota monitoring and/or a network slice quota distribution (if any) NOTE 3: Solution only addresses the aspect of back-off timer to be sent to the UE for network slice quota enforcement. NOTE 4: Solution does not describe whether the UE should be aware of a rejection cause due to a network slice quota has been reached. NOTE 5: Solution requires a change in both the H-PLMN and the V-PLMN to support a network slice quota monitoring and a network slice quota enforcement. NOTE 6: No descriptions of roaming aspect. NOTE 7: Although the solution proposes a new NF, it also mentions that this new NF could be deployed together with existing NF. In such case, no new NF is needed. |  |  |  |  |  |

From Table 1 or Table 2 above, one can derive a commonality among those solutions as following:

-	All solutions do not require changes in RAN

-	All solutions propose to store an information related to the network slice quota information in the CN and the UE is not aware of it.

- 	All solutions propose to monitor the number of UEs accessing a network slice in the CN and to enforce the quota in the CN. Difference among those solutions are a) where to store the network slice quota information, b) where to place the network slice quota monitoring and the network slice quota enforcement. There are two main approaches, one is to put them into an existing 5GC network function or introducing a new network function/new service operation to be deployed in the existing 5GC network function.

Furthermore, there are some other aspects that we could also draw some remarks, for example,

-	Rejection cause + Back-off timer: When a network slice quota in terms of number of UEs is overflown, and a CN NF sends a rejection to the UE requesting for a S-NSSAI, it is up to the Stage 3 to determine whether a new rejection cause or an existing rejection cause should be used. To suppress further signalling load for a subsequent request for the same S-NSSAI, the CN NF may provide a back-off timer to the UE. The CN NF responsible for sending a rejection for the requested S-NSSAI and optionally the back-off timer for the rejected S-NSSAI is the AMF.

# Proposal

It is proposed to capture the summary of the analysis and the table of solution comparison (either Table 1 or Table 2) in the TR23.700-40.

It is to note that our preference is to have Table 1 as it clearly shows what are the difference in terms of where to place the network slice quota related functionalities. But if SA2 would agree the simplified table of comparison, it is also fine.

********************************  START OF CHANGES **************************************

# 7	Overall Evaluation

## 7.1	Evaluation on solutions of KI#1

From all 10 solutions proposed for KI#1 (Solution #1-#4, #8, #9, #15, #18, #19, #38), some may have a complete solution and some not. But overall, we can summarize that there are three main functionalities for supporting quota management on the maximum number of UEs as described below.

- NW Slice quota information storage function: This function is responsible for storing a NW Slice quota information, which consists one of the following information.

- An indication whether a S-NSSAI is subject to the network slice quota checking

- The maximum number of UEs for the S-NSSAI (either the global quota of the overall maximum number of UEs in a S-NSSAI or the local quota that is a subset of global quota)

- NW Slice quota monitoring function: This function is responsible for counting and collecting the number of UEs that have been registered for a S-NSSAI that is subject to the network slice quota checking. It may also have a functionality to send an indication whether the network slice quota has been reached.

- NW Slice quota enforcement function: This function is responsible for enforcing a network slice SLA, i.e., to accept or reject the Requested S-NSSAI by taking the network slice quota into account. In case of rejection, the function may provide a rejection cause and a back off time.

Alternative#1: (Preferable)

Table 7.1-1 illustrates 5GS system impacts for all solutions proposed for the KI#1. Moreover, it also compares where to place the afore-mentioned network slice quota functionalities in the 5G system.

Table 7.1-1: Key impacts of the solutions

|  | UE Impact | RAN Impact | 5GC CN Impact |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Existing NF |  |  | New NF or new service operation in existing NF |  |  |  |
|  |  |  | NW Slice Quota Storage | NW Slice Quota Monitor | NW Slice Quota Enforce | NW Slice Quota Storage | NW Slice Quota Monitor | NW Slice Quota Enforce |  |
| Sol#1 | Yes | No | UDR, PCF | PCF | PCF AMF | - | - | - | (Note 1) (Note 5) |
| Sol#2 | Yes | No | - | - | AMF | NSQ |  | - | (Note 1) (Note 5) |
| Sol#3 | Yes | No | NSSF | NSSF | NSSF, AMF | - | - | - | (Note 1) (Note 5) |
| Sol#4 | Yes | No | NWDAF |  | AMF, PCF (roaming case) | - | - | - | (Note 1) (Note 5) |
| Sol#8 | Yes | No | O&M, AMF | O&M, AMF | AMF | - | - | - | (Note 1) (Note 2) (Note 5) |
| Sol#9 | Yes | No | UDM, NWDAF | NWDAF, CHF | AMF | - | - | - | (Note 1) (Note 5) |
| Sol#15 | Yes | No | - | - | AMF | - | - | - | (Note 3) |
| Sol#18 | No | No | - | - | AMF | SQM |  | - | (Note 4) (Note 5) (Note 7) |
| Sol#19 | No | No | NWDAF |  | - | QCF |  | QEF | (Note 4) (Note 6) (Note 7) |
| Sol#38 | No | No | CHF | CHF | AMF, CHF | - | - | - | (Note 1) (Note 2) (Note 5) |
| NOTE 1: Solution requires a UE to support a (new/existing) rejection cause and a (new/existing) back-off timer due to the network slice quota has been reached. NOTE 2: Solution has an impact on O&M to support either a network slice quota monitoring and/or a network slice quota distribution (if any) NOTE 3: Solution only addresses the aspect of back-off timer to be sent to the UE for network slice quota enforcement. NOTE 4: Solution does not describe whether the UE should be aware of a rejection cause due to a network slice quota has been reached. NOTE 5: Solution requires a change in both the H-PLMN and the V-PLMN to support a network slice quota monitoring and a network slice quota enforcement. NOTE 6: No descriptions of roaming aspect NOTE 7: Although the solution proposes a new NF, it also mentions that this new NF could be deployed together with existing NF. In such case, no new NF is needed. |  |  |  |  |  |  |  |  |  |

Alternative#2:

Table 7.1-1 illustrates 5GS system impacts for all solutions proposed for the KI#1.

Table 7.1-1: Key impacts of the solutions

|  | UE Impact | RAN Impact | 5GC CN Impact |  | Notes |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Existing 5GC CN NF | New 5GC CN NF or new service operation in existing NF |  |
| Sol#1 | Yes | No | UDR, PCF, AMF | - | (Note 1) (Note 5) |
| Sol#2 | Yes | No | AMF | NSQ | (Note 1) (Note 5) |
| Sol#3 | Yes | No | AMF, NSSF | - | (Note 1) (Note 5) |
| Sol#4 | Yes | No | AMF, NWDAF, PCF (only in roaming case) | - | (Note 1) (Note 5) |
| Sol#8 | Yes | No | AMF | - | (Note 1) (Note 2) (Note 5) |
| Sol#9 | Yes | No | UDM, AMF, NWDAF, CHF | - | (Note 1) (Note 5) |
| Sol#15 | Yes | No | AMF | - | (Note 3) |
| Sol#18 | No | No | AMF | SQM | (Note 4) (Note 5) (Note 7) |
| Sol#19 | No | No | - | QCF, QEF | (Note 4) (Note 6) (Note 7) |
| Sol#38 | No | No | AMF, CHF | - | (Note 1) (Note 2) (Note 5) |
| NOTE 1: Solution requires a UE to support a (new/existing) rejection cause and a (new/existing) back-off timer due to the network slice quota has been reached. NOTE 2: Solution has an impact on O&M to support either a network slice quota monitoring and/or a network slice quota distribution (if any) NOTE 3: Solution only addresses the aspect of back-off timer to be sent to the UE for network slice quota enforcement. NOTE 4: Solution does not describe whether the UE should be aware of a rejection cause due to a network slice quota has been reached. NOTE 5: Solution requires a change in both the H-PLMN and the V-PLMN to support a network slice quota monitoring and a network slice quota enforcement. NOTE 6: No descriptions of roaming aspect. NOTE 7: Although the solution proposes a new NF, it also mentions that this new NF could be deployed together with existing NF. In such case, no new NF is needed. |  |  |  |  |  |

From Table 7.1-1 above, one can derive a commonality among those solutions as following:

-	All solutions do not require changes in RAN

-	All solutions propose to store an information related to the network slice quota information in the CN and the UE is not aware of it.

- 	All solutions propose to monitor the number of UEs accessing a network slice in the CN and to enforce the quota in the CN. Difference among those solutions are a) where to store the network slice quota information, b) where to place the network slice quota monitoring and the network slice quota enforcement. There are two main approaches, one is to put them into an existing 5GC network function or introducing a new network function/new service operation to be deployed in the existing 5GC network function.

Furthermore, there are some other aspects that we could also draw some remarks, for example,

-	Rejection cause + Back-off timer: When a network slice quota in terms of number of UEs is overflown, and a CN NF sends a rejection to the UE requesting for a S-NSSAI, it is up to the Stage 3 to determine whether a new rejection cause or an existing rejection cause should be used. To suppress further signalling load for a subsequent request for the same S-NSSAI, the CN NF may provide a back-off timer to the UE. The CN NF responsible for sending a rejection for the requested S-NSSAI and optionally the back-off timer for the rejected S-NSSAI is the AMF.