SA WG2 Meeting #141E	S2-2007701

12 - 23 October 2020, Elbonia

Source:	Nokia, Nokia Shanghai Bell

Title:	KI #2, Evaluation & Interim conclusion – re-selection category

Document for:	Discussion/Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_Ph2 / Rel-17

Abstract of the contribution: Proposes interim conclusions related to NWDAF re-location.

# 1		Discussion

In Solution #26 the old NWDAF notifies the NWDAF service consumer about the relocation. In solutions #10, #57, #59 the NWDAF service consumer is informed by the target NWDAF, which is inline with AMF relocation procedures as specified in Release-16.

Solutions #26 and #57 propose a new NWDAF AnalyticsRelocation service, while solutions #10 and #59 re-use existing service operations. Thereby, in solution #10 the target NWDAF requests the analytics context from the source NWDAF ("pull"), whereas in solution #59 the source NWDAF will include the analytics context in the handover or handover preparation request to the target NWDAF ("push").

Solution #10 describes a procedure, where the re-location is triggered by AMF (due to UE mobility). Solutions #26, #57, #59 describes re-location being triggered by the NWDAF (due to UE mobility, load balancing, etc.).

Based on those observations, a set of principles is derived for the KI#2 conclusions.

# 2	Proposal

It is proposed to update TR 23.700-91 with the following conclusions.

*** Start Change (all new text) ***

## 7.2	Key Issue #2: Multiple NWDAF instances

## 7.2.x	Re-selection of NWDAF (category #2)

In Solution #26 the old NWDAF notifies the NWDAF service consumer about the relocation. In solutions #10, #57, #59 the NWDAF service consumer is informed by the target NWDAF, which is inline with AMF relocation procedures as specified in Release-16.

Solutions #26 and #57 propose a new NWDAF AnalyticsRelocation service, while solutions #10 and #59 re-use existing service operations. Thereby, in solution #10 the target NWDAF requests the analytics context from the source NWDAF ("pull"), whereas in solution #59 the source NWDAF will include the analytics context in the handover or handover preparation request to the target NWDAF ("push").

Solution #10 describes a procedure, where the re-location is triggered by AMF (due to UE mobility). Solutions #26, #57, #59 describes re-location being triggered by the NWDAF (due to UE mobility, load balancing, etc.).

*** Next Change (all new text) ***

## 8.2	Key Issue #2: Multiple NWDAF instances

### 8.2.1	Hierarchical NWDAF architecture (category #1)

Editor's Note: placeholder for conclusions/recommendations related to category "hierarchical NWDAF architecture"

### 8.2.2	Re-selection of NWDAF (category #2)

The following principles are proposed for the normative work on "Re-selection of NWDAF":

- a)	Extend or re-use existing Release-16 service operations, i.e. do not introduce new AnalyticsRelocation service.

- b)	Support different triggers conditions for the NWDAF re-selection corresponding to UE mobility, load balancing, etc.

- c)	Support the analytics handover and handover preparation, e.g., as described in clause 6.59.2.2.

- d)	Transfer the analytics context in Nnwdaf_AnalyticsSubscription_Subscribe Request from the source to the target NWDAF (in case of re-selection triggered by the NWDAF) or in Nnwdaf_AnalyticsInfo_Request response message (in case of re-selection triggered by the AMF).

- e)	The target NWDAF notifies the NWDAF service consumer about the re-location.

### 8.2.3	Specific aspects of NWDAF interactions (category #3)

Editor's Note: placeholder for conclusions/recommendations related to category "specific aspects of NWDAF interactions"

*** End Change ***