SA WG2 Meeting #141E	S2-2007752

12 October – 23 October 2020

Source:	Nokia, Nokia Shanghai Bell

Title:	KI#3A: Solution update: #5 Addendum on Configuring Hold and Forward Buffers

Document for:	Discussion/Approval

Agenda Item:	8.5

Work Item / Release:	FS_IIoT / Rel-17

Abstract of the contribution: This contribution proposes am addendum to Solution #5 to resolve discussions about Hold and Forward Buffers

# 1		Discussion

S2-2005738 was discussed at SA WG2 Meeting #140E to resolve several Editor’s Notes in Solution #5, including three related to the use of Hold and Forward Buffers.

- Editor's note:	Whether and how to apply Hold and forward buffers for IP PDU Sessions and Ethernet PDU sessions if CNC is not present  is FFS.

- Editor's note: How the parameters for H&F buffering are created and sent to the UE and UPF is FFS.

- Editor's note:	UE and UPF impacts are FFS.

At SA2 #140E there was no agreement on the proposals to resolve these three ENs.

Solution #5 addesses KI#3A, which explores how to support “Exposure of deterministic QoS”. What distinguishes Deterministic QoS from URLLC is principally the ability to support “absolute delay and jitter requirements”.  Without a mechanism to limit jitter the, 5GS can introduce jitter up to the AN-PDB (due to opportunistic scheduling in the RAN that results in early Tx/Rx), which far exceeds jitter introduced by a non-TSN wired network.

Observation #1:  It is important to find an acceptable solution to limit jitter for Deterministic QoS in Rel. 17

In Rel-16 Hold and Forward Buffers were selected as the mechanism to control jitter in the 5GS TSN Bridge, and it was decided to configure these buffers using IEEE802.1Qbv managed objects provided by the CNC and transferred to the DS-TT and NW-TT using the PMIC. This is only supported when an IEEE TSN network is deployed with the 802.1Qcc “Fully Centralized” model.

Observation #2: Using Hold and Forward Buffers to limit jitter for Deterministic QoS in Rel. 17 allows reuse of the mechanism defined in Rel. 16 for networking scenarios where IEEE TSN is not used.

Two possible options are envisioned to support Hold and Forward Buffers in Rel-17:

- H&F buffers are configured using 3GPP defined parameters for both IP and Eth (as was proposed in S2-2005738)

- H&F buffers are configured according to R16 IEEE Qbv parameters, possibly only for Ethernet PDU Sessions

# 2		Proposal

Remove the H&F related ENs in Solution #5 and capture in an addendum of Solution #5 the options for Hold and Forward Buffers to support KI#3A. It is recommended that selection of at least one of the options be made in the Conclusion phase.

The following change is proposed for TR 23.700-20.

# * * * * First change * * * *

## 6.5	Solution #5: Deterministic QoS for Native 5GS

### 6.5.1	Introduction

For Key Issue #3A: Exposure of deterministic QoS, a solution is needed that allows "any AF that has knowledge of deterministic application requirements" to "be able to request TSC services from the 5GS and as authorized, be notified of pertinent network events." This solution provides a mechanism to do that whereby the AF may learn 5GS capabilities to support TSC, request QoS with specified requirements and supply information that can be used to derive TSCAI for 5GS QoS flows. The solution is applicable for all PDU Session Types (IP, Ethernet and unstructured), and is independent of DN TSN protocols (IEEE 802.1Qbv [8], IEEE 802.1Qci, IEEE 802.1Qcc [7]).

To enable the above capabilities, this solution proposes to enhance External Exposure of Network Capability (see TS 23.501 [2], clause 5.20), specifically for Provisioning and Monitoring capabilities, to notify for the support of the capability to provide deterministic QoS. To reuse the "Setting up an AF session with required QoS" procedure in TS 23.502 [3], clause 4.15.6.6 is proposed for setting QoS for TSC.

To cover a wide range of possible QoS parameters for TSC applications, the QoS Reference currently used in AF QoS requests is supplemented with requirements for specific parameters. When a requirement for a specific parameter is provided in the AF request, the PCF may, for the referenced Qos profile, dynamically replace default values with values that meet the specific requirement provided by the AF.

### 6.5.2	Functional Description

The following capabilities are proposed using QoS request from AF:

-	The AF requests TSC QoS and provides traffic pattern as assistance parameters via procedure "Setting up an AF session with required QoS procedure", supplying the NEF. with requirements for one or more of a 5GS delay, Guaranteed Flow Bit Rate, Flow Direction, Burst Arrival Time at UE (uplink) or UPF (downlink), Burst Size, Burst Periodicity, Survival Time, and a Time Domain. The AF also provides a Traffic Description, Target UE PDU session Identification, AF Identification, and a QoS Reference. AF may also request subscription to events defined in Table 6.1.3.18-1, TS 23.503 [12]. Example of relevant events are Resource allocation outcome, QoS targets can no longer (or can again) be fulfilled, QoS Monitoring parameters. Service Data Flow deactivation (when the QoS flow is removed); and then the PCF may decide to subscribe to PCRTs in the SMF example PCRTs that may be used as defined in, Table 6.1.3.5-1, TS 23.503 [12] such as GFBR of the QoS Flow can no longer (or can again) be guaranteed. Additional events and PCRTs may be considered in the normative phase. Note GPSI may be applied to identify the individual UE in a manner similar to that used for AF influence on Traffic Routing (see TS 23.501 [2], clause 5.6.7). The NEF authorizes the AF request and forwards the request to the PCF. Then:

-	The PCF may, according to PCC rule authorization, choose a 5QI based on the QoS Reference and dynamically set the PDB and/or MDBV according to the received 5GS Delay and Burst Size requirements. As authorized, AF specified parameter values are used to over-ride default values, for the 5QI. If an AF request for a parameter value exceeds an authorization, the PCF may assign the highest authorized value. The GFBR for QoS Flow is set according to the Guaranteed Flow Bit Rate.

-	The 5GS may set the TSCAI Burst Arrival Time (BAT), Periodicity and Survival Time according to the requested Periodicity, Burst Arrival Time and Survival Time received from the AF.

-	If Time Domain information was supplied by the AF and the 5GS uses the same Time Domain, then no adjustment is needed for deriving TSCAI information. If the Time Domain provided by AF is different from 5GS Time Domain and the SMF has clock drift information for that Time Domain (i.e. clock drift between 5G timing and AF supplied Time Domain), then 5GS may adjust the TSCAI information so that it reflects the 5GS Clock. If Time Domain information is not provided or the SMF does not have synchronization information for a requested Time Domain, then the TSCAI information will be used without adjustment.

-	If the AF provides burst spread, the 5GS will provide burst spread as part of TSCAI to the NG-RAN.

-	When subscribed events are met the PCF reports them to the NEF, and NEF will notify the AF. The subscribed event may be reported once the SMF reports a PCRT to the PCF.

-	Following the procedure in clause 4.16.5.1, TS 23.502 [3] PCF reports the events to the NEF, and NEF notifies the AF.

-	The 5GS exposes the following Deterministic QoS Capability information to aid the AF in formulating a request for TSC QoS that will be acceptable to the 5GS:

-	5GS Support for Determinists QoS (yes/no).

-	Minimum and maximum 5GS Delay supported (provided if 5GS Support for Deterministic QoS set to "yes").

Editor's note:	The need for and how to enforce minimum 5GS delay is FFS.

-	The 5GS provides the response to a request from an AF for TSC QoS.

The following capabilities are proposed for TSC connectivity monitoring:

-	The AF sends request to NEF to monitor the status of TSC connectivity via a set of events for a UE or group of UEs.

-	If the request is authorized, the NEF subscribes the required monitoring events to suitable NFs (e.g. AMF, SMF).

-	When the NF detects the subscribed events occurs, the NF sends the event notification to NEF. The NEF forwards to the AF the received reporting event.

To support notification, the additional events may be added to the list of monitoring capabilities specified for the NEF in TS 23.502 [3] (see clause 4.15.3.1). Existing events in that list that are relevant for TSC and can be subscribed toare : PDU Session Status (detected by SMF), Communication failure(detected by AMF), and Loss of Connectivity (detected by AMF).

### 6.5.2.1	Hold and Forward support

Two principle options are envisioned to support Hold and Forward Buffers in Rel-17:

- H&F buffers are configured using the Traffic Pattern Parameters from the TSC Assistance Container, as was proposed in SA2 #140E S2-2005738. With this option, the NEF may include TSC Assistance Container Traffic Pattern Parameters in the PMIC. Those parameters are Burst Arrival Time (referenced to the 5GS ingress as in Rel. 16), the flow Periodicity and flow direction. These parameters are the same as those specified in the Rel-16 TSC Assistance Container. For Rel. 17 it is proposed to also include Burst Spread. The UE/DS-TT or UPF/NW-TT then determines Hold and Buffer parameters using the PMIC provided information and QoS flow parameters. It could determine H&F buffering as illustrated below:

- 	H&F_Open_Time = BAT + PDB + UE-DS-TT Residence Time + n*Periodicity + Burst_Spread

-	H&F_Close_Time =H&F_Open_Time + Window

where “Window” may be set to MDBV / R, where R is the UE or UPF egress (eg: N6) interface rate.

- H&F buffers are configured according to R16 IEEE Qbv parameters, possibly only for Ethernet PDU Sessions. Since there is no CNC, it is the responsibility of the AF/NEF to construct the 802.1Qbv managed objects which are transferred in the PMIC as in Rel. 16. The Gate Control managed objects are defined in TS23.501, Table 5.28.3.1-1,  are reproduced below.

Table 6.5.2.1-1: Gate Control Parameters in PMIC

There are two sub-options when using 802.1Qbv Managed Objects (MOs):

2a – 802.1Qbv MOs are used to configure H&F buffers only for Ethernet PDU Sessions

2b – 802.1Qbv MOs are used to configure H&F buffers for Ethernet or IP PDU Sessions. Note IP DiffServ code points used for TSC can be mapped to Ethernet PCPs so an Ethernet based hold and forward buffer may be used for IP PDU Sessions.

A comparison of these options is given in the table below:

Table 6.5.2.1-2: Comparison of Options for Configuring H&F Buffers

|  | Option 1: TSC Traffic Pattern Parameters are sent in the PMIC | Option 2a: 802.1Qbv for Ethernet PDU Sessions | Option 2b: 802.1Qbv for Ethernet or IP PDU Sessions |
| --- | --- | --- | --- |
| Impacts UE/DS-TT and UPF/NW-TT | Yes- need to use the new parameters to configure H&F buffers | No Impact | No Impact if Ethernet H&F buffers are reused and IP DSCPs are mapped to PCPs. |
| Supports IP applications (eg: VIAPA, AR/VR. Gaming) | Yes | No | Yes |
| AF/NEF needs to create Qbv parameters | No | Yes | Yes |
| 802.Qbv parameters are used in the 5GS but not in the DN (no TSN DN or CNC). | No | Yes | Yes |

### 6.5.3	Procedures

An overall procedure for Native 5GS deterministic QoS is illustrated in Figure 6.6.3-1:

Figure 6.5.3-1: Procedure for Exposure the deterministic QoS

1.	PDU Session Establishment for UE1 as specified in clause 4.3.2.2.1 of TS 23.502 [3]. If the AF is located inside the operator's domain, the PCF exposes the UE-DS-TT residence time to the AF as defined in TS 23.502 [3] and the AF is configured with the UE to UPF/NW-TT delay per Type of Service.

2.	If the AF is located outside the operator's domain, the NEF exposes Deterministic QoS Capability to AF, as shown in Figure 6.5.3-2. If the NEF learns the UE-DS-TT residence time from PCF and NEF is configured with the UE to UPF/NW-TT delay per Type of Service.

3.	Setting up an AF session with required QoS procedure specified in the TS 23.502 [3] clause 4.15.6.6. is executed with the following modifications:

-	The parameters described in clause 6.5.2 "Function Description" may be specified in the AF request to the NEF

-	The NEF may forward to the PCF the additional parameters received at the NEF, and the PCF derives the 5QI and sets the PCC Rule accordingly. The PCF includes the Time Domain to the PCC Rule.

-	The response from PCF to NEF, and NEF to AF contains additional information on the TSC resources allocated for the request.

4.	The PCF creates the PCC rule and provides to SMF. And PCF also includes the TSC traffic pattern information from NEF/AF.

5.	The SMF may calculate the TSCAI similar to TS 23.501 [2] clause 5.27.2, with the following modifications:

The SMF assigns the TSCAI with the Time Domain as received from the PCF to a QoS Flow. If the SMF receives clock drift information from UPF for that Time Domain (i.e. clock drift between 5G timing and AF supplied Time Domain), then SMF may adjust the TSCAI information of the QoS Flow so that it reflects the 5GS Clock.

6.	The SMF configures the U-plane and sends the N2 message with TSCAI to NG-RAN. The SMF distributes the parameters for Hold and Forward Buffering to the UE and UPF.

.

AF requested TSC QoS:

The following procedure is used to expose 5GS information to aid the AF in formulating a request for TSC QoS (step 3 of Figure 6.5.3-1).

Since the "Minimum 5GS Delay supported" and "Maximum 5GS Delay supported" are dependent on UE residence time, the following is supported to report this information:

1.	The AF may specify in the Nnef_TSC_Capability_Request, one or more MAC addresses of devices, GPSIs or IP addresses. Note some of these MAC addresses may belong to devices connected via a UE, and some to devices connected via the DN as the AF may be unaware of network topology.

2.	For devices where UPF MAC learning has detected the AF requested MAC address as a Source Address in an uplink frame (and hence the device is connected via the associated PDU session), or where the IP address and GPSI is associated with a UE, the 5GS provides in the Nnef_TSC_Capability_Response:

a.	The "minimum 5GS delay supported" and "maximum 5GS delay supported", taking into account UE Residence time and the delay associated with a TSC 5QI.

3.	For other devices (those whose MAC address has not been detected by the UPF as a Source Address in an uplink frame), or where the IP address or GPSI is not recognized, the 5GS indicates in the Nnef_TSC_Capability_Response that the device is not connected via a PDU Session.

The information provided in the Capability Response can aid the AF in formulating a request for TSC QoS by allowing the AF to:

1.	Requesting delay based on 5GS reported minimum and maximum delay (between two UE(s) or UE and DN) for PDU Sessions.

2.	Specify in the request for QoS, the relevant MAC address, GPSI or IP address.

NOTE:	For Ethernet PDU sessions, "UE MAC address" is a required input parameter for the Nnef_AFsessionWithQoS_Create service operation - see TS 23.502 [3], clause 5.2.6.9.2.

Editor's note:	How the NEF provides capability response based on support by other NFs is FFS or whether a more static approach that does not require the reporting described in Figure 6.5.3-2: TSC Capability exposure towards external AF is FFS.

Figure 6.5.32: TSC Capability exposure towards external AF

The NEF obtains 5GS capability to support Determinists QoS for a PDU session using the PCF provided information and the local configuration in NEF. The AF receives 5GS bridge minimum and maximum delay information.

Method 1 for 5GS Capability exposure towards AF:

1.	The AF requests the 5GS Capabilities to support Determinists QoS. The AF provides the UE MAC address or the GPSI and the Application Identifier (VIAPA).

2.	The NEF checks finds the PCF that serves the PDU session, then NEF requests PCF whether a UE-DS-TT residence time can be provided.

3.	The PCF provides the UE-DS-TT Residence time for the DS-TT port MAC address if available.

4.	If the DS-TT Residence time is provided, the NEF provides to the AF the minimum and maximum delay for the Application Identifier.

NOTE 2:	NEF aggregates UE-DS-TT residence time and per QoS configured UE to UPF/NW-TT delay to determine the delay to be provided to the AF.

Editor´s note:	Whether the NEF can provide an indication on whether the network supports Deterministic QoS and the max (and if needed min) delay based on other information, not configured in NEF, is FFS. Whether it is PCF or NEF that determines (e.g. based on configuration) support for Deterministic QoS is FFS.

TSC connectivity monitoring:

Figure 6.5.3-3: Procedure for TSC connectivity monitoring

1.	The AF sends Nnef_EventExposure_Subscribe Request to NEF to monitor the status of TSC connectivity via a set of events for a UE or group of UEs. The events are those listed in clause 4.15.3.1 of TS 23.502 [3].

Event Reporting Information defines the type of reporting requested (e.g. one-time reporting, periodic reporting or event based reporting, for Monitoring Events).

2.	If the reporting event subscription is authorized by the NEF, the NEF sends the Subscribe Requests to the related NFs that detect the events, as described in TS 23.502 [3] procedures for subscription to these events.

3.	The NEF sends Nnef_EventExposure_Subscribe Response to AF as described in TS 23.502 [3] procedures for subscription to these events.

4.	The NFs detects the monitored event occurs and sends the event report to the NEF as described in TS 23.502 [3] procedures for subscription to these events.

5.	The NEF sends to the AF the notification of TSC communication monitoring event changed as described in TS 23.502 [3] procedures for subscription to these events.

### 6.5.4	Impacts on services, entities and interfaces

Impacts on existing entities and interfaces

The solution has the following impacts:

1.	An additional Nnef service is used to expose 5GS TSC capabilities to support Deterministic QoS.

2.	Additional parameters are included in existing messages in the "AF session with required QoS procedure" specified in the TS 23.502 [3] clause 4.15.6.6.

3.	The PCF uses the additional AF supplied parameters to calculate a PCC rule.

4.	"Burst Spread" is sent from the PCF to the SMF, which uses it to determine a Burst Spread TSCAI parameter.

5.	Time Domain is provided from PCF to the SMF, which is used by the SMF to associate the QoS Flow to a Time Domain.The solution assumes that the 5GS can be made aware of the timing in the AF supplied Time Domain, this can be provided via Time Synchronization service as described in Solution #7.

NOTE: UE and UPF impacts are noted in the Addendum on Hold and Forward Buffers

AF:

-	Deliver TSC connectivity monitoring request message to NEF.

-	Deliver TSC QoS request with TSC QoS reference, TSC traffic pattern, and subscription to events to the PCF.

### 6.5.5	Addendum: Hold and Forward Buffers for Deterministic QoS

### 6.5.5.2	H&F Buffer Options

Two principle options are envisioned to support Hold and Forward Buffers in Rel-17:

- H&F buffers are configured using the Traffic Pattern Parameters from the TSC Assistance Container, as was proposed in SA2 #140E S2-2005738. IEEE 802.1Qbv managed objects need not be computed by the AF/NEF. Instead the NEF includes in the PMIC, TSC Assistance Container Traffic Pattern Parameters. Those parameters are Burst Arrival Time (referenced to the 5GS ingress as in Rel. 16), the flow Periodicity and flow direction. These parameters are the same as those specified in the Rel-16 TSC Assistaance Container. For Rel. 17 it is proposed to also include Burst Spread. The UE/DS-TT or UPF/NW-TT then determines Hold and Buffer parameters using the PMIC provided information and QoS flow parameters. Specifically:

- 	H&F_Open_Time = BAT + PDB + UE Residence Time + n*Periodicity + Burst_Spread

-	H&F_Close_Time =H&F_Open_Time + Window

where “Window” may be set to MDBV / R, where R is the UE or UPF egress (eg: N6) interface rate.

- H&F buffers are configured according to R16 IEEE Qbv parameters, possibly only for Ethernet PDU Sessions. Since there is no CNC, it is the responsibility of the AF/NEF to fabricate the 802.1Qbv managed objects which are transferred in the PMIC as in Rel. 16. The Gate Control managed objects are defined in TS23.501, Table 5.28.3.1-1, and are reproduced below.

Table 6.x.2-1: Gate Control Parameters in PMIC

There are two sub-options when using 802.1Qbv Managed Objects (MOs):

2a – 802.1Qbv MOs are used to configure H&F buffers only for Ethernet PDU Sessions

2b – 802.1Qbv MOs are used to configure H&F buffers for Ethernet or IP PDU Sessions. Note IP DiffServ code points used for TSC can be mapped to Ethernet PCPs so an Ethernet based hold and forward buffer may be used for IP PDU Sessions.

A comparison of these options is given in the table below:

|  | Option 1: TSC Traffic Pattern Parameters are sent in the PMIC | Option 2a: 802.1Qbv for Ethernet PDU Sessions | Option 2b: 802.1Qbv for Ethernet or IP PDU Sessions |
| --- | --- | --- | --- |
| Impacts UE/DS-TT and UPF/NW-TT | Yes- need to use the new parameters to configure H&F buffers | No Impact | No Impact if Ethernet H&F buffers are reused and IP DSCPs are mapped to PCPs. |
| Supports IP applications (eg: VIAPA, AR/VR. Gaming) | Yes | No | Yes |
| AF/NEF needs to backwards engineer Ingress/Egress times to create Qbv parameters | No | Yes | Yes |
| 802.Qbv parameters are used in the 5GS but not in the DN (no TSN DN or CNC). | No | Yes | Yes |

Table 6.x.2-2: Comparison of  Options for Configuring H&F Buffers

# * * * * End of Changes * * * *