3GPP TSG-SA WG2 Meeting #141e 	S2-2007861

12 -23 October 2020, Electronic meeting

Source:	Ericsson

Title:	KI#1, Additional Conclusions for Multiple Anchors

Document for:	Agreement

Agenda Item: 	8.3

Work Item / Release:	FS_enh_EC /Rel-17

Abstract of the contribution: This contribution proposes some additions to the conclusions for KI#1 for multiple anchors.

# 1         Introduction

The evaluation section for KI#1 for Multiple anchors (section 7.1.4) includes a number of scenarios in which the solutions will not work so as a proposal for one of them. The proposal was to capture them in normative phase. This pCR proposes to add them to the Conclusions Clause.

# 2	Proposal

## **************************** Start Change ***************************

### 7.1.4	Evaluation for Key Issue #1: DNS based solutions for Multiple PDU Sessions

The Multiple Session connectivity model enables that Edge Computing applications can use a specific PDU session with the PDU Session anchor in the local site whether the rest of applications use a PDU Session with a central PDU Session anchor. The mapping between applications and PDU sessions is steered by the URSP rules.

Using DNS for EAS Discovery, also the mapping between the DNS and PDU sessions needs to be carefully considered

From evaluation of solutions #1, #4, #5 (URSP part), #13 and #21 the following is concluded:

For an EC Use Case to work on multiple sessions connectivity model using URSP rules, the solution involves:

-	The UE, and how it handles the application requests and related DNS queries and the relation of that to user defined DNS settings, URSP rules and the established PDU sessions and their settings.

-	The URSPs traffic and route selection descriptors.

-	The mapping rules between AF request parameters and URSP rules parameters in UE such as relationship between DNAI and DNN.

The recommendation is to define the URSPs so that Domain Descriptors are used to steer the DNS and the Application traffic into PDU sessions.

The current specifications do not explicitly define whether the establishment of the PDU session due to execution of the Domain descriptor in URSP is done before or after sending out the DNS Query.

The following clarifications are therefore required on how the Domain descriptors in URSP are used on the UE:

-	When a UE issues a DNS request targeting the FQDN of an App, the PDU Session in which to send the DNS request is controlled by URSP rules.

-	The UE should be able to use URSP procedures to setup the PDU session prior to sending a DNS query

-	The DA of the DNS request is the DNS server address of this PDU Session configured by the 5GC (SMF) to the UE by one of available means.

In addition, the UE should be able to (re)evaluate the application association with a PDU session based on Location Criteria generated per AF requested Spatial Validity Condition.

The solution is not guaranteed to work, when the UE doesn't support URSP rule provisioned from PCF, when the DNS Server at the UE has been configured by the user, when the Application Client generates the DNS Query outside of DNS libraries in the OS with DoT, DoH or other over the top mechanisms. The study conclusion shall address guidelines to address such scenarios.

NOTE: 	This can correspond to devices doing tethering, or to devices deployed for specific corporate purposes.

In addition, as a pre-requisite:

-	The configuration in SMF for the parameters in the PDU Session Establishment Request shall assist SMF to select the right UPF/PSAs for the PDU Session.

Editor's note:	the list of the new configuration data in the SMF needs to be detailed, possibly in the conclusions clause.

Solution #1 proposing to Enhance the NEF service to allow the AF to influence PCF decisions for URSP rules is recommended for normative phase. The enhancements shall support the recommendations above.

Solution #1 fulfils the mechanisms to enable AF to influence PCF decision for URSP rules.

Solution #1 has no impact to existing UE configuration. URSP rules are not applicable to session breakout scenarios (e.g.: UL CL situation).

How AF can guide PCF to send proper URSP rules need to be defined.

## **************************** Next Change ***************************

### 9.1.1	Conclusions regarding solutions for Key Issue #1 for DNS based solutions for Multiple PDU Sessions

Solution #1 proposing to Enhance the NEF service(s) (AF Requests need to be enhanced) to allow the AF to influence PCF decisions for URSP rules, is recommended for normative phase.

The recommendation to define the URSPs so that Domain Descriptors are used to steer the DNS and the Application traffic into PDU sessions is to be captured in normative phase too, so as the clarifications that are required on how the Domain descriptors in URSP are used on the UE (see clause 7.1).

The solution is not guaranteed to work when  the UE doesn't support URSP rule provisioned from PCF, when the DNS Server at the UE has been configured by the user or when the Application Client generates the DNS Query outside of the DNS libraries in the OS with DoT, DoH or other over the top mechanisms.

NOTE: 	This can correspond to devices doing tethering, or to devices deployed for specific corporate purposes.

These limitations and informative guidelines shall be captured to cover scenarios where the OS, user or applications may override the operator-provided URSP or DNS settings. The guidelines should assume no restriction on the OS, user or application.

Editor's note:	It is FFS whether other solution like solution 13 variant on URSP is needed to define how AF can guide PCF to send proper URSP rules.

## ****************************** End Change ******************************