3GPP TSG-WG SA2 Meeting #141E e-meeting 	S2-2007878

Elbonia, October 12 – 23, 2020	(revision of S2-2007462r02)

Source:	Huawei, HiSilicon, ZTE

Title:	KI#5: Solution 50 update and evaluation/conclusion of KI#5

Document for:	Approval

Agenda Item:	8.5

Work Item / Release:	FS_enh_EC / Rel-17

Abstract: This contribution proposes the clarification regarding when to remove the I-SMF in solution 50. This contribution also proposes the evaluation and conclusion for KI#5.

- Discussion

- When to insert or remove the new SMF/I-SMF in Solution #12/#50.

The SMF get the interested DNAI from the SM-PCF. In the solution#12/#50, when the SMF detects that DNAI need be used to SMF/I-SMF selection, it notifies AMF. But it is unclear when the DNAI shall be notified to AMF? Our understanding is that this notification to AMF only happens when based on the current UE location the SMF determines a new DNAI(s) is possible but this DNAI is not supported by the SMF, i.e. the related UPF is not controlled by SMF.

So the SMF determines the target DNAI(s) by considering the UE location. When the UE move in/out the related interested area, the SMF notifies the addition/removal of DNAI to the AMF. This is unrelated to the Nnef_TraficInfluence_AppRelocationInfo, i.e. the DNAI is determined by the SMF not by the AF. The SMF determine whether the UPF can be inserted by the AF response but the DNAI information is determined by the SMF. Step 3b may cause some confusion. It is proposed to be removed.

Base on that information in session breakout case the AMF can determine to add/remove the I-SMF or not. And for the SMF selection case, the AMF can re-select the SMF associated with that DNAI or not.

- Proposal

It is proposed to make the following changes to the TR 23.748.

* * * * First change * * * *

6.50	Solution #50: Activating the traffic routing towards Local Data Network per AF request

6.50.1	Description

The following solution corresponds to the key issue #5 on activating the traffic routing towards Local Data Network per AF request as specified in clause 5.5. This solution is applicable for Session Breakout Connection Model (for SSC Mode 1 PDU sessions).

The AF uses AF influence traffic mechanism to activate the traffic routing towards the requested DNAI. The AMF performs I-SMF selection. In order to select a proper I-SMF, the AMF selects an I-SMF based on requested DNAI(s).

The solution assumes the supported DNAI list is stored in the SMF profile in NRF or locally configured in AMF. After receiving the AF influenced Traffic Steering Enforcement Control in the PCC rule, if the SMF determines it cannot serve the requested DNAI, it sends the requested DNAI to the AMF in e.g. Nsmf_PDUSession_SMContextStatusNotify. This option can be applied to existing PDU session.

Figure 6.50.1-1 Non-roaming architecture with I-SMF insertion to the PDU Session in reference point representation, with UL CL/BP

6.50.2	Procedure

6.50.2.1	I-SMF insertion for existing PDU Session

Figure 6.50.2.1-1: Solution - I-SMF insertion for existing PDU Session

1.	AF provides the target DNAI (s) (List of DNAIs, indicates to access the target Local DN, which is identified by DNAIs), N6 traffic routing info to the SM_PCF and the AF will subscribe to the SMF exposure event. This procedure is same as the steps 1-4 defined clause 4.3.6.2 in TS 23.502 [3].

2.	void

3a.	The SM_PCF updates the SMF with corresponding new PCC rule(s) including the DNAI(s) for the PDU sessions by invoking Npcf_SMPolicyControl_UpdateNotify service operation. The service procedure is described in the step 5 in clause 4.3.6.2 of TS 23.502 [3].

Based on the received DNAI(s) information, the SMF may subscribe to the UE mobility event notification from the AMF (e.g. UE moving into or out of Area of Interest).

The SMF determines the target DNAI(s) which are applicable to the current UE location.

The SMF may decide the target DNA(s) finally when the SMF receives the Nsmf_TraficInfluence_AppRelocationInfo request if the AF acknowledge is expected.

3b.	void.

3c.	The SMF invokes a Nsmf_PDUSession_SMContextStatusNotify(target DNAI(s) for existing PDU session) service operation if it (or the associated old I-SMF) can not serve the target DNAI(s), and the content of the message includes the target DNAI(s), the PDU Session ID. This message triggers the AMF to select a suitable I-SMF for the PDU Session.

If there is an I-SMF serving the PDU session, the SMF invokes Nsmf_PDUSession_StatusNotify and then the I-SMF invokes Nsmf_PDUSession_SMContextStatusNotify message to send the target DNAI(s) for existing PDU session to AMF..

4.	The AMF may select a new I-SMF which can serve the target DNAI for the existing PDU Session.

If the AMF doesn't have the knowledge which DNAIs the I-SMF/SMF can serve the target DNAI based on local configuration, it invokes the NF discovery request with NRF which provides the list of SMFs supporting the requested DNAI(s).

If the AMF receives more than one requested DNAI(s), the AMF selects a new I-SMF which can serve all the DNAI(s) associated with this PDU Session. If the AMF can’t find such I-SMF, the AMF determines to select an I-SMF which serves the target DNAI(s) per operator’s policy.

5.	The AMF sends a Nsmf_PDUSession_CreateSMContext Request to the new I-SMF, the request message includes the target DNAI(s).

6.	The new I-SMF retrieves SM Context from the old I-SMF (in the case of I-SMF change) or SMF (in the case of I-SMF insertion) by invoking Nsmf_PDUSession_Context Request.

7.	The new I-SMF selects a new I-UPF based on the received SM context, e.g. target DNAI, S-NSSAI, and UE location information.

8.	The new I-SMF initiates a N4 Session Establishment to the new I-UPF.

9.	The new I-SMF invokes Nsmf_PDUSession_Update Request (in the case of I-SMF insertion) or Nsmf_PDUSession_Create Request (in the case of I-SMF change) towards the SMF. The SMF initiates N4 Session Modification toward the PSA1. And The SMF responds Nsmf_PDUSession_Create Response or Nsmf_PDUSession_Update Response to the new I-SMF.

10.	If the AF has subscribed to user plane management event notifications, the SMF notifies the AF of the PSA change by invoking Nsmf_EventExposure_Notify service operation.

11.	The new I-SMF sends a Nsmf_PDUSession_CreateSMContext Response to the AMF.

12.	In the case of I-SMF reselection, the AMF sends Nsmf_PDUSession_ReleaseSMContext Request to old I-SMF for the release of resources in old I-SMF.

13.	Add a PDU Session Anchor and Branching Point or UL CL controlled by I-SMF. The same procedure is performed as described in clause 4.23.9.1-1 starting from step 3 in TS 23.502 [3].

Steps 6 to 13 are per R16 specifications; Other ETSUN related procedures are not modified.

The SMF removes the target DNAI(s) from the AMF when the SMF determines that there is no applicable DNAI(s) for the current UE location by invoking the Nsmf_PDUSession_StatusNotify service operation. The AMF remove the I-SMF according to the ETSUN related procedures.

6.50.3	Impacts on services, entities and interfaces

AMF:

1.	Receives the target DNAI(s) from SMF.

2.	Selects I-SMF supporting the target DNAI

SMF:

1.	Sends target DNAI(s) to AMF and triggers it to select an I-SMF can serve the target DNAI.

2.	Register the supported DNAI list in NRF.

NRF:

1.	Store the supported DNAI list in the service profile of the I-SMF/SMF.

* * * * End of Change * * * *