3GPP TSG-SA WG2 Meeting #141E e-meeting	S2-2008229

Elbonia, October 12 – 23, 2020 	(Revision of S2-2007109)

# Title:	LS Response on Bulk operation of LCS-service

# Response to:	(S2-2004803/C4-203629) LS on Bulk operation of LCS-service

# Release:	Release-16

# Work Item:	5G_eLCS

Source:	SA WG2

To:	CT WG4

Cc:	CT WG3

Contact Person:

Name:	Ake Busin

Tel. Number:

E-mail Address:	ake.busin@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	S2-2008230

- Overall Description:

This LS supersedes the response in S2-2005921.

Since response LS in S2-2005921 was sent, SA2 has agreed on solution also including (immediate) MT-LR, the approved CR is attached to this LS. Thus, response to question 2 is updated.

- When using Service Based Interface (SBI), which node is expected to be the aggregation point of Bulk Operation of LCS Service, i.e. the node who resolves the group identifier to member UEs and aggregate the location requests and responses for each UE in the group, NEF or GMLC or both?

Answer: The GMLC is the node to resolve the group identifier to member UEs and is the aggregation point of location response to NEF and LCS client.

- If the GMLC is expected to be the aggregation point of Bulk Operation of LCS Service, what should the GMLC respond for a request of bulk operation, especially for 5GC_MT_LR procedure?

NOTE:	CT4 observed that if NEF sends a 5GC-MT-LR request towards GMLC for group of UEs, there is no way for GMLC to relay the multiple responses (one per UE) to the NEF, using current service operations specified in TS 23.273 (Only one service response is allowed per service request in SBI and location for one UE is allowed in Ngmlc_Location_ProvideLocation response).

Answer:  For a MT-LR, the GMLC will confirm the request with a Ngmlc_Location_ProvideLocation Response message. Location reporting will use one or more Ngmlc_Location_EventNotify operation that may aggregate location estimate of multiple UEs.

- If used as aggregation point, when does the GMLC / NEF acknowledge the request for Bulk Operation, i.e. does it respond the LCS client / AF after it received all positioning responses from network side for all UEs in the group or respond without AMF checking?

Answer: The GMLC will acknowledge the request for Bulk Operation prior to checking of privacy profile. and retrieval of serving AMF.

- If GMLC / NEF responds the LCS client / AF immediately, based on what criterion does the GMLC / NEF decide to respond success or failure or partial success?

Answer: This is considered an implementation decision.

And How should GMLC / NEF to notify the LCS Client / AF if the requests for positioning for some UEs fail because of some different reasons (e.g. The UE is not online) after GMLC / NEF responded success to LCS client / AF?

Answer: This will be reported in a Nnef_Location_EventNotify towards NEF or LCS Service Response towards LCS Client.

2. Actions:

To CT WG4 group.

ACTION: 	SA2 kindly requests CT4 to take response into consideration.

3. Date of Next SA2 Meetings:

3GPP TSG SA2 #142e	November 16 – 20 2020	E-Meeting

3GPP TSG SA2 #143	March 01 - 05        2021	TBD