3GPP TSG-SA2 Meeting #141E	S2-2007260

# Oct 12- 23 Oct 2020	(revision of)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2423 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Transfer Procedure of PDU Session used for IMS voice from N3GPP to 3GPP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1, TEI16 |  |  |  |  |  | Date: |  |  | 2020-10-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change |  | According to RP-200795 (on Rel-16, approved in RAN#88e), when the UE requested to handover an existing PDU session from non-3GPP access to 3GPP access, the NG-RAN may reject the establishment of PDU session due to the trigger EPS fallback or RAT fallback, however, it is unclear how the UE and the SMF should behave, and there is a need to clarify. CT1 agreed C1-205300 during CT1#125e that the UE shall abort the non-3GPP to 5GS handover procedure if it does not receive PDU Session Establish accept, however the change is agreed on Rel-17. In order not to prohibit such UE implementation in Rel-16, it’s proposed to add a note in clause 4.13.6.3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new case (according to RP-200795) that NG-RAN rejects the PDU session resource setup due to the trigger of EPS fallback or RAT fallback, and clarify the SMF behavior. For the existing case that NG-RAN accepts PDU Session setup but rejects IMS voice QoS Flow setup, move up the text related to EPS fallback and inter-RAT Fallback to improve readability. Add a note that, depending on UE implementation, the UE can stop the non-3GPP to 5GS handover procedure in the case that RAN rejecting the PDU Session setup. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | System behaviour when NG-RAN rejects the PDU Session handover remain unspecified. Misalignment with RAN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.13.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

#### 4.13.6.3	Transfer of PDU session used for IMS voice from non-3GPP access to 5GS

Figure 4.13.6.3-1: Transfer of PDU session used for IMS voice from non-3GPP access to 5GS

When the UE has an ongoing IMS voice session via non-3GPP access using ePDG or N3IWF, and the session is transferred to NG-RAN, depending on the selected RAT in 5GS (NR or E-UTRA), and the support of EPS/inter-RAT fallback in NG-RAN, either the IMS voice session continues over NG-RAN (E-UTRA) or EPS/inter-RAT fallback is triggered.

Steps 1, 2 and 3 apply to either of the above two cases.

1.	UE has ongoing IMS voice session via non-3GPP access using ePDG or N3IWF. UE is triggered to move to 3GPP access and camps in NG-RAN.

2. If the UE is not registered via 3GPP access, the UE shall initiate Registration procedure as defined in clause 4.2.2.2.2.

3.	UE initiates PDU session establishment for the PDU session used for IMS voice service in order to initiate handover from EPC/ePDG to 5GS as defined in clause 4.11.4.1 step 2 or to initiate handover from N3IWF to 3GPP access in 5GC in step 2 of clauses 4.9.2.1 and 4.9.2.3. The SMF accepts the successful PDU session transfer to the UE in NAS.

NOTE 1:	If the UE is aware (e.g implementation-dependent mechanisms) that voice over NR may not be natively supported in the current Registration area, the UE can attempt to move to E-UTRA to initiate a handover of the IMS PDU Session to EPC or 5GC to continue the IMS voice session. The remaining steps are not executed.

4.	NG-RAN may decide to trigger EPS or inter-RAT fallback, taking into account UE capabilities, indication from AMF that "Redirection for EPS fallback for voice is possible" (received as part of initial context setup as defined in TS 38.413 [10]), network configuration (e.g. N26 availability configuration) and radio conditions. NG-RAN may initiate measurement report solicitation from the UE including E-UTRA as target.

If NG-RAN does not trigger EPS or inter-RAT fallback, then the procedure stops here and following steps are not executed.

5.	If NG-RAN responds indicating rejection to set up the QoS flow for IMS voice received in step 3 towards the PGW-C+SMF (or H-SMF+P-GW-C via V-SMF, in the case of roaming scenario) via AMF with an indication that mobility due to fallback for IMS voice is ongoing, steps 5-7 from clause 4.13.6.1 are executed if EPS fallback is triggered, or steps 5-6 from clause 4.13.6.2 are executed if inter-RAT Fallback for IMS voice is triggered. The PGW-C+SMF reports the EPS Fallback event to the PCF if the PCF has subscribed to this event. The PGW-C+SMF executes the release of resources in non-3GPP AN as specified in clause 4.11.4.1 and clauses 4.9.2.1 and 4.9.2.3.

NOTE 2:	The timing of executing the release of resources in non-3GPP AN will depend on whether NG-RAN decides to trigger EPS or inter-RAT fallback but will take place at least after step 5.

If NG-RAN rejects PDU Session Resource Setup due to EPS fallback, the SMF should stop the ongoing procedure and keep the PDN connection / PDU Session at non-3GPP side. The NG-RAN performs AN release with redirection.

NOTE 3:	Depending on UE implementation, the UE can re-initiate handover of an IMS voice session over non-3GPP access to 3GPP access when receiving release with redirection from NG-RAN.

* * * * End of Changes * * * *