SA WG2 Meeting #141E (e-meeting)	S2-2007091r01

Oct 12 – Oct 23, 2020, Elbonia                             (merged with S2-2006952, S2-2006953)

Source:	NEC, Nokia, China Telecom

Title:	KI#15 – Evaluation and Conclusion for KI#15 User consent for UE data collection/analysis

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_ph2 / Rel-17

Abstract of the contribution: This contribution proposes evaluation and conclusion for KI#15 User consent for UE data collection/analysis.

# 1	Background

This contribution proposes the following evaluation and conclusion for KI#15 User consent for UE data collection/analysis in TR 23.700-91 1.0.0.

* * * * Start of 1st change * * * *

## 7.15	Key Issue #15: User consent for UE data collection/analysis

Key Issue 15 has two complementary solutions:

Solution 3 (User consent for UE data collection) introduces User consent for the UE data collection. The User consent for UE data collection may be provided by the service provider via the NEF or directly by the network operator and stored in the UDM as a subscription information which can be retrieved by the NFs. The User consent for UE data collection may define the user consent for UE level data collection from UE or application or NFs. Before initiating UE data collection and reporting by the UE, data consumers (e.g. NWDAF(s)) request for user consent from UDM. Solution 3 addresses all the architectural requirements of Key Issue 15.

Solution 76 (Revocation of user consent) introduces a functionality (User Consent Tracking Functionality, URDCF) tracking the distribution and usage of any user related information subject to user consent. In case of a revocation of the user consent, past data consumers affected by this revocation are identified and informed about the revocation requesting them to delete any user related data subject to the revoked user consent. The proposed functionality may be provided by the NWDAF or a Data Collection Function (e.g. DCCF, DCNF).

Solution #3 and Solution #76 are complementing each other. While Solution #3 defines the user consent provision in the UDM and its retrieval by the NWDAF and the AMF, Solution #76 further defines the UE related data management within the 3GPP system that is subject to user consent. Both solutions have no UE impact.

* * * * End of 1st change * * * *

* * * * Start of 2nd change * * * *

# 8	Conclusions

8.15	Key Issue 15: User consent for UE data collection/analysis

For Key Issue #15 (User consent for UE data collection/analysis) Solution #3 (User consent for UE data collection) and Solution #76 (Revocation of user consent) are the only solutions and they complement each other.

It is recommended for Key Issue #15 to use the concepts and procedures described in Solution #3 and Solution #76 as the bases for the normative work with potential further alignments based on the SA3 feedback.

* * * * End of 2nd change * * * *