SA WG2 Meeting #140-e 	S2-2006851r06

October 12 – October 23, 2020, Online

(Merging S2-2006851, S2-2007116 and S2-2007390)

Source:	China Telecom

Title:		KI#12_Solution Evaluation and Interim Conclusion for NWDAF-assisted RFSP policy

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_Ph2 / Rel-17

Abstract of the contribution: Interim conclusion for 23.700-91-050 of eNA Key Issue #12

1 Proposal

The aim of key issue 12 is to address the use case #1 NWDAF-assisted RAT/frequency selection. It is to study:

-	The kind(s) of information is made available to the NWDAF for the data analytics, e.g. the information collected for assisting RFSP index) and from which provided NFs.

-	The kind(s) of information should the NWDAF output to the NWDAF consumer as the data analytics.

-	The method(s) for NWDAF consumer of analytics, such as PCF, uses the NWDAF analytics to enhance the RFSP index value assigned to a UE.

It proposes to document the following texts into TR 23.700-91.

* * * * Start of Changes * * * *

8.  Conclusions

8.X 	Conclusion on solutions for Key Issue #12: NWDAF-assisted RFSP policy

Interim conclusion:

It is proposed to select  Solution #4,  for normative work for KI#12.The input data for service experience analytics is to reuse the ones defined in clause 6.4, TS 23.288 [5] with exception that Table 6.4.2-3: UE level Network Data from OAM is extended to further retrieve RAT/Frequency information related to the service experience observation period from OAM in order to allow NWDAF to be able to provide Service Experience per RAT type and Frequency.

And Solution #40 can be a complementary solution.

NOTE: NWDAF does not collect subscriber category as input for Observed Service Experience Analytics. Subscriber Category is used as extended analytics filter information to indicate specific UE group for Observed Service Experience in addition to Table 6.4.1-1 in TS 23.288.

* * * * End of Changes * * * *