SA WG2 Meeting #141E (e-meeting)	S2-2006832

Oct 12 – Oct 23, 2020, Elbonia

Source:	CATT

Title:	KI #1 evaluation

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	FS_5MBS / Rel-17

Abstract of the contribution: This contribution proposes the evaluation for Key Issue #1.

- Discussion

For Key Issue #1 "MBS session management", there are following solutions in the TR:

-	Solution #2-#6, #8-#16, #32 and #33.

These solutions can be categorized and evaluation are made for solutions of different categories as follows:

-	General MBS Session Management (MBS Session configuration, join/leave, release, etc.): Sol#2, #3, #4, #6, #8, #10, #14 and #16.

-	MBS Session activation and deactivation: Sol#13 and #32.

-	Interactions between MBSF and MBSU: Sol#33.

-	Broadcast specific MBS Session Management: Sol#5 and #9.

-	UE mobility / Inter-RAN HO related MBS Session Management: Sol#11, #12 and #15. Besides, solutions for UE mobility/Inter-RAN HO related delivery method switching, i.e. solutions #26, #27, #29, #39 and #40, are also listed here for a common evaluation.

# 2. Proposal

It is proposed to include the following changes in TR 23.757.

***** Start of 1st Change (all new text) *****

7.x	Key Issue #1: MBS session management

For Key Issue #1 "MBS session management", there are following solutions in the present document:

- Solution #2-#6, #8-#16, #32 and #33.

These solutions can be categorized and evaluation are made for solutions of different categories as follows:

- General MBS Session Management (MBS Session configuration, join/leave, release, etc.)

A comparison of solutions covering general MBS Session Management procedures is shown in Table 7.X-1.

Table 7.x-1: Comparison of solutions for KI#1 MBS Session Management (Part 1)

|  | Procedures |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sol | MBS Session (/Service) configuration | MBS Session Join | MBS Session Start | MBS Session Modification/Update | MBS Session Leave | MBS Session Stop | MBS Session Delete (/Release) |
| 2 |  | Y. Dedicated MBS session signalling | Y. Dedicated MBS session signalling |  | Y. Dedicated MBS session signalling | Y. Dedicated MBS session signalling. | Y. Dedicated MBS session signalling. |
| 3 | Y. Dedicated MBS session procedures. | Y. Enhanced PDU session mod./est. procedure | Y. See MBS Session Join. |  | Y. Enhanced PDU session modification procedure |  |  |
| 4 | Y. Dedicated MBS session procedures. | Y. (MB/PDU) session procedure | Y. Dedicated MBS session signalling. | Y. (MB/PDU) session procedure | Y. (MB/PDU) session procedure | Y. Dedicated MBS session signalling. | Only release per RAN node |
| 6 | Y. Dedicated MBS session procedures. | Y. Enhanced PDU session est./mod. procedure | Y. See MBS Session Join. |  |  |  |  |
| 8 |  |  | Y. Triggered by MBS Service specific parameter provisioning. | Y. Triggered by MBS Service specific parameter provisioning. |  |  |  |
| 10 |  | Y. Enhanced PDU session mod./est. procedure |  |  | Y. Enhanced PDU session mod. procedure |  | Y. Enhanced PDU session mod. procedure |
| 14 |  | Y. Dedicated MBS session signalling | Y. Dedicated MBS session signalling |  | Y. Dedicated MBS session signalling | Y. Dedicated MBS session signalling |  |
| 16 |  | Y. Enhanced PDU session est./mod. procedure |  |  |  |  |  |

From Table 7.X-1, it can be seen that most solutions use enhanced PDU Session Enhancement / Modification procedure for MBS Session Join, to achieve an integrated MBS and PDU session management as well as an easier upgrade from unicast support to multicast support.

- MBS Session activation and deactivation

Sol#13 and #32 propose procedures for MBS Session activation and deactivation, which are both based on Sol#3 and share much similarity, with the difference that Sol#32 adds control plane (i.e. multicast group configuration update) triggered MBS Session activation and deactivation.

- Interactions between MBSF and MBSU

Sol#33 proposes enhancements on N4/PFCP procedures to support interactions between MBSF(MSF-C) and MBSU (MSF-U). This may need feedback from other WGs (e.g. SA WG4).

- Broadcast specific MBS Session Management

Sol#5 and #9 propose Broadcast Session start/activation procedures, which are similar though MSF-C and MBSF are used respectively based on different baseline architectures.

- UE mobility / Inter-RAN Handover related MBS Session Management

A comparison of solutions for UE mobility/Inter-RAN HO related MBS Session Management procedures is shown in Table 7.X-2.

Most of these solutions have aspects dependent on the feedback from RAN WGs and also the conclusion of solution(s) selected for general MBS Session Management.

Table 7.X-2: Comparison of solutions for UE mobility/Inter-RAN handover related MBS Session Management

|  | Sol#11/#12 | Sol#15/#29 | Sol#26 | Sol#27 | Sol#39 | Sol#40 (dual RANs case) |
| --- | --- | --- | --- | --- | --- | --- |
| Procedures for delivery method switching | Enhanced Xn HO: MB Session resource setup during Xn Handover preparation or execution phase Enhanced N2 HO: MB Session resource setup during N2 Handover preparation phase | Enhanced Xn/N2 HO with UE initiated NAS multicast session join. | Enhanced Xn/N2 HO: Forwarding multicast data via mapped QoS flow over PDU session from source to target RAN | Enhanced Xn/N2 HO: - Multicast or unicast transport established in target RAN, depending on whether target RAN supports MBS. - Forwarding MBS data to Target RAN supporting MBS. | Multicast->Uni-cast:UE initiated PDU Session establishment or activation; Unicast->Multi-cast: UE initiated PDU session release or deactivation. | Enhanced Xn/N2 HO: - Multicast or unicast transport established in target RAN, based on UE request of unicast or multicast mode in application layer. - Forwarding multicast data over PDU session |
| Based on which solution | 2 | 4 | - | 3 |  |  |

NOTE:	Solutions for UE mobility/Inter-RAN HO related delivery method switching, i.e. solutions #26, #27, #29, #39 and #40, are also listed here for a common evaluation.

***** End of Changes *****