3GPP TSG-WG SA2 Meeting #141E e-meeting 	S2- 2006882

Elbonia, October 12 – 23, 2020	(revision of S2-200xxxx)

Source:	Huawei, HiSilicon

Title:	V2X layer assisted DRX over PC5

Document for:	Approval

Agenda Item:	8.12

Work Item / Release:	FS_eV2XARC_Ph2 / R17

Abstract: This P-CR updates solution 4 and removes the FFS in this solution.

1. Introduction/Discussion

This P-CR updates solution 4 and removes the FFS in this solution

The changes inherit from the discussion last meeting are listed as follow:

-	modify "upper layer" to "application layer".

-	remove the remaining texts related to the Wake Up Notification.

-	remove the texts and the figure about the use case in clause 6.4.1.

-	add the texts to clause 6.4.3.

Some changes proposed in this meeting are listed as follows:

-	Replace the phase “reception window information” to a terminology “Incomplete Reception Information”

-	add procedure to 6.4.2.2

-	add the clarification on the two proposed(but not captured) ENs.

-	Wording enhancement.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.776.

# * * * * First change * * * *

6.4	Solution #4: V2X Layer assisted DRX over PC5

6.4.1	Functional Description

This solution is for Key Issue #1 "Support of QoS aware NR PC5 power efficiency for pedestrian UEs".

This solution specifies how the V2X layer activates or deactivates the discontinuous reception over PC5 for pedestrian UE. Power saving reception can be effective to achieve power efficiency for a pedestrian UE, and two reception modes are introduced as below:

1)	Complete Reception Mode: the UE in this mode receives V2X service data over PC5 continuously, e.g. the radio interface of PC5 is on all the time for a pedestrian UE, which is extremely power consuming; and

2)	Incomplete Reception Mode: the UE in this mode receives V2X service data over PC5 periodically, by setting a reception window length and an incomplete reception cycle, to receive V2X service data during the reception window per incomplete reception cycle.

Application layerof a pedestrian UE determine whether the Incomplete Reception Mode can be applied, based on collected information (e.g. location, speed, predicted path, curvature etc.) from the received V2X service data. The upper layer indicates to the V2X layer of the pedestrian UE to change its reception mode when possible. One example is that, if the pedestrian is walking in a low safety risk area, e.g. in a park near the road, or along the sidewalk with road side fence. Another example is that  the pedestrian UE is in a suburb area or a rural area, where pedestrian occasionally encounters a vehicle passing by.

In order to achieve the mechanism introduced above, following PC5 signalling message is introduced:

-	Incomplete Reception Announcement: a PC5 signalling message to indicate the pedestrian UE is in Incomplete Reception Mode. The pedestrian UE in Incomplete Reception Mode includes its Incomplete Reception Information and user info in the Incomplete Reception Announcement message. The Incomplete Reception Information includes the starting point time value(s) , the reception window length and incomplete reception cycle.

The Incomplete Reception Announcement message is broadcasted periodically, e.g., once every second.  The vehicle UE's V2X layer may store the information included in the message when receiving such messages.

NOTE:	It is up to the UE implementation in the V-UE to maintain the information of the P-UEs.

6.4.2	Procedures

6.4.2.1	Activation of Incomplete Reception Mode

Figure 6.4.2.1-1: Incomplete Reception Mode activation procedure

0.	The UE(s) (i.e. pedestrian UE and vehicle UE) determines the destination Layer-2 ID for broadcast as specified in TS 23.287 [5] clause 5.6.1.2, the UE(s) performs broadcast mode V2X communication (transmit only, receive only or both) as required by application layer. If required by  application layer to receive V2X service data, the pedestrian UE starts to receive data in Complete Reception Mode.

1.	If the application layer of pedestrian UE determine the Incomplete Reception Mode can be activated, the application layer indicates to the V2X layer to change into Incomplete Reception Mode,  by providing the user info (i.e. pedestrian UE's application layer ID), the reception window length and incomplete reception cycle (e.g. 200ms for every 1s or 200ms for every 500ms).

2.	Based on the input from the application layer, the V2X layer generates the Incomplete Reception Information,  and indicates the AS layer to change into Incomplete Reception Mode. The V2X layer provides the Incomplete Reception Information to the AS layer. The AS layer monitors and receives V2X service data periodically as indicated by the Incomplete Reception Information.

NOTE:	The V2X layer of the P-UE randomly select a value within the incomplete reception cycle as the starting point time value in the Incomplete Reception Information.

3.	After the Incomplete Reception Mode is activated, the pedestrian UE sends the Incomplete Reception Announcement message periodically to broadcast the Incomplete Reception Information (as specified in clause 6.4.1), using the self-assigned source Layer-2 ID and the default destination Layer-2 ID (this default destination Layer-2 ID will only be used for this purpose).

6.4.2.2	Deactivation of Incomplete Reception Mode

Figure 6.4.2.2-1: Incomplete reception mode deactivation procedure

1.	Same to clause 6.2.2.1 step 3.

2.	The Vehicle UE and Pedestrian broadcast V2X service data. The pedestrian UE in Incomplete Reception Mode receives the V2X service data periodically (i.e. following the Incomplete Reception Information).

3. 	The application of Pedestrian UE determines indicates to the V2X layer of the Complete Reception Mode. This is triggered either by Vehicle UE (e.g., receive a dedicated warning message) or Pedestrian UE (e.g., monitoring the periodical broadcast messages).

3a.	(Pedestrian UE triggered) If the pedestrian UE’s application layer detects safety risk based on the received V2X service data (e.g., monitoring the periodical broadcast messages), the application layer indicates to the V2X layer to change into Complete Reception Mode.

3b.	(Vehicle UE triggered) If the application layer of Vehicle UE decides to send a V2X service data (e.g., warning for potential collision) to a specific pedestrian UE, the application layer provides the user info of the specific pedestrian UE to the V2X layer along with the V2X service data. The user info is included as part of the V2X application Requirements.

NOTE: 	This procedure doesn’t address the case when the Vehicle UE intends to send V2X service data(e.g., the warning message) to multiple Pedestrian UEs. It is expected Vehicle UE broadcasts the V2X service data with a well-known destination Layer 2 ID and adequate frequency. Consequently, Pedestrian UEs in proximity can receive the broadcast V2X service data in time.

If the user info is provided by the application layer and the corresponding Incomplete Reception Information exists in the V2X layer, the V2X layer passes the V2X service data along with the Incomplete Reception Information and the Layer-2 ID of the P-UE to the AS layer.

The AS layer of vehicle UE sends the V2X service data using the Layer-2 ID of the pedestrian UE as the destination layer-2 ID.

4.	The V2X layer of pedestrian UE indicates the AS layer to change into Complete Reception Mode. The UE returns to Complete Reception Mode.

6.4.3	Impacts on services, entities and interfaces

The solution has the following impact to the existing entities:

-	Pedestrian UE:

-	Application layer provides reception mode requirements or parameters;

-	V2X layer broadcasts an Incomplete Reception Announcement message periodically when the P-UE is in Incomplete Reception Mode;

-	supports V2X service data reception in Incomplete Reception Mode; and

-	switches between Complete Reception Mode and Incomplete Reception Mode.

-	Vehicle UE:

-  receives the Incomplete Reception Announcement message from P-UE(s) and

-	the vehicle UE may store the reception window information of surrounding pedestrian UEs; and

-	Application layer provides the user info along with the V2X message to the V2X layer to send a V2X message;

-	the V2X layer passes the V2X service data along with the Incomplete Reception Information to the AS layer.

# * * * * End of changes * * * *