3GPP TSG-WG SA2 Meeting #141E	S2-2007104

12 - 23 October, 2020, Electronic Meeting

Source:	Ericsson

Title:	KI #5: Conclusion Update

Document for:	Discussion/Approval

Agenda Item:	8.5

Work Item / Release:	FS_IIoT / Rel-17

Abstract of the contribution: This contribution provides an update to the conclusion of KI #5 to clarify the definition and implications of Survival Time.

# 1		Proposal

Based on discussion paper S2-2007103, we propose the following changes for TR 23.700-20.

* * * * Start changes * * * *

## 8.4	Key Issue #5: Use of Survival Time for Deterministic Applications in 5GS

Principles for the way forward:

-	Survival Time is transferred as part of the TSCAI parameter but the TSCAI may not always comprise of Survival time.

-	Survival Time is specified by the AF in units of "time" with respect to burst periodicity or as the maximum number of consecutive message transmission failures (i.e. whose loss can be tolerated). Note that there is a single message per burst periodicity and the burst contains the application message. It is conveyed together with TSCAI Periodicity parameter (the time between periodic TSC bursts) and burst size (e.g. MDBV).

If the Survival Time is specified by the AF in units of "time" with respect to TSN working domain burst periodicity, the the Survival Time needs to be mapped to the 5GS time domain by the SMF based on latest cumulative rateRatio between the TSN time and 5G time.

When survival time information is provided for a TSN stream, then it should not be aggregated with other TSN streams into a single QoS flow, or if they are aggregated, then the survival time parameter shall not be provided as part of TSCAI.

-	Survival Time is included in the TSC Assistance Container and delivered to PCF in an AF request by NEF or TSN AF.

-	The PCF provides the Survival Time to SMF in the TSC Assistance container.

-	The SMF determines Survival Time and sends it to the NG-RAN as part of TSCAI without requiring AN or N1 specific signalling exchange with the UE.

-	It is assumed that only one format will be supported over NGAP.

Editor's note:	Further work is needed to determine how Survival time is communicated towards RAN, i.e., which form is preferred over NGAP. Preferred format over NGAP depends on the feedback from RAN WG2.

* * * * End changes * * * *