3GPP TSG-SA WG2 Meeting #141E 	S2-2007188

12 – 23 October, 2020, Elbonia	 (revision of S2-200xxxx)

Source:	CATT

Title:	KI # 1&4, Sol#9: Update the solution to clarify unknown UAV tracking

Document for:	Approval

Agenda Item:	8.7

Work Item / Release:	FS_ID_UAS-SA2 / Rel-17

Abstract of the contribution: this paper proposes to update solution #9 to clarify the unknown UAV tracking procedure to add the description of step 9.

Discussion

Based on the unknown UAV tracking procedure in clause 6.9.3.2, step 9 is not described. This pCR proposes to add the description of step 9.

Proposal

It is proposed to update solution #9.

First change

6.9.3.2	Remote identification and tracking for Unknown UAV (Unknown to TPAE or UTM)

Figure 6.9.3.2-1: UAV Identity registration and allocation based on UAV bootstrapping

1.	UAV conducts registration and data connectivity procedure with 3GPP network. In this phase the 3GPP system and UFES will have this new UAV identifier information (e.g. create an entry with mapping between the 3GPP UAV ID and the CAA-Level UAV ID).

2.	UAV updates UFES with its remote identification information (e.g. if the controller may be changed during the flight) as well as the operation status information.

3.	UFES updates the record of UAV remote Identification information which is stored in the UDM.

4.	TPAE queries (the query may reach UFES or USS depending on how TPAE is setup, depending on specific deployments; if sent to USS, the USS forwards to the UFESs of the MNOs serving UAVs registered with the USS) regarding the remote identification information of an un-identified UAV in certain location.

5.	UFES sends Nnef_EventExposure_Subscribe (Location) to NEF.

6.	NEF sends Namf_EventExposure_Subscribe (Area of interest) to AMF. The message includes a request for the identifier(s) of UAV(s) in the area of interest.

7.	AMF sends Namf_EventExposure_Notify (3GPP UAV ID(s)) to NEF.

8.	NEF sends Nnef_EventExposure_Notify (3GPP UAV ID(s)) to UFES.

9.	For each 3GPP UAV ID received in step 8, the UFES queries UE location via LCS procedure. Based on the UE location, the UFES determines the UE near the location provided by TPAE as the UAV requested by TPAE.

10.	UFES queries the latest stored UAV remote identification and tracking information from UDM using the UE ID.

11.	UFES (directly or via USS) sends to the TPAE the UAV remote identification, UAV location and tracking information including the CAA-level UAV ID of the UAV in the queried location.

End of changes