# 3GPP TSG SA2 Meeting #141E	S2-2007226

# October 12 – 23, 2020, Electronic meeting 	(revision of S2-200xxxx)

Source:	NTT DOCOMO

Title:	KI#1: Update the evaluation and conclusion

Document for:	Discussion/Approval

Agenda Item:	8.5

Work Item / Release:	FS_IIoT / Rel-17

Abstract of the contribution: Complete the evaluation of the BMCA methods for Key Issue#1. Update the conclusions of the Key Issue #1 for IP type PDU Sessions.

# 1		Proposal

The following change is proposed for TR 23.700-20.

*** Start Change ***

## 7.1	Key Issue #1: Uplink Time Synchronization

Solution #1 is the only solution addressing KI#1 Uplink Time Synchronization. The solution describes four alternative methods for BMCA procedure for gPTP. In method 1 most of the BMCA logic is centralized to the TSN AF (or NEF for non-TSN sessions), but also DS-TT and NW-TT participate to BMCA operation, e.g. the DS-TT must be able to determine whether the received Announce is worse or better than the current Announce.

Method 2 and Method 4 are very similar, both run the BMCA function in NW-TT. The main difference is whether the Announce messages are generated on port basis from each DS-TT (Method 2) or from NW-TT on behalf of the DS-TT (Method 4). In both methods, the BMCA result may need to be reported from NW-TT to the NEF (or TSN AF) if the port states in the DS-TT(s) need to be changed (e.g. GM needs to be activated or deactivated in DS-TT).

*** Next Change ***

## 8.1	Key Issue #1: Uplink Time Synchronization

To enable support for Uplink Time Synchronization, following are some interim principles to be considered for the way forward:

-	The following interim agreement are applied for transmission of the gPTP event messages (carried within Sync message for one-step operation or Follow_up message for two-step operation):

-	DS-TT which is attached by one or more TSN GMs will perform exactly the same operations for UL gPTP messages as what the NW-TT performs for the DL gPTP messages as specified in clause 5.27.1.2.2 of TS 23.501 [2].

-	The PDU session of DS-TT port receiving the source GM's gPTP messages is used to forward the gPTP messages to the UPF/NW-TT:

-	the UPF/NW-TT forwards the gPTP messages to the NW-TT and DS-TT ports in Master state except for the PDU session of the source ("avoids play back to the source DS-TT port").

- 	If the egress port is in NW-TT, the NW-TT port performs exactly the same operations for UL gPTP messages as what the DS-TT performs for the DL gPTP messages as specified in clause 5.27.1.2.2 of TS 23.501 [2].

-

-	If the egress port is in other DS-TT, the other DS-TT(s) perform the operation as specified in clause 5.27.1.2.2 of TS 23.501 [2].

-	As specified in TS 23.501 [2] already for Rel-16 Time Synchronization, 5GS is required to guarantee delivery of (g)PtP message in less than 10ms as recommended by IEEE 802.1AS[6] clause B2.2, if the operator decides to follow such recommendation.

-	The following interim agreement are applied for IP type PDU Sessions for the transmission of the GM time in PTP over UDP/IP messages (carried within Sync message for one-step operation or Follow_up message for two-step operation):

-	DS-TT which is attached by one or more GMs will perform exactly the same operations for UL PTP messages as what the NW-TT performs for the DL gPTP messages as specified in clause 5.27.1.2.2 of TS 23.501 [2].

-	The PDU session of DS-TT receiving the source GM's PTP messages is used to forward the PTP messages to the UPF/NW-TT:

-	the UPF/NW-TT forwards the PTP messages to DS-TTs and the N6 interface if it knows the DS-TT or N6 interface has active listeners for PTP multicast messages.

-	If the PTP message is sent to N6 interface, the NW-TT performs exactly the same operations for UL PTP messages as what the DS-TT performs for the DL gPTP messages as specified in clause 5.27.1.2.2 of TS 23.501 [2],

-	If the PTP message is sent to DS-TT/UE via a PDU Session,  the DS-TT(s) perform the operation as specified in clause 5.27.1.2.2 of TS 23.501 [2].

-	The following agreement are applied for BMCA procedure:

-	Solution #1 Method 2 in clause 6.1.3.3 is recommended as the basis for normative work.

*** End of Changes ***