SA WG2 Meeting #S2-141E	S2-200xxxx

12 October - 23 October 2020, Electronic, Elbonia	(revision of S2-200xxxx)

Source:	ZTE

Title:	KI#1, proposal on evaluation for KI#1.

Document for:	Agreement

Agenda Item:	8.9

Work Item / Release:	FS_5MBS / Rel-17

Abstract of the contribution: This paper gives the proposal on how to make the evaluation for the key issue#1.

1. Discussion

There are many aspects for all solutions on the key issue#1. E.g.

- How the UE join/leave the MBS session (CP vs UP)

- How to identify the MBS session

- How the 5GC know the MBS capability of NG-RAN and UE

- If the CP join is applied, whether the UE join MBS session in PDU session signalling or via dedicated signalling.

- Whether the MBS session is controlled by dedicated MBS session signalling or not.

- Whether MBSF/MBSU is required or not

- Whether support the unicast PDU session associated with MBS session

- MBS session activation and deactivation;

- ………

So using a unified table to cover all the aspects of MBS session management is infeasible. So it proposes to compare the every aspect for the KI#1. And try to get the detailed conclusion.

2. Proposal

*****************************************START of CHANGE **********************************************

7.x	Key Issue #1: MBS session management

For Key Issue #1 "MBS session management", there are 16 solutions in the TR:

- Solution #2,#3,#4,#5,#6,

- #8, #9, #10, #11, #12, #13, #14, #15, #16,

- #32, #33.

These solutions can be categorized and evaluated as:

- How to identify a MBS session. i.e. What is the format of the MBS session ID ?

| Category | 1 | 2 | 3 |
| --- | --- | --- | --- |
| MBS session ID | TMGI | IP multicast address | unknown |
| solution | #2, #4,#10, #11, #12, #14, #15, #16 | #3, #13, #32 | #5, #6, #8, #9, |

- How the UE join/leave the MBS session

| Category | 1 | 2 |
| --- | --- | --- |
| How the UE join/leave the MBS session. | Both CP and UP | CP only |
| solution | #2, #3, #4, #10, | #6, #16 |

- How the 5GC know the UE MBS capability

| Category | 1 | 2 |
| --- | --- | --- |
| How the 5GC know the MBS capability of UE | NSA indication to SMF | NAS indication to AMF |
| solution | #3, | #2, |

- If the CP join is applied, whether the UE join MBS session in PDU session signalling or via dedicated signalling

| Category | 1 | 2 |
| --- | --- | --- |
| UE join MBS session in PDU session signalling or via dedicated signalling | PDU session signalling | Dedicated signaling |
| solution |  | #2, |

- Which NF needs to store the MBS context.

| Category | 1 | 2 |
| --- | --- | --- |
| NF store the MBS context | AMF | SMF |
| solution | #2 | #3, # |

Editor's note:	The further work on the table is TBD.

*******************************************End of CHANGE **********************************************