SA WG2 Meeting #S2-141E	S2-2007593

12 October - 23 October 2020, Electronic, Elbonia	(revision of S2-200xxxx)

Source:	ZTE

Title:	KI#7 proposal on the conclusion

Document for:	Agreement

Agenda Item:	8.9

Work Item / Release:	FS_5MBS / Rel-17

Abstract of the contribution: This paper give the interim conclusion for the key issue#7.

1. Proposal

*****************************************START of CHANGE **********************************************

8.X	Conclusions for Key Issue #7: Reliable delivery method switching between unicast and multicast

For the switching between PTP and PTM delivery, Solution #24 and #25 are selected as baseline solutions for normative work. The NG-RAN decides the switching between PTP and PTM delivery methods.

For the switching between Individual MBS traffic delivery (i.e. unicast PDU session) and shared N3 delivery, the solution#25 is selected as baseline solutions for normative work.

For the switch caused by intra-RAN HO:

- HO from NG-RAN not supporting MBS to NG-RAN supporting MBS, the PDU session associated with MBS session is handed over to target RAN as normal intra-Handover. After HO, the 5GC triggers the conversion from the unicast PDU Session to the MBS Session.

- HO from NG-RAN supporting MBS to NG-RAN not supporting MBS, The PDU session associated with MBS Session is handed over to the target RAN.

Editor's note:	The detail of HO will depend on the conclusion for Key issue#1.

*******************************************End of CHANGE **********************************************