3GPP TSG-SA2 Meeting #142E	S2-2007741

# Online,  12th - 23rd October 2020									   (revision of S2-200xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2511 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to N4 for NW-TT port number reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2020-09-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | During SA2#141E, it was agreed in S2-2005900 to move NW-TT port numbers to BMIC. However, the N4 impacts were left out as it was too late to make it accurate and aligned with CT4 specs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | N4 impact due to S2-2005900 is as follows: - Remove NW-TT ports from bridge info. - Keep only one NW-TT port # associated to PMIC as part of TSC Management Info IE. TS 29.244 defines TSC Management IE which is not present in Stage 2: |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | N4 parameters not consistent with the parameters sent within BMIC and it leads to incorrect implementation. Also, the TSC management information (BMIC, PMIC along with NW-TT port number) is missing. Inconsistent stage 2 can also lead to incorrent interpretations for stage 3 and implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.2.11.1, 5.8.2.11.9, 5.8.2.11.13 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### ***FIRST CHANGE***

#### 5.8.2.11	Parameters for N4 session management

##### 5.8.2.11.1	General

These parameters are used by SMF to control the functionality of the UPF as well as to inform SMF about events occurring at the UPF.

The N4 session management procedures defined in clause 4.4.1 of TS 23.502 [3] will use the relevant parameters in the same way for all N4 reference points: the N4 Session Establishment procedure as well as the N4 Session Modification procedure provide the control parameters to the UPF, the N4 Session Release procedure removes all control parameters related to an N4 session, and the N4 Session Level Reporting procedure informs the SMF about events related to the PDU Session that are detected by the UPF.

The parameters over N4 reference point provided from SMF to UPF comprises an N4 Session ID and may also contain:

-	Packet Detection Rules (PDR) that contain information to classify traffic (PDU(s)) arriving at the UPF;

-	Forwarding Action Rules (FAR) that contain information on whether forwarding, dropping or buffering is to be applied to a traffic identified by PDR(s);

-	Multi-Access Rules (MAR) that contain information on how to handle traffic steering, switching and splitting for a MA PDU Session;

-	Usage Reporting Rules (URR) contains information that defines how traffic identified by PDR(s) shall be accounted as well as how a certain measurement shall be reported;

-	QoS Enforcement Rules (QER), that contain information related to QoS enforcement of traffic identified by PDR(s);

-	Session Reporting Rules (SRR) that contain information to request the UP function to detect and report events for a PDU session that are not related to specific PDRs of the PDU session or that are not related to traffic usage measurement.

-	Trace Requirements;

-	Port Management Information Container in 5GS;

-	Bridge Information.

The N4 Session ID is assigned by the SMF and uniquely identifies an N4 session.

If the UPF indicated support of Trace, the SMF may activate a trace session during a N4 Session Establishment or a N4 Session Modification procedure. In that case it provides Trace Requirements to the UPF. The SMF may deactivate an on-going trace session using a N4 Session Modification procedure. There shall be at most one trace session activated per N4 Session at a time.

For the MA PDU Session, the SMF may add an additional access tunnel information during an N4 Session Modification procedure by updating MAR with addition of an FAR ID which refers to an FAR containing the additional access tunnel information for the MA PDU session for traffic steering in the UPF. For the MA PDU Session, the SMF may request Access Availability report per N4 Session, during N4 Session Establishment procedure or N4 Session Modification procedure.

A N4 Session may be used to control both UPF and NW-TT behaviour in the UPF. A N4 session support and enable exchange of TSN bridge configuration between the SMF and the UPF:

-	Information that the SMF needs for bridge management (clause 5.8.2.11.9);

-	Information that 5GS transparently relays between the AF the NW-TT: transparent Port Management Information Container along with the associated NW-TT port number.

-	Information that 5GS transparently relays between the AF the NW-TT: transparent Bridge Management Information Container (clause 5.8.2.11.13).

When a N4 Session related with bridge management is established, the UPF allocates a dedicated port number for the DS-TT side of the PDU Session. The UPF then provides to the SMF following configuration parameters for the N4 Session:

-	DS-TT port number.

After the N4 session has been established, the SMF and UPF may at any time exchange transparent bridge Port Management Information Container over a N4 session.

#### ***NEXT CHANGE***

##### 5.8.2.11.9	Bridge Management Information

The following table describes the Bridge Management Information (BMI) that includes the information required to configure a 5GS logical bridge for TSC PDU Sessions.

Table 5.8.2.11.9-1: Bridge Management Information

| Attribute | Description | Comment |
| --- | --- | --- |
| DS-TT Port Number | Port Number allocated by the NW-TT for the DS-TT for a given TSC PDU Session |  |
| Bridge ID | Bridge identifier of the 5GS TSN bridge |  |

#### ***NEXT CHANGE***

##### 5.8.2.11.10	Port Management Information Container

The following table describes the Port Management Information Container (PMIC) that includes information exchanged transparently via 5GS between TSN AF and NW-TT for TSC PDU Sessions.

Table 5.8.2.11.10-1: Port Management Information Container

| Attribute | Description | Comment |
| --- | --- | --- |
| Port Management Information Container as in Table 5.28.3.1-1 | Information exchanged transparently between NW-TT and TSN AF via 5GS |  |

#### ***NEXT CHANGE***

##### 5.8.2.11.13	TSC Management Information

The following table describes the TSC Management Information Container (TSC MIC) that includes BMIC, PMIC and the associated NW-TT port number.

Table 5.8.2.11.13-1: TSC Management Information Container

| Attribute | Description | Comment |
| --- | --- | --- |
| Bridge Management Information Container | 5GS TSN Bridge information exchanged transparently between NW-TT and TSN AF via 5GS (clause 5.8.2.11.13). |  |
| Port Management Information Container | Information exchanged transparently between NW-TT and TSN AF via 5GS (clause 5.8.2.11.10) |  |
| NW-TT Port Number | When PMIC IE is present, this field contain the related NW-TT Port Number. |  |