3GPP TSG-WG SA2 Meeting #141E e-meeting 	S2-2007768

Elbonia, October 12 – 23, 2020	(revision of S2-xxxxx)

Source:	Nokia, Nokia Shanghai Bell

Title:	KI #5: Conclusion

Document for:	Discussion/Approval

Agenda Item:	8.3

Work Item / Release:	FS_enh_EC / Rel-17

Abstract: This paper proposes conclusion to KI#5

# Proposal

It is proposed to agree and include the following changes to the TR 23.748.

* * * * Start of change * * * *

9. 	Conclusions

9.x	Conclusion for Key Issue #5

There are two solutions in TR 23.748 v1.0.0 corresponding to KI#5. These are solution#12 and solution #50. Solution#12 is for SSC mode 2 and SSC mode 3, while solution#50 is specific to SSC mode 1. In both these solutions SMF having  received a DNAI of interest from SM-PCF, may provide the requested DNAI to the AMF using Nsmf_PDUSession_SMContextStatusNotify i.e. in case this SMF (and the current I-SMF) do(es) not support this DNAI and decides that a new SMF (SSC mode 2 / 3, solution 12)  / I-SMF(SSC mode 1, solution #50) needs to be selected. Accordingly, the AMF selects an I-SMF for existing (SSC mode 1, solution #50) or subsequent PDU session (SSC mode 2 / 3, solution 12).

Therefore, both solution #50 and the subset of solution 12 described in § 6.12.2.2 are complementary and are recommended for normative phase related with KI#5, with below considerations:

- Normative work for KI5 shall only impact N11 (AMF-SMF interface) and N16a (to relay information). No PCF impact and no N4 impact are to be considered

- There is no I-SMF chaining induced by these solutions

- SMF decides for a PDU Session whether it is required to insert an I-SMF or not in order to support a requested DNAI. AMF considers the last DNAI received from the SMF and the AMF does not need to have policies related to potential conflicts in case a UE would need to be simultaneously served by different DNAI(s)

- Nsmf_PDUSession_SMContextStatusNotify operation is used by normative extensions due to these solutions. They will use 2 different notifications: DNAI for the current PDU Session / DNAI for the subsequent PDU Session for the AMF to be able to distinguish between I-SMF selection for the same PDU Session (situation of sol 50) and SMF selection for SSC mode 2/3 (situation of of solution 12 ).

- Based on Nsmf_PDUSession_SMContextStatusNotify with an indication of a DNAI for the current PDU Session the AMF may have to insert an I-SMF or to replace existing I-SMF.

* * * * End of Change * * * *