3GPP TSG-WG SA2 Meeting #141E e-meeting 	S2-2007871

Elbonia, October 12 – 23, 2020	(revision of S2-2007012r01)

Source:	Huawei, HiSilicon

Title:	KI#2: Conclusion for C5 solutions for KI#2 on other sub-issues

Document for:	Discussion

Agenda Item:	8.3

Work Item / Release:	FS_enh_EC / Rel-17

Abstract: This contribution proposes evaluation and conclusion on C5 solutions that addressing different sub-issues listed in the paper.

# 1. Discussion

Following the categories of solutions described in the way forward paper S2-2007008, this paper addresses the evaluation and conclusion of the following C5 solutions, especially on the AF based re-discovery aspects.

The following solutions mapping to KI#2 address different sub-issues for edge relocation. It is proposed to evaluate these solutions individually.

Table 1: Evaluations of other solutions for edge relocation

| # | Sub-issue to be addressed | Candidate Solutions | High-level descriptions |
| --- | --- | --- | --- |
| 1 | Applications do not support the change of client address when PSA changes | Sol #26 | SMF allocates a persistent address for a UE for MEC access. N6 routing is based on host routes. |
| 4 | Application status information transferring between AFs/EASs | Sol #40 | NEF transfers the application status information from the old AF/EAS to the new AF/EAS. |

In Solution #26, the UE is allocated with a persistent address by the SMF during PDU session establishment, and N6 routing is based on host routes in the DN. This solution relies on the DN has an appropriate routing mechanism so that the UE IP can be re-anchored on the new PSA. The solution will cause the fragmentation of the routing table then a huge routing table in routers in DN, considering the routing table will become per IP address not per IP prefix after UE moves. Furthermore, with UL CL, the UE can already be allocated with a IP address anchored on remote PSA, which can keep unchanged during local PSA relocation. Hence Solution#26 is not recommended in normative work.

In Solution #40, the application layer context transferring is go via NEF. Considering the context transferring is under discussion in SA6. It is recommended to decide whether this is need during normative phase after SA6 solution is stable.

# 2. Proposals

It is proposed to capture the following changes in the TR 23.748.

# * * * * First change * * * *

# 7	Overall Evaluation

## 7.x  Evaluation of Solutions for Key Issue #2

### 7.x.y Evaluation for Key Issue #2: other sub-issues for edge relocation

In Solution #26, the UE is allocated with a persistent address by the SMF during PDU session establishment, and N6 routing is based on host routes in the DN. This solution relies on the DN has an appropriate routing mechanism so that the UE IP can be re-anchored on the new PSA. The solution will cause the fragmentation of the routing table then a huge routing table in routers in DN, considering the routing table will become per IP address not per IP prefix after UE moves. Furthermore, with UL CL, the UE can already be allocated with a IP address anchored on remote PSA, which can keep unchanged during local PSA relocation. Hence Solution#26 is not recommended in normative work.

In Solution #40, the application layer context transferring is go via NEF. Considering the context transferring is under discussion in SA6. It is recommended to decide whether this is need during normative phase after SA6 solution is stable.

# * * * * Second change * * * *

## 9.x  Conclusions for Key Issue #2

### 9.x.y Conclusions regarding solutions for Key Issue #2 for other sub-issues for edge relocation

-	Solution #26 are not recommended in normative work.

-	Whether AF context transferring can be done via NEF is to be decided during normative phase after SA6 solution is stable.

# * * * * End of changes * * * *