SA WG2 Meeting #142E e-meeting	S2-2008454

Elbonia, November 16 – 30, 2020

Source:	Spirent, Huawei

Title:	KI #9, Sol #30: Update to remove Editorial note addressing Input Data split and Data Correlation

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_Ph2 / Rel-17

Abstract of the contribution: This contribution provides modifications to Solution #30 clause 6.30.1.3 (input data) to address an Editor Note.

# 1. Discussion:

An editor's note in clause 6.30.1.3 as follows is left during the SA2#141E meeting:

Editor´s note:	For the tables below should be split per source and how NWDAF correlates the input from different sources should be explained.

This paper firstly splits the Table for Input Data per data resource.

Furthermore, the Correlation information in each NF input data which helps NWDAF correlate data from different NFs has been defined in Table 6.2.4-1 of TS 23.288, so this paper also clarifies that the Table could be re-used to correlate the input data for the Dispersion Analytics.

## 2. Text Proposal

It is proposed the following changes to TR 23.700-91 [2].

# * * * * First change * * * *

#### 6.30.1.3	Input Data

The NWDAF supporting dispersion analytics shall be able to collect UE dispersion information from NF(s) and AFs. The detailed information collected by the NWDAF is network data from 5GC including NFs and AFs:

-	Network data related to UE dispersion and bound by location collected from 5GC are defined in the Tables 6.30.1.3-1, 6.30.1.3-2, 6.30.1.3-3 and 6.30.1.3-4;

Table 6.30.1.3-1: Location based UE dispersion collected from serving AMF

| Information | Source | Description |
| --- | --- | --- |
| UE ID | AMF | SUPI |
| Type Allocation Code | AMF | Terminal model and vendor information of the UE. |
| UE locations (1..max) | AMF | UE positions |
| >UE location |  | Area of Interest (TA or cells that the UE enters) |
| >Timestamp |  | A time stamp when the AMF detects the UE entering this location |
| >Transactions | AMF | Either per UE transaction or optionally if subscription with periodic notification is requested an amount of UE transactions exchanged at the location. |

Table 6.30.1.3-2: Location based UE dispersion collected from serving SMF

| Information | Source | Description |
| --- | --- | --- |
| UE IP | SMF | UE IP address |
| UE ID | SMF | SUPI |
| >Timestamp |  | A time stamp associated with the collected information |
| >Transactions | SMF | Either per UE transaction or optionally if subscription with periodic notification is requested an amount of UE transactions exchanged. |

Table 6.30.1.3-3: Location based UE dispersion collected from the AF

| Information | Source | Description |
| --- | --- | --- |
| IP 5-tuple | AF | IP address 5-tuple |
| Application ID (1..max) | AF | To identify the application for the UE |
| Locations of Application | AF/NEF | Location of application represented by a list of DNAI(s). The NEF may map the AF-Service identifier information to a list of DNAI(s) when the DNAI(s) being used by the application are statically defined. |
| Timestamp |  | A time stamp of the collected information |
| >Data Volume | AF | Amount of UE data volume exchanged per application during the period of observation, if application ID is provided in the Analytics request. This parameter is only available when subscriptions with periodic notification is requested |
| >Application duration | AF | (Duration for the application (e.g. Voice talk time) |

Table 6.30.1.3-4: Location based UE dispersion collected from serving UPF

| Information | Source | Description |
| --- | --- | --- |
| UE IP | UPF | UE IP address |
| IP 5-tuple | UPF | IP address 5-tuple |
| Application ID (1..max) | UPF | To identify the application for the UE |
| Location of Application | UPF | List of Internet applications represented by DNAI(s). |
| Timestamp |  | A time stamp of the collected information |
| >Data Volume | UPF | amount of UE data volume exchanged per application during the period, if application ID is provided in the Analytics request. This parameter is only available when subscriptions with periodic notification is requested |
| >Application duration | UPF | (Duration for the application (e.g. Voice talk time) |

The focus of the NWDAF is to calculate the volume of data and amount of transactions dispersed by the UE at each visited location during a period of interest. The entry time and exit time to a location is provided by the AMF.

If the optional application ID is requested, the volume per application can be provided by the UPF or AF.

The Correlation Information as defined in Table 6.2.4-1 of TS 23.288 [5] could be re-used to correlate the input data from AMF and SMF, the input data from SMF and UPF, the input data from UPF and AF.

-	Slice data related to UE dispersion from NF(s)  are defined in the Tables 6.30.1.3-5, 6.30.1.3-6, 6.30.1.3-7 and 6.30.1.3-8;

Table 6.30.1.3-5: Slice based UE dispersion collected from serving AMF

| Information | Source | Description |
| --- | --- | --- |
| UE ID | AMF | SUPI |
| Type Allocation Code | AMF | Terminal model and vendor information of the UE. |
| Slice (1..max) | AMF | UE assigned slice |
| >S-NSSAI |  | Slice (up to 8 per UE at any given instance) |
| >Timestamp |  | A time stamp when the slice was assigned |
| >Transactions | AMF | Either per UE transaction or optionally if subscription with periodic notification is requested an amount of UE transactions exchanged at the slice |

Table 6.30.1.3-6: Slice based UE dispersion collected from serving SMF

| Information | Source | Description |
| --- | --- | --- |
| UE IP | SMF | UE IP address |
| UE ID | SMF | SUPI |
| Slice (1..max) |  | UE assigned slice |
| >S-NSSAI |  | Slice (up to 8 per UE at any given instance) |
| >Timestamp |  | A time stamp associated with the collected information |
| >Application ID | SMF | Identify the application for the UE at the slice |
| >Transactions | SMF | Either per UE transaction or optionally if subscription with periodic notification is requested an amount of UE transactions exchanged at the slice |

Table 6.30.1.3-7: Slice based UE dispersion collected from the AF

| Information | Source | Description |
| --- | --- | --- |
| IP 5-tuple | AF | IP address 5-tuple |
| Application ID (1..max) | AF | To identify the application for the UE |
| Locations of Application | AF/NEF | Location of application represented by a list of DNAI(s). The NEF may map the AF-Service identifier information to a list of DNAI(s) when the DNAI(s) being used by the application are statically defined. |
| Timestamp |  | A time stamp of collected information |
| >Data Volume | AF | Amount of UE data volume exchanged per UE application during the period of observation, if application ID is provided in the Analytics request. This parameter is only available when subscriptions with periodic notification is requested |
| >Application duration | AF | (Duration for the application (e.g. Voice talk time) |

Table 6.30.1.3-8: Slice based UE dispersion collected from the serving UPF

| Information | Source | Description |
| --- | --- | --- |
| UE IP | UPF | UE IP address |
| IP 5-tuple | UPF | IP address 5-tuple |
| Application ID (1..max) | UPF | To identify the application for the UE |
| Location of Application | UPF | List of Internet applications represented by DNAI(s). |
| Timestamp |  | A time stamp of collected information |
| >Data Volume | UPF | amount of UE data volume exchanged per application during the period, if application ID is provided in the Analytics request. This parameter is only available when subscriptions with periodic notification is requested |
| >Application duration | UPF | (Duration for the application (e.g. Voice talk time) |

The focus of the NWDAF is to calculate the volume of data and amount of transactions dispersed by the UE at each slice during a period of interest. A time stamp for the slice assignment is provided by the AMF.

If the optional application ID is requested, the volume per application can be provided by the UPF or AF.

The Correlation Information as defined in Table 6.2.4-1 of TS 23.288 [5] could be re-used to correlate the input data from AMF and SMF, the input data from SMF and UPF, the input data from UPF and AF.

Data collection may be provided on samples (e.g. spatial subsets of UEs or UE group, temporal subsets of UE location information).

NOTE:	How the data from UPF is retrieved is not specified in this release.

The Namf_EventExposure and Nsmf_EventExposure services are enhanced to provide transaction count in Tables 6.30.1.3-1 through 6.30.1.3-6.

# * * * * End of change * * * *