SA WG2 Meeting #142E (e-meeting)	S2-2008565

November 16 – November 20, 2020, Elbonia	(revision of S2-200xxxx)

Source:	LG Electronics

Title:	KI #3, conclusion update

Document for:	Approval

Agenda Item:	8.10 Study on system enablers for multi-USIM devices

Work Item / Release:	FS_MUSIM / Rel-17

Abstract of the contribution: This paper proposes conclusion

Background

Proposal

It is proposed to capture the following proposal to TR 23.761.

* * * * Start of 1st Change * * * *

8.3	Conclusions for Key Issue #3: Coordinated leaving for Multi-USIM device

Based on the evaluation in clause 7.3 the following conclusions are agreed for the baseline functionality:

-	For leaving in E-UTRA/EPS access, the NAS-triggered leaving procedure is recommended to be supported.

-	For leaving in NR/5GS access, the NAS-triggered leaving procedure is recommended to be supported.

-	For leaving in E-UTRA/5GS access, the NAS-triggered leaving procedure is recommended to be supported.

NOTE:	A UE triggers leaving procedure only when the UE is in RRC_Connected state.

Editor's note:	It depends on RAN feedback on if changes to 5GS/E-UTRA (Option 5) are in scope of the TSG RAN work item for this KI.

Editor's note:	RAN WG's feedback is expected as decision input info for the leave procedure (NAS level leaving, and/or AS level leaving). Once the leave procedure is decided, the details about the resume procedure will be concluded.

-	For NAS level leaving procedure (EPS and 5GS case), the UE provides leaving indication to the CN and the UE may provide Paging Filtering information to the network in the leaving procedure regarding MT data/signalling handling in the CN.

-	The Paging Filtering information include information to temporarily restrict/filter MT data/signalling in this network while the UE has left. The Paging Filtering information is represented by:

-	An indication that the UE wants to be paged for All service,

-	PDN connection / PDU Session ID

-	Paging Cause

-	An indication that the UE wants be paged at all.

-	The AMF and MME provide response message to the UE to confirm the leaving procedure. The AMF and MME trigger the AN release procedure.

* * * * End of Change * * * *