3GPP TSG-WG SA2 Meeting #142E	S2-2008647

Nov 16 – Nov 20, 2020, Elbonia	(revision of S2-200xxxx)

Source:	Huawei, HiSilicon

Title:	KI #1, Conclusion: Update to clarify the ML model discovery scenarios

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_ph2 / Rel-17

Abstract: This contribution proposes to update the conclusions on Key Issue #1.

1. Introduction

Based on the discussion in S2-2008645, this contribution proposes to update the conclusions on KI #1.

2. Text Proposal

It is proposed to capture the following new text in TR 23.700-91.

* * * * Start of Changes * * * *

8	Conclusions

Editor's note:	This clause is intended to list conclusions that have been agreed during the course of the study item activities. This should also capture the guiding principles and documentation approach for creating CRs to normative specifications within the responsibility of SA WG2.

8.1	Key Issue #1: Logical decomposition of NWDAF and possible interactions between logical functions

An NWDAF can be decomposed into Model Training and Analytics logical functions.

-	A Rel-17 NWDAF containing the Model Training logical function trains ML models and exposes training service (e.g. providing trained model to the NWDAF);

-	A Rel-16 and onward NWDAF containing the Analytics logical function performs inference by deriving analytics and exposes analytics service (i.e. the Nnwdaf_AnalyticsSubscription or Nnwdaf_AnalyticsInfo as defined in clause 7, TS 23.288 [5]).

NOTE 1: Model Training Functionality and Analytics Functionality can be deployed as standalone NWDAFs.

NOTE 2:	It can also be possible to identify other standalone NF(s) based on other KI results, e.g. Data Dollection function. These conclusions do not address possible NWDAF decomposition for data collection as this is part of KI#11.

NOTE 3: Messaging framework can be used between the decomposed functions from KI#11

NOTE 4: The definition of services and their associated parameters provided by the Training functionality needs to be aligned with the conclusion of Key Issue #19: Trained data model sharing between multiple NWDAF instances.

Pre-trained ML model storage and provisioning to NWDAF is out of the scope of 3GPP.

Model Training logical function may register the supported ML model information to NRF to help another Model Training logical function in Federated Learning or an Analytics logical function discovery the ML model.

NOTE 5:	The details of ML model  discovery via NRF may also impact how the sharing of trained data model between multiple NWDAF instances is performed so it needs to be aligned with the conclusion of Key Issue #19.

* * * * End of Changes * * * *