SA WG2 Meeting #142E	S2-2008673

Nov 16 – 20, 2020, Electronic

Source:	vivo

Title:	Update of solution 22 for option 2b

Document for:	Agreement

Agenda Item: 	8.3

Work Item / Release:	FS_enh_EC /Rel-17

Abstract of the contribution: This contribution proposes an update of solution 22 for removing the ENs of option 2b.

1	Introduction

This contribution proposes an updated of solution 22 for removing the ENs of option 2b.

For Option 2b, LDNSR has two options to deal with. When LDNSR doesn’t have direct connectivity to L-DNS, one option is that LDNSR will trigger SMF to configure user plane path to L-DNS. After the configuration of user plane, it is the same as Option 2a, that LDNSR will send the DNS query to L-DNS resolve. If it doesn’t support LDNSR triggering SMF to configure the user plane to L-DNS, another function is that LDNSR reuses the Option 1 to add ECS option for DNS query.

2	Proposal

************* Start Changes *************

6.22.1	Description

6.22.1.4	DNS resolution before and after Dynamic UL CL/BP/L-PSA insertion

The LDNSR is configured as DNS server to the UE during PDU session establishment by SMF via PCO.

LDNSR have visibility of the DNS parameters. In case DoT [10], DoH [11] or DNS over DTLS[x1] is used, LDNSR terminates the DNS security. The LDNSR is operated by the serving PLMN. LDNSR does not apply in case the security (e.g. for DoT) of the interaction between the UE and its DNS server shall be terminated in a 3rd party (corporate) domain.

NOTE: UL CL/BP/L-PSA insertion in HR case is not supported in Release 17.

The EAS discovery can happen in two phases:

-	Phase 1: Before Dynamic UL CL/BP/L-PSA insertion

Option 1: LDNSR receives UL DNS query, inserts an ECS option, which includes an IP address/prefix obtained from SMF, into the DNS query and sends to C-DNS. The ECS option is related with the user location and possibly the requested FQDN.

Option 2a: LDNSR receives UL DNS query, and it forwards the UL DNS Query to a L-DNS. The L-DNS address is related with the user location and possibly requested FQDN.

Option 2b: LDNSR receives UL DNS query determines the L-DNS address but cannot send traffic to the L-DNS directly. This is further described in clause 6.22.2.

For the above options, SMF maps user location into DNAI/Local PSA for the Application Traffic. That may reuse existing information defined in R16:

- Information of the topology of UPFs and N6 accesses to the DNs.

- Application Layer information received via AF influence on routing API.

- PDU Session information, e.g. any PDU Session Local PSA.

And then, depending on the option, SMF provides to LDNSR either a L-DNS (options 2a&b) or ECS (option 1) related to the selected DNAI/Local PSA.

For above options, the SMF is informed by the LDNSR when conditions on the DNS response are met. These conditions are set by the SMF. Upon such information the SMF triggers UL CL/L-PSA insertion. In order to support different scenarios, LDNSR may inform SMF based on, EAS IP address in DNS response and/or FQDN.

As an optimization and if DNS request is sent in clear, SMF may optionally configure UL CL to local route some subsequent DNS queries based on FQDNs supported by the Local DN/DNAI. This corresponds to option 3 described as part of phase 2 below.

-	Phase 2: After Dynamic UL CL/BP/L-PSA is inserted

Unless configured to be locally routed (see option 3), LDNSR receives subsequent DNS Queries.

For queries related to that same FQDN from same UE, LDNSR can follow the procedure as described in Options 1&2 above (e.g. when previous decisions should be reconsidered).

For requests related to new FQDNs, the process is as described in Phase 1. The information of the already inserted UL CL/BP/ Local PSA can also be considered in Option 1 & 2 above.

When LDNSR is configured to apply UE redirect for an FQDN, ULCL has also been configured by SMF to locally divert to Local PSA the DNS Queries to L-DNS (e.g. on Destination IP address).

Option 3:

Option 3a: the SMF notifies the UE with the address of local DNS resolver or L-DNS server through PCO in PDU session modification command. And SMF is responsible for the DNS address selection based on location, subscription and/or local policy. The ULCL forwards the DNS query to the local PSA based solely on inspection of L3/L4 information on packet header. UPF(PSA) routes the DNS query to the local DNS resolver or L-DNS server. The DNS query can be processed in local DNS resolver or L-DNS server based on current DNS mechanism. The local DNS resolver or L-DNS server may do recursive DNS resolution to other DNS servers.

Option 3b: If in Phase 1, the SMF has configured that a FQDN query can be locally routed on the UL CL, then the subsequent DNS queries for the FQDN will be locally routed to the Local DNS Resolver. The local DNS resolver receives and handles the DNS Query that is addressing LDNSR: modifies the packet's destination IP address (corresponding to LDNSR) to that of the L-DNS and stores the original IP address (LDNSR IP) and the packet's source IP address (corresponding to UE's IP address) to its own (i.e. the local DNS resolver's) IP address and stores the original source IP address (UE IP) for later processing. The local DNS resolver then forwards the modified DNS request to the L-DNS, if the L-DNS can resolve the IP address, it responds to the local DNS resolver with the IP address of EAS; otherwise, it does recursive DNS resolution.

NOTE:	Option 3b assumes that either ULCL steering is based on L4 information (i.e. DNS port number) or ULCL has visibility of the DNS traffic (i.e. FQDN in the DNS Query message) The UPF may be instructed by the SMF to apply different forwarding of non ciphered UL DNS traffic based on the target domain of the DNS request. To support this the UPF would only need to support FQDN domain based PDR..

NOTE: 	The Local DNS Resolver can be deployed in an operator-controlled domain.

The above options for the EAS discovery using LDNSR can be described in Figure 6.22.1.4-1.

Figure 6.22.1.4-1: Options for the EAS discovery using LDNSR for PDU session breakout

6.22.2	Procedures

L-DNS server/resolver

Figure 6.22.2-1: DNS resolving in session breakout scenario

The AF may provide EAS deployment information to UDR via Nnef_TrafficInfluence, this includes FQDNs, IP addresses/prefixes, Local DNS resolver/server IP address per DNAI. Multiple AF(s) may provide the information.

The SMF gets the information from PCF (PCC rule) during PDU session establishment. During PDU Session Establishment procedure, the address of C-DNS server or LDNSR is provided by SMF via PCO to UE (LDNSR is provided if Dynamic UL CL/L-PSA insertion applies). Steps 1 - 6 are processed for each DNS request if UL CL/BP is not inserted or if UL CL/BP is not configured to locally route DNS query to local PSA after insertion.

For the "pre-established" ULCL/BP/L-PSA scenario, the SMF performs UL CL/BP/L-PSA selection and insertion based on UE location and sets up uplink filter rule on UL CL/BP. Steps 1 and 8 are processed for each DNS request if UL CL/BP is configured to route DNS query to Local PSA. Also Steps 1 and 8 are processed for each DNS request in the dynamic case for option 3. Step7 m ay happen any time, and among other, it may update the Local DNS Revolver/L-DNS in the UE. The procedures in 6.22.1.3 may be used to configure L-DNS address in UE using PCO during PDU Session Establishment.

At PDU session establishment, the SMF decides the LDNSR and informs the LDNSR for the PDU Session and provides it to the UE as the PDU Session DNS Server. The LDNSR needs to maintain a PDU session context during the lifetime of the PDU session that may also be created at this stage.

The SMF may provide to the LDNSR

Option 1: The ECS option (IP address corresponding to DNAI available to UE's location).

Option 2a: The address of L-DNS server serving the DNAI available to UE's location.

Option 2b: An indication to forward DNS query with FQDN(s) corresponding to DNAI which has no direct connectivity to LDNSR.

Based on the UE location, at PDU session establishment and during UE mobility the SMF may provide the LDNSR with ECS option and L-DNS server address for the relevant FQDNs.

The SMF may also provide the IP range(s) that shall trigger the LDNSR to notify SMF when the IP address (of EAS) in DNS response is within one of these the IP range(s). LDNSR could also be instructed to notify SMF when certain FQDN in the DNS response is matched. At PDU session establishment and during UE mobility, the SMF may provide the LDNSR with the FQDNs that, when received in the DNS query, will trigger the LDNSR to fetch the forwarding parameters from the SMF (option 1 and 2a) or forward the DNS query to the SMF (option 2b) in step 3 of the following procedure.

1.	The UE sends a DNS query including the requested FQDN.

2.	If an ULCL/BP exists and the DNS query matches the uplink filter rule on UL CL/BP to route the DNS query to Local PSA, then step 3 to 6 are skipped. Otherwise the PSA1 UPF forwards the received DNS query to the LDNSR.

3.	Based on the configuration received from the SMF for the FQDN in the DNS query, the LDNSR determines the forwarding parameters:

Option 1: The IP address to add as ECS DNS option. This IP address may correspond to the DNAI/Local PSA selected by the SMF for the UE location and a target domain.

Option 2a (with direct connectivity between LDNSR and L-DNS): The address of L-DNS server towards which to propagate a DNS request. This L-DNS address may correspond to the DNAI associated by the SMF with the UE location and a target domain..

Option 2b (without direct connectivity between LDNSR and L-DNS): The LDNSR requests the SMF to forward a DNS query (in step 5) or reuses the Option 1 to add an ECS option for C-DNS.

The LDSNR may have these forwarding parameters as part of the instructions received from SMF before or may fetch these forwarding parameters from the SMF by providing the FQDN to the SMF in this step. The LDNSR identifies the UE and the PDU session by using the information received from SMF.

Editor's note:  It is FFS whether the User PDU Session context in LDNSR could be created at this stage

4a-4b:	Option 1: The LDNSR adds the IPv4 subnet or IPv4 address or IPv6 prefix provisioned by SMF in step 3 as ECS option as specified in RFC 7871 and sends it to C-DNS server. The C-DNS returns the DNS response including EAS IP address.

4c-4d:	Option 2a (with direct connectivity between LDNSR and L-DNS): the LDNSR sends the DNS query to the L-DNS server provisioned by SMF and get the DNS response including the EAS IP address.

For both 4a-4b and 4c-4d, after receiving the DNS response, the LDNSR may inform SMF with IP address of EAS and FQDN if certain criteria set by SMF are matched, e.g., the IP address of EAS in DNS response is within the IP range(s) indicated by SMF, or the FQDN is matched. Then, SMF may decide whether to trigger the insertion of the ULCL/Local PSA.

5a.	The SMF decides DNAI and performs UL CL/L-PSA selection and set up. The DNAI and UL CL/Local PSA2 are selected based on the information provided by LDNSR to ensure the selected local PSA and EAS are corresponding to same DNAI.

Option 2b: If the LDNSR requested the SMF to forward the DNS query in step 3, the SMF sends the DNS query to the address of the local DNS server via the Local PSA2. The SMF configures the Local PSA2 to forward the DNS response to the SMF. When receiving the DNS response, the SMF forwards it to the LDNSR. After receiving the DNS response, the LDNSR may notify the SMF with IP address of EAS as in option 1 and 2a. The SMF may perform update of the ULCL/L-PSA based on this notification as in option 1 and 2a..

If the SMF doesn’t support to be indicated by LDNSR to reconfigure the user plane to L-DNS, LDNSR will reuse the Option 1 to add an ECS option for C-DNS.

Optionally, the SMF may configure the L-DNS address into ULCL to be diverted to L-PSA. In order to redirect subsequent DNS queries directly to the L-DNS, the LDNSR acts as an iterative DNS server and redirects the DNS Query to the L-DNS selected for the UE Location by responding with a referral to the UE. This triggers the UE to send a new DNS query towards the L-DNS.

As an optimization, the SMF may configure uplink forwarding rules on UL CL to route the traffic and (if the SMF is configured to determine that the DNS traffic is not encrypted) subsequent DNS queries for FQDNs corresponding to the DNAI to the Local PSA2 (as option 3a in step 7).

5b. In the case of IPv6 multi-homing, the SMF notifies the UE of the availability of the new IP prefix @ PSA2 using an IPv6 Router Advertisement message (RFC 4861 [32]). Optionally, in this RA message, the SMF may notify UE the new address of DNS server (RFC 8106 [33]) and indicate the UE to contact with DHCP server to get the DNS-related configuration information (RFC 4861 [32]), which is used to select DNS server for a target FQDN (RFC 6731 [26]).

Also, the SMF sends IPv6 multi-homed routing rule along with the IPv6 prefix to the UE to influence the selection of the source Prefix for the subsequent DNS queries(RFC 4191 [29]) as described in TS 23.501 [2] clause 5.8.2.2.2. The SMF may re-configure the UE for the Source IP prefix @ PSA1.

6.	The LDNSR sends DNS response to UE via PSA1.

In the case of IPv6 multi-homing, if the DNS-related information is configured on UE, the UE decides the DNS server to use for the subsequent DNS queries. Otherwise, the UE uses the DNS server address provided in the PCO during PDU session establishment. Further, the UE selects the source IP prefix based on the IPv6 multi-homed routing rule provided by SMF.

7.	The SMF may send PDU session modification command with the local DNS resolver/L-DNS server address via ePCO to the UE. And then the UE updates the DNS server address accordingly.

8.	Option 3a and "pre-established" case: Based on the uplink forwarding rules, if an ULCL exists and the query matches the uplink filter rule on UL CL, the UL CL route the DNS query to local PSA2, and then local N6 routing forwards to Local DN. If no filter is present in ULCL, the DNS request is routed to PSA-1. AF influenced routes with anycast service address may be used to provision PCF which is then used during PDU session establishment to setup traffic filters in ULCL. All application traffic including Do53, DoT and DoH requests are steered using this mechanism.

The DNS query may be routed to a local DNS resolver/L-DNS which then resolves the DNS Query or forwards it for resolution. It may derive an ECS option (using locally configured value or the source address/prefix of the received DNS request), then send the DNS query to Authoritative DNS server using the ECS option. Alternatively, the DNS query may be routed to C-DNS via Local N6 or DN. The deployment should ensure the DNS query packet has topologically correct UE source address when DNS resolver gets it, e.g. via NAT in Local DN. See Figure 6.22.1.4-1.

The local DNS resolver or C-DNS server sends DNS response to the UE by retracing the forward path.

Option 3b:

a.	The local DNS resolver modifies the packet's destination IP address (corresponding to LDNSR) to that of the L-DNS and stores the original IP address (LDNSR-IP) and the packet's source IP address (corresponding to UE's IP address) to its own (i.e. the local DNS resolver's) IP address and stores the original source IP address (UE-IP) for later processing.

b.	The local DNS resolver then forwards the modified DNS request to the L-DNS and processes as follows:

-	If the L-DNS can resolve the IP address for the requested FQDN of EAS, it responds to the local DNS resolver with the desired IP address of the local EAS.

-	If the L-DNS cannot resolve the IP address for the requested FQDN of EAS but it is connected to a C-DNS, it communicates with the C-DNS to recursively resolve the EAS IP address.

NOTE 6:	Option 1 and 2 can be still applicable in Phase 2 if the requested FQDN is not configured to be local routed on the UL CL or the DNS traffic is encrypted.

NOTE 7: Option 3 can be applied when SMF is configured to know that there is connectivity between local DN and central DN.

When the PDU Session is released, the LDNSR removes the corresponding UE context for the PDU session (UE IP address).

*************** End Changes ***************