3GPP TSG-WG SA2 Meeting #142E e-meeting 	S2-2008685

Elbonia, November 16 – 20, 2020	(revision of S2-200xxxx)

Source:	Huawei, HiSilicon

Title:	KI#1: Conclusion update

Document for:	Approval

Agenda Item:	8.10

Work Item / Release:	FS_MUSIM / Rel-17

Abstract: This contribution updates the conclusion part for Key issue #1.

1. Introduction/Discussion

During the last SA3 meeting, a SID (S3-202523) has been agreed to study the MUSIM related open issues,

The objective of this study item shall be based on SA2’s work and SA2’s conclusions of the study, such as the potential architecture impact and signaling procedures including following:

- Study security and privacy issues exposing the Paging Cause in cleartext in paging message

….

In order to address the security and privacy issues, the paging cause can’t be sent in clear text, which means a few bits have to be reserved, for example 128 bits per UE for confidentially. Furthermore, the paging cause has to be integrity protected, with will require additional bits (e.g., 128 bits) per UE. Even though the solution will be further studied in SA3, it is obvious that adding paging causes consumes additional paging resources over the air.

Observation 1: Paging causes requires ciphering and integrity protection, which makes the system complex and consumes the paging resources.

The granularity of the paging causes which are currently proposed in the TR is not sufficient to assist the UE to make a decent decision. Take SMS as an example, even though SMS may be important (e.g., the certification code), it can also be useless (e.g., most of the time advertisement). The UE can’t make a decision whether it is important or not based solely on the indication of SMS. Consequently, the paging causes add no obvious benefit but only complexity.

Observation 2: The paging cause add no obvious benefit but only complexity.

The Busy indication is mandatory component for paging cause. Without the Busy indication, the network cannot determine whether the UE is not reachable or if it does not want to respond. Consequently, the network has to enlarge the paging area, e.g., to the whole TA, then further repeat the paging procedure. Paging cause solution without a Busy indication contradicts with this objective of to Key Issue 1.

However, it is also worthy to note that, it is not correct that the UE responds to the network with the Busy Indication all the time, as the UE maybe engaged in a certain delay critical communication, e.g., online game. If the UE has to answer the network paging with Busy Indication, this again contradicts with the this objective of Key issue 1

The present key issue shall study:

-	How to handle the MT service for a Multi-USIM device with the aim of avoiding any unnecessary interruptions of the service in the current system and saving system resources.

-	How to prevent the other system, which triggered the paging message, from performing undesirable operations (e.g. wasting resources, reaching misleading assumption of reachability, etc.).

-	…..

Observation 3: Paging cause solution with Busying indication solution cannot achieve the objective of Key #1.

Lastly, RAN2 made very little progress on this issue at this meeting and RAN2 has not decided whether paging cause solution is feasible or not.

- Provide SA2 with information on paging cause costs based on the email discussion + contributions. Indicate that this may change if assumptions change.

- From RAN2 perspective, we haven't decided on paging cause feasibility yet.

- RAN will evaluate short/long time switching in this WI

Considering the huge complexity and little benefit, it is proposed:

Proposal: Paging cause related solutions are not pursued in normative work.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.776.

# * * * * First change * * * *

8.1	Conclusions for Key Issue #1: Handling of Mobile Terminated service with Multi-USIM device

The following interim conclusions are agreed for the baseline functionality:

Editor's note:	According to the conclusion in KI#3, upon NAS-level leaving the UE may provide assistance information including information to temporarily restrict/filter MT data in this network while the UE has left. Whether UE is allowed to provide information to temporarily restrict/filter MT data in other circumstances is FFS.

# * * * * End of changes * * * *