SA WG2 Meeting #142E	S2-2008695

16 – 20 November, 2020

Source:	Samsung

Title:	KI#2: Conclusion - Reselection Aspects

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_Ph2/ Rel-17

Abstract of the contribution: This contribution proposes an update on conclusion of KI#2 solutions regarding reselection aspects.

1. Introduction

This contribution paper proposes an update on conclusion for the reselection aspect in KI#2.

2. Text Proposal

It is proposed to adopt the following text within the TR 23.700-91.

*** Start of the 1st change ***

8.2.2	Re-selection of NWDAF (category #2)

The following principles are proposed for the normative work on "Re-selection of NWDAF":

a)	Extend or re-use existing Release-16 service operations, i.e. do not introduce new AnalyticsRelocation service.

b)	Support different triggers conditions for the NWDAF re-selection corresponding to UE mobility, load balancing, etc.

c)	Support the handover of analytics subscriptions between NWDAFs.

d)	Support the transfer of analytics context:

-	In case NWDAF re-selection is initiated by the source NWDAF, the source NWDAF sends the request for handover of analytics subscriptions to the target NWDAF. The target NWDAF requests analytics context from the source NWDAF and the analytics context is transferred from the source NWDAF to the target NWDAF in a Nnwdaf_Analytics Info_Request response service. In this case, the NF consuming the NWDAF Analytics is not changed.

-	If NWDAF re-selection is initiated by the target consumer NF due to a change of the consumer NF, the analytics context is requested by the target NWDAF from the source NWDAF and the analytics context is transferred to the target NWDAF in a Nnwdaf_AnalyticsInfo_Request response service.

e)	Analytics context may include the following information:

-	Information related to entities that are subscribed to receive analytics.

-	Historical data available at the source NWDAF and related to the Analytics to be handed over to the target NWDAF (including not only the raw data collected from the data source(s), but also "analytics metadata" such as the time period of the collected data, information about the data source(s), etc.).

NOTE: The availability of the data at the NWDAF in the above bullet may depend on the data collection model (e.g., DCCF or Rel16 data collection framework) used for collecting input data from NFs.

-	Pending Analytics (i.e. not yet notified to the consumer) or historical Analytics Output information - if available at the source NWDAF.

f)	If no change of Analytics consumer is performed, the target NWDAF notifies the NWDAF service consumer about the re-location. If change of Analytics consumer is performed no notification is needed.

g)	Support the prepared analytics handover to inform target NWDAF(s) about a potential analytics handover, so that the source NWDAF checks acceptability of the analytics handover at the target NWDAF(s) and the target NWDAF(s) may prepare to take over those Analytics subscriptions from the source NWDAF

*** End of the 1st change ***