3GPP TSG-SA/WG2 Meeting #142e	S2-2008824

# Elbonia, 16-20 Nov, 2020

Source:	ZTE

Title:	KI#6 conclusion

Document for:	Agreement

Agenda Item: 	8.4

Work Item / Release:	FS_eNS_Ph2 / Rel-17

Abstract of the contribution: This contribution proposes conclusion for KI#6

1	Discussion

2	Proposal

It is proposed to agree the changes:

/********************Start of Change***************/

8.6	Conclusion for Key Issue #6

In order to support the Key Issue #6 (enforcement the constraints related to simultaneous usage of Network Slices), the followings are concluded:

-	All the S-NSSAI in the Allowed NSSAI are considered as compatible so the UE can request these S-NSSAIs for PDU Session simultaneously. This is existing (Rel-15/16) assumption.

-	For those S-NSSAIs which are supported in the current Registration Area, but are not compatible with the Allowed NSSAI, the AMF send them in rejected NSSAI with a new cause value indicating that this S-NSSAI is supported in the current Registration Area but not compatible with the Allowed NSSAI. When the UE has no PDU session associated with any S-NSSAI in the Allowed NSSAI, the UE can request those S-NSSAI(s) for registration within the current Registration Area.

/*******************End of Change****************/