SA WG2 Meeting #142e	S2-2009005

16-23 Nov. 2020	(revision of S2-200xxx)

Source:	Xiaomi

Title:	correction for conclusion KI#1

Document for:	Approval

Agenda Item:	8.2

Work Item / Release:	FS_eNPN/ Rel-17

Abstract: UE(s) with credentials owned by separate entity offering AAA Server does n’t equal to SNPN UE, so to change from “SNPN UE” to “UE”.

*******************Start of changes *******************

8	Conclusions

8.1	Key Issue #1: Enhancements to Support SNPN along with credentials owned by an entity separate from the SNPN

Editor's note:	This clause will capture conclusions for Key Issue #1.

8.1.1	Conclusions for scenario where the SNPN offers connectivity for UE(s) with credentials owned by separate entity offering AAA Server

The scenario where the separate entity owning the credentials has only an AAA Server is supported. It is proposed to adopt the following conclusion principles:

-	The SNPN will host a function (e.g. enhanced AUSF or new NF) supporting primary authentication and authorization of SNPN UEs that use credentials from the AAA Server.

NOTE:	Whether to use a new NF or enhanced AUSF will be determined based on feedback from SA WG3.

-	The function (see above) forwards EAP messages to the AAA server that is acting as the EAP Server for the purpose of primary authentication and authorization.

Editor's note:	Details of External UE ID used for this primary authentication and authorization and related aspects are FFS.

-	The UDM supports common mobility management and session management subscription information that is needed at the time of Registration and Session Management procedure.

Editor's note:	It is FFS if the AAA server supports providing the subscription information needed for registration and session management procedure.

Editor's note:	It is FFS if subscription information for individual UEs (with credentials stored in AAA) are supported in the UDM.

-	After completing the primary authentication and authorization successfully, the AMF or SMF can retrieve the UE subscription data (i.e. mobility management and session management subscription) using SUPI from UDM.

Editor's note:	Furthermore, SA WG2 should decide, if interfaces between existing/enhanced AUSF (or new NF) and AAA server should be in scope of SA WG2. The architecture to support AAA interfaces to the AAA server will be revised based on SA WG3's feedback.

Editor's note:	The Registration procedure supporting AAA interfaces towards the AAA server will be revised based on input from SA WG3 including the aspect which subscription data the SNPN should use in this scenario.

********************End of changes ********************