3GPP TSG-SA WG2 Meeting #142E e-meeting 	S2-2009159

Elbonia, November 16 – 20, 2020

Source:	Huawei, HiSilicon

Title:	KI#1, Update to Solution #16

Document for:	Approval

Agenda Item: 	8.3

Work Item / Release:	FS_enh_EC /Rel-17

Abstract of the contribution: This contribution proposes update to solution #16.

1	Introduction

2	Proposal

It is proposed to add the following changes in TR 23.748.

**************************** Start Change ***************************

6.16.2.1	Functional Description

Edge Configuration Server Information may be provided by the SMF to the UE during the PDU Session Establishment procedure in the PCO. During PDU Session Establishment, the UE may provide an indication of Requesting Edge Configuration Server Information to retrieve the Edge Configuration Server Information. The indication of Requesting Edge Configuration Server Information is provided during PDU Session Establishment because not all UEs will support SA6 Edge Enabler Clients. Additionally, when the UE does host SA6 Edge Enabler Client(s), not all of the UE's PDU Sessions can make use of Edge Configuration Server Information. Thus, not all UEs and not all PDU Sessions can make use of Edge Configuration Server Information. The indication of Requesting Edge Configuration Server Information is needed so that the network knows that the UE supports the feature and so that the network knows what PDU Session the Edge Configuration Server Information should be associated with.

The SMF may derive the Edge Configuration Server Information based on local configuration, the UE's location, and/or UE subscription information received from the UDM. In HR sessions, the Edge Configuration Server Information comes from the H-SMF. The UE's subscription information may include identities of Edge Configuration Servers that the UE may access. For example, some UEs might have a relationship with a 3rd party and the network wants those UEs to be able to discover ECS(s) that are deployed by the 3rd party. If the Edge Configuration Server Information changes (e.g. due to, local configuration change, or a change of UE location), the SMF may use the PDU Session Modification procedure to send the updated Edge Configuration Server Information to the UE in the PCO.

NOTE 1: 	When the Edge Configuration Server Information comes from the HPLMN (H-SMF) (i.e. in a home routed scenario), any Edge Configuration Server Information that is associated with the VPLMN needs to come from the HPLMN. For example, the information that the H-SMF sends to the UE may be based on the VPLMN the UE is registered with and/or the UE’s subscription information. The ECS Information that is sent to the UE might point the UE to ECS(s) that are managed by the home operator, visited operator, or a 3rd party.

NOTE 2:	The UE might use a HR session to get ECS Information and access to the ECS to get information about an EAS that is accessible via the VPLMN. After that, since the HR session cannot be used to access to a EAS in VPLMN in this release, the UE needs to establish a new LBO PDU Session to access the EAS in VPLMN. In this case the new PDU Session has to be established to a different (DNN, S-NSSAI) that is configured in the UDM to be eligible to LBO deployment with the current VPLMN of the UE.

NOTE 3:	In LBO scenarios, the SMF may send the UE Edge Configuration Server Information.

The Edge Configuration Server Information sent to the UE in the PCO corresponds to the IP address (and possibly port) of the ECS and/or to the FQDN of the ECS.

********************************* End Change ****************************