SA WG2 Meeting #143e	S2-2100089

# Feb 24th – March 9th, 2021 ; Elbonia              		 (revision of S2-2100089)

Source: 	Nokia, Nokia Shanghai Bell, NTT DOCOMO

Title: 	 KI2 Target PSA buffering

Document for: 	Agreement (P-CR)

Agenda Item: 	8.3

Work Item / Release:	eEDGE_5GC / Rel-17

Abstract of the contribution: P-CR on KI2 Target PSA buffering

# 1 Discussion

### TR 23.748 § 9.2.1	Conclusions for Key Issue #2: Reducing packet loss during EAS relocation

Buffering uplink packets in the target PSA until receiving the indication of successful EAS relocation from the AF as proposed in Solution #27 and Solution #38 is recommended for normative phase. The old EAS may continue to serve the UE until the successful EAS relocation is done in order to reduce the packet loss. When the EAS relocation starts is out of scope of SA2.

This solution may be applied to all connectivity models. Whether Buffering of uplink packets applies to the application traffic depends on the application requirement.

NOTE:	How 5GC gets the application requirement and in what granularity are to be decided in normative phase.

There are companion 23.501 CR 2528 and 23.502  CR 2462.

# 2 Proposal

It is proposed to update TS 23.548 as follows

FIRST CHANGE All text is new

### 6.3.5	Packet buffering for low packet loss

This procedure aims at synchronizing between EAS relocation and UL traffic from the UE, ensuring that UL traffic from the UE is sent to the new EAS only when EAS context transfer has been carried out

Figure 6.3.5-1: Packet buffering for low packet loss

.

1.	The SMF decides to change the local PSA of a PDU Session with UL CL.

2.	The SMF may send an early notification to the AF after target PSA (i.e. PSA2) is selected and waits for a notification response from the AF. The AF may reply in positive to the notification by indicating that buffering of uplink traffic to the target DNAI is needed as long as traffic to the target DNAI is not authorized by the AF . This is e.g. as defined in Steps 1 and 2 of 23.502 Figure 4.3.6.3-1

3.	The SMF configures the PSA2 as specified in step 2 in clause 4.3.5.6  and step 2 in clause 4.3.5.7 of TS 23.502 [3], which may request the PSA2 to buffer uplink traffic. The PSA1 (i.e. source PSA) keeps receiving downlink traffic from EAS1 and send it to the UE until it is released in step 7.

4.	The SMF sends an N4 Session Modification Request to the UL CL to update the UL CL rules regarding to the traffic flows that the SMF tries to move from PSA1to PSA2..This is e.g. as defined in 23.502 [3] Figure 4.3.5.7-1 step 3

5.	The SMF sends a Late Notification to the AF. This corresponds e.g. to step 4a-c of TS 23.502 [3] Figure 4.3.6.3-1 and is also described in step 6 or 7 of TS 23.502 [3] Figure 4.3.5.7-1.

6	The application layer performs the EAS relocation (This corresponds to step 4d of 23.502 [3] Figure 4.3.6.3-1). The UE context is relocated from the old EAS to new EAS. The old EAS stops to serve the UE

6a The old EAS may send the new EAS IP address to the UE via application mechanism, for example the HTTP redirection.

6b.	The UE may send an acknowledge of the HTTP redirection to the old EAS. The UE then can start to use the new EAS IP address as target IP address.

7.	The SMF sends a Late Notification to the AF. When EAS relocation is completed, the AF sends a notification response to the SMF. This corresponds to step 4e-g of 23.502 Figure 4.3.6.3-1(and is e.g. also described in  step 6 or 7 of TS 23.502 [3] Figure 4.3.5.7-1) and may indicate that buffering of uplink traffic to the target DNAI is no more needed as traffic to the target DNAI /EAS is now authorized by the AF

8.	(if AF has indicated that buffering of uplink traffic to the target DNAI is no more needed as traffic to the target DNAI /EAS is now authorized by the AF) The SMF updates the PSA2 by indicating the PSA2 to send the buffered uplink packets (step 8b) and to stop buffering.

The SMF releases PSA1.

NEXT CHANGE (2)

End of changes