3GPP TSG-SA WG2 Meeting #143E	S2-2100249

# 24 Feb – 9 March, 2021, Elbonia											(Revision of S2-210xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2547 | rev |  | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Network selection for NR satellite access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_ARCH |  |  |  |  |  | Date: |  |  | 2021-01-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR adds high level description of network selection enhancements for NR satellite access under the new satellite access clause 5.X. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New sub-clause is added to explain the main network selection principles via reference to other clauses / specifications. This CR re-uses the text of DRAFT CR S2-2008309 that was agreed in SA2 #141E. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not possible to meet the regulatory requirements identified under key issue #10 for NR satellite access. Regulatory requirements on UE registration to PLMN in the same country where the UE is currently cannot be met if the UE cannot use its awareness of the UE location in certain country and the network cannot enforce this requirement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.x.y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### **** First change *****

### 5.x.y	Network selection for NR satellite access

Network selection principles specified in clause 5.2.2 apply also for NR satellite access.

For NR satellite access, a UE with location capability should use its awareness of its location to select a PLMN that is allowed to operate in the country of the UE location as specified in TS 23.122 [17]. In order to ensure that the regulatory requirements are met, the network shall enforce this UE choice by either rejecting any NAS request targeted towards a PLMN that is not allowed to operate at the known UE location or by accepting the NAS procedure and later deregister the UE once the UE location is known, as specified in TS 23.502 [3].