3GPP TSG-WG SA2 Meeting #143E e-meeting 	S2-2100506

Elbonia, February 24 – March 09, 2021	(revision of S2-210xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2594 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Discovery of UDM/AUSF in the separate entity and related reference architecture |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  | Date: |  |  | 2021-02-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | TR23.700-07 clause 8.1.6 concluded that “Discovery of AUSF/UDM in the separate entity can be supported by cross network service discovery and registration procedure as specified in TS 23.502 [6] clause 4.17.5 where Home PLMN is replaced by the separate entity. “ This contribution implements this aspect in TS23.501 clause 5.30.2.x and clause 4.2.x |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added the feature of registration and discovery of AUSF/UDM in the separate entity in TS23.501 and reference to the related procedure in TS23.502 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Agreed feature in TR23.700-07 not capatured. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.30.2.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

4.2.x	Architecture Reference Model for accessing to SNPN using credentials from a separate entity

The architecture where PLMN acts as the Separate entity is depicted in Figure 4.2.4-1, Figure 4.2.4-3, Figure 4.2.4-4 and Figure 4.2.4-6 with the difference that VPLMN is replaced by serving SNPN and HPLMN is placed by PLMN which acts as Separate entity.

The architecture where Separate entity, which is not PLMN but hosts UDM, AUSF is depicted in figure 4.2.X-1.

Figure 4.2.X-1: architecture where Separate entity, which is not the PLMN but hosts UDM, AUSF

Editor's note:	Other architectures where Separate entity, which is not PLMN but hosts UDM, AUSF is FFS.

# * * * * Second change * * * *

5.30.2.x	Access to SNPN using credentials from a separate entity

UE supports access serving SNPN using credentials from a separate entity. The supported architectures are listed as below and corresponding figures are shown in clause 4.2.X:

- PLMN acts as the Separate entity.

- Separate entity, which is not PLMN but hosts UDM, AUSF.

In both cases, AUSF/UDM in the separate entity can be discovered using service discovery and registration procedure described in clause 4.17.5a in TS 23.502 [6].

Editor's note:	Description on the architecture where AAA is involved in separate entity will be added once SA3 provide feedback.

# * * * * Third change * * * *

# * * * * Fourth change * * * *

# * * * * Fifth change * * * *

# * * * * End of changes * * * *