3GPP TSG-WG SA2 Meeting #143E e-meeting  	S2-2100578

# Elbonia, Feb 24 – Mar 09, 2021	(revision of S2-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2608 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | X | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of prioritized list of preferred SNPNs and Group IDs to the UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  | Date: |  |  | 2021-02-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Update of prioritized list of preferred SNPNs and Group IDs to the UE per the conclusion of FS_eNPN as described in clause 8.1.7 of TR 23.700-07. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update of prioritized list of preferred SNPNs and Group IDs to the UE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support of dynamic update of prioritized list of preferred SNPNs and Group IDs to the UE |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.30.2.3.0 (NEW), 5.30.2.3.1 (NEW) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.30.2.3	UE configuration and subscription aspects

5.30.2.3.0	General

An SNPN-enabled UE is configured with subscriber identifier (SUPI), credentials for each subscribed SNPN identified by the combination of PLMN ID and NID. If an SNPN-enabled UE is configured with an N3IWF, it is also configured with an identifier of the country where the configured N3IWF is located.

A subscriber of an SNPN is either:

-	identified by a SUPI containing a network-specific identifier that takes the form of a Network Access Identifier (NAI) using the NAI RFC 7542 [20] based user identification as defined in TS 23.003 [19] clause 28.7.2. The realm part of the NAI may include the NID of the SNPN; or

-	identified by a SUPI containing an IMSI.

An SNPN-enabled UE supports the SNPN access mode. When the UE is set to operate in SNPN access mode the UE only selects and registers with SNPNs over Uu as described in clause 5.30.2.4.

Emergency services are not supported in SNPN access mode.

NOTE 1:	Voice support with emergency services in SNPN access mode is not specified in this release.

If a UE is not set to operate in SNPN access mode, even if it is SNPN-enabled, the UE does not select and register with SNPNs. A UE not set to operate in SNPN access mode performs PLMN selection procedures as defined in clause 4.4 of TS 23.122 [17]. For a UE capable of simultaneously connecting to an SNPN and a PLMN, the setting for operation in SNPN access mode is applied only to the Uu interface for connection to the SNPN. Annex D.4 provides more details.

NOTE 2:	Details of activation and deactivation of SNPN access mode are up to UE implementation.

5.30.2.3.1	SNPN selection for UEs with subscriptions/credentials of a Separate entity

To enable SNPN selection for UEs with subscriptions/credentials of a Separate entity, the UE configuration shall include the

-	User-controlled prioritized list of preferred SNPNs;

-	Separate entity controlled prioritized list of preferred SNPNs;

-	Separate entity-controlled prioritized list of Group IDs (GIDs).

The UE subscription data shall also contain the information of subscribed SNPN (s) that allow the UE to access using subscriptions/credentials of a Separate entity. The information of subscribed SNPN (s) consists of the Separate entity controlled prioritized list of preferred SNPNs and Separate entity-controlled prioritized list of GIDs.

When UE is configured to access SNPNs using an SNPN subscription when the separate entity is offering a UDM/AUSF, the Separate entity controlled prioritized list of preferred SNPNs and/or GIDs in the UE configuration can be updated using the UE Parameters Update (UPU) via UDM Control Plane Procedure as defined in TS 23.502 [3] clause 4.20 or Steering of Roaming (SoR) as defined in TS 23.122 [17] Annex C.

When UE is configured to access SNPNs using a PLMN subscription when the separate entity is offering a UDM/AUSF, the UE subscription and/or UE configuration shall also include an “SNPN-only” indication whether the UE is only allowed to access the SNPN. Together with the Operator Controlled PLMN Selector with Access Technology as defined in TS 23.122 [17] or the “SNPN-only” indication, the PLMN controlled prioritized list of preferred SNPNs and/or GIDs in the UE configuration can be updated using the UE Parameters Update (UPU) via UDM Control Plane Procedure as defined in TS 23.502 [3] clause 4.20 or Steering of Roaming (SoR) as defined in TS 23.122 [17] Annex C.

When UE is configured to access SNPNs using an SNPN subscription when the separate entity is offering an AAA server, the UE configuration for SNPN selection is statically configured. When the UDM of the SNPN supports mobility management and session management subscription information, the Separate entity controlled prioritized list of preferred SNPNs and/or GIDs in the UE configuration can be updated using the UE Parameters Update (UPU) via UDM Control Plane Procedure as defined in TS 23.502 [3] clause 4.20 or Steering of Roaming (SoR) as defined in TS 23.122 [17] Annex C.

# * * * * End of changes * * * *